| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0321 |
| --- | --- |

| Title: | AHG15: Signaling of chroma QP mapping table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhao Wang, Yan Ye, Jiancong Luo | Tel: Email: | (+86)13716516722 baixiu.wz@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

At the 15th JVET meeting, a new signalling mechanism for the luma-to-chroma QP mapping table proposed in JVET-0186 was adopted into the VVC draft 6. VVC draft 6 supports signalling of only one luma-to-chroma QP mapping table used for Cb, Cr and joint CbCr coding simultaneously, or three tables for Cb, Cr and joint CbCr separately. This contribution proposes two changes to the signalling of luma-to-chroma QP mapping table. Firstly, it is proposed to support a default table without explicit signalling, such that for most use cases an encoder is not required to put the same luma-to-chroma QP table repeatedly in the bitstreams. Secondly, it is proposed to condition the signalling and derivation of the joint CbCr table based on whether the joint CbCr residual coding tool is enabled or not.

Introduction

Instead of using a fixed luma-to-chroma QP mapping table, VVC draft 6 supports a flexible signalling mechanism of luma-to-chroma QP mapping table by the manner of piece-wise linear function. The syntax structure is shown in the following.

Table 1. The syntax structure of signalling the luma-to-chroma QP mapping tables in VVC draft 6

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_out_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

same_qp_table_for_chroma equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals as well as joint CbCr residuals. same_qp_table_for_chroma equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS. When same_qp_table_for_chroma is not present in the bistream, the value of same_qp_table_for_chroma is inferred to be equal to 1.

num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 63 + QpBdOffsetC, inclusive. When num_points_in_qp_table_minus1[ 0 ] is not present in the bitstream, the value of num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_in_val_minus1[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

delta_qp_out_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_out_val[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_out_val[ 0 ][ j ] is inferred to be equal to 0.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..same_qp_table_for_chroma ? 0 : 2 is derived as follows:

qpInVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_in_val_minus1[ i ][ 0 ]
qpOutVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_out_val[ i ][ 0 ]
for( j = 1; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j ] = qpOutVal[ i ][ j − 1 ] + delta_qp_out_val[ i ][ j ]
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	
for( j = 0; j < num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 2 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInval[ i ][ j + 1 ]; k++, m++ )
	    ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
	    ( delta_qp_out_val[ i ][j + 1] * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When same_qp_table_for_chroma is equal to 1, ChromaQpTable[ 1 ][ k ] and ChromaQpTable[ 2 ][ k ] are set equal to ChromaQpTable[ 0 ][ k ] for k = −QpBdOffsetC..63.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffsetC to 63, inclusive for i = 0..same_qp_table_for_chroma ? 0 : 2 and j = 0..num_points_in_qp_table_minus1[ i ].

Proposed method

In H.264/AVC and H.265/HEVC, a fixed luma to chroma QP mapping relationship was used, and no explicit signalling was needed. In comparison, VVC draft 6 always requires explicit signalling of at least one luma-to-chroma QP table. In this contribution, it is proposed to add support for a default luma-to-chroma QP table. This way, for the most common use cases, the encoder does not need to signal the same table repeatedly in the bitstreams, Further, it is proposed to separate the joint CbCr QP table from the Cb table and the Cr table, and to only signal and derive the joint CbCr QP table when the joint CbCr residual coding tool is enabled. The proposed syntax and semantics of signalling luma-to-chroma QP mapping tables are shown in the following.

Table 2. The proposed syntax structure of signalling the luma-to-chroma QP mapping tables

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| same_qp_table_for_chroma | u(1) |
| qp_table_for_chroma_present_flag | u(1) |
| if(qp_table_for chroma_present_flag){ |  |
| same_qp_table_for_cbcr | u(1) |
| if(sps_joint_cbcr_enabled_flag) |  |
| qp_table_for_jointcbcr_present_flag | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| for( i = 0; i < 2 - same_qp_table_for_cbcr+qp_table_for_jointcbcr_present_flag; i++){ |  |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_out_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

qp_table_for_chroma_present_flag equal to 1 specifies that at least one luma-to-chroma QP table is signalled in the SPS. qp_table_for_chroma_present_flag equal to 0 specifies that the luma-to-chroma QP tables are not present in the SPS. When qp_table_for_chroma_present_flag is not present in the bitstream, the value of qp_table_for_chroma_present_flag is inferred to be equal to 0.

When the luma-to-chroma QP tables are not present, the luma-to-chroma QP table for the Cb, Cr and joint CbCr residuals will be derived by the default process. The default luma-to-chroma mapping table is set to the current mapping table used in common test condition (CTC).

same_qp_table_for_cbcr equal to 1 specifies that only one luma-to-chroma QP mapping table is signalled and this table applies to both Cb and Cr residuals. same_qp_table_for_chroma equal to 0 specifies that two luma-to-chroma QP mapping tables are signalled in the SPS for the Cb and Cr residuals separately. When same_qp_table_for_cbcr is not present in the bistream, the value of same_qp_table_for_cbcr is inferred to be equal to 1.

qp_table_for_jointcbcr_present_flag equal to 1 specifies that the luma-to-chroma QP table for the joint CbCr residuals is signalled in SPS. qp_table_for_jointcbcr_present_flag equal to 0 specifies that the luma-to-chroma QP mapping table applied to joint CbCr residuals is not present in the SPS. When qp_table_for_jointcbcr_present_flag is not present in the bistream, the value of qp_table_for_jointcbcr_present_flag is inferred to be equal to 0.

When luma-to-chroma QP table for joint CbCr residuals is not present in the SPS, and when the joint CbCr residual coding tool is enabled, the mapping table for joint CbCr will be derived by averaging the luma-to-chroma QP tables for the Cb and Cr residuals.

The chroma QP mapping tables ChromaQpTable[ i ] (i = 0,1, 2 for Cb, Cr and joint Cb Cr, respectively) are derived as follows:

if( !qp_table_for_chroma_present_flag ){

num_points_in_qp_table_minus1[ 0 ] = 1

delta_qp_in_val_minus1[ 0 ][ 0 ] = 32 + QpBdOffsetC

delta_qp_in_val_minus1[ 0 ][ 1 ] = 11

delta_qp_out_val[ 0 ][ 0 ] = 32 + QpBdOffsetC

delta_qp_out_val[ 0 ][ 1 ] = 9

}

for( i = 0; i<3; i++ ){

if( i == 1 && same_qp_table_for_cbcr )

continue

if( i == 2 && sps_joint_cbcr_enabled_flag && !qp_table_for_jointcbcr_present_flag )

continue

qpInVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_in_val_minus1[ i ][ 0 ]
qpOutVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_out_val[ i ][ 0 ]
for( j = 1; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
   qpInVal[ i ][ j ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
   qpOutVal[ i ][ j ] = qpOutVal[ i ][ j − 1 ] + delta_qp_out_val[ i ][ j ]
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
   ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	
for( j = 0; j < num_points_in_qp_table_minus1[ i ]; j++ ) {
   sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 2 ) >> 1
   for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInval[ i ][ j + 1 ]; k++, m++ )
      ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
      ( delta_qp_out_val[ i ][j + 1] * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] ] + 1; k <= 63; k++ )
   ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

}

if( same_qp_table_for_cbcr ){

for( k = −QpBdOffsetC; k<=63; k++ )

ChromaQpTable[ 1 ][ k ] = ChromaQpTable[ 0 ][ k ]

}

if( sps_joint_cbcr_enabled_flag && !qp_table_for_jointcbcr_present_flag ){

for( k = −QpBdOffsetC; k<=63; k++ )

ChromaQpTable[ 2 ][ k ] = ( ChromaQpTable[ 0 ][ k ] + ChromaQpTable[ 1 ][ k ] ) >> 1

}

References

K. Ramasubramonian, G. Van der Auwera, D. Rusanovskyy, M. Karczewicz, “AHG15: On signalling of chroma QP tables,” JVET-O0650, July 2019.

S. Ikonin, R. Chernyak, E. Alshina, “AHG15: Signalling of chroma Qp mapping table,” JVET-O0186, July 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).