| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0324 |
| --- | --- |

| Title: | AHG15: Quantization matrix selection based on block area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Philippe Bordes, Karam Naser, Edouard François | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to change quantization matrix selection for rectangular transform blocks, so that it is based on block area instead of block maximal dimension. This is said to be consistent with other parts of VVC like quantization groups, DMVR, BCW, and expected to derive QMs of rectangular blocks more likely from square-sized QMs.

# Introduction

In VVC draft 6, Quantization Matrices (QM) that are transmitted in the bitstream are square-sized. QM adaptation to rectangular transform blocks is the following:

- select a QM with size identifier (sizeId) based on the maximal dimension of the block (either width or height)

- drop either lines or columns to make the QM the same size as the block

The process is found in the scaling_list_data semantics, and reproduced below, with relevant part underlined:

For a quantization matrix with rectangular size, the five-dimensional array ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ], with wId = 0..6, hId = 0..6, matrixId = 0..5, x = 0..( 1  <<  wId ) − 1, y = 0..( 1  << hId ) − 1, and wId != hId, specifies the array of scaling factors for size (1 << wId)x(1 << hId). The elements of ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ] are generated by using ScalingList[ maxSizeId ][ matrixId ][ i ] with maxSizeId = max( wId, hId ), as follows:

k = min( maxSizeId, 3 ), 
x = DiagScanOrder[ k ][ k ][ i ][ 0 ]
y = DiagScanOrder[ k ][ k ][ i ][ 1 ]
ratioW = ( 1 << wId )  /  ( 1 << k )
ratioH = ( 1 << hId )  /  ( 1 << k )
diffWH = 1 << abs(wId − hId)						(7-86)
if( wId > hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( ( y * diffWH ) / ratioW ) + x / ratioW ] ]
else( wId < hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( y / ratioH ) + (x * diffWH) / ratioH ] ]

However, in VVC draft 6, a transform block that has an aspect ratio far from 1:1 is assigned a QM that was intended for blocks with a very different number of samples. The block area (or number of samples) could be considered as better matching the idea of “size” than the maximal dimension. This would be consistent with other parts of VVC, like quantization groups, and tool activation like DMVR, BCW, affine, bi-prediction, merge region, local dual-tree, etc, that are based on block area.

Besides, this could be in line with block size adapted quantization methods, as suggested in JVET-M0114 which proposed implicit QPs based on block size. This can be implemented by specific QMs for each sizeId, e.g. by applying a different QM offset for each sizeId, which is made possible at very low cost by JVET-P0110. In that context, selecting sizeId based on block area is deemed preferable, because the block area is better correlated with image local activity than the block maximal dimension.

This contribution proposes to change QM selection for rectangular blocks so that they are based on block area instead of maximal dimension.

# Proposal description

## VVC draft 6 context

The semantics shown in the previous section is changed as follows (changed or added text highlighted, and removed text lined-through):

For a quantization matrix with rectangular size, the five-dimensional array ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ], with wId = 0..6, hId = 0..6, matrixId = 0..5, x = 0..( 1  <<  wId ) − 1, y = 0..( 1  << hId ) − 1, and wId != hId, specifies the array of scaling factors for size (1 << wId)x(1 << hId). The elements of ScalingFactor[ wId ][hId][ matrixId ][ x ][ y ] are generated by using ScalingList[ avgSizeId ][ matrixId ][ i ] with avgSizeId = ( wId + hId + 1 ) / 2, as follows:

k = min( avgSizeId, 3 ), 
x = DiagScanOrder[ k ][ k ][ i ][ 0 ]
y = DiagScanOrder[ k ][ k ][ i ][ 1 ]
ratioW = ( 1 << wId )  /  ( 1 << k )
ratioH = ( 1 << hId )  /  ( 1 << k )
diffWH = 1 << abs(wId − hId)					(7-86)
if( wId > hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( ( y * diffWH ) / ratioW ) + x / ratioW ] ]
else( wId < hId )
	ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
		ScalingList[ maxSizeId ][ matrixId ]
			[ Raster2DiagScanPos[ k ][ k ][ ( 1 << k ) * ( y / ratioH ) + (x * diffWH) / ratioH ] ]
ScalingFactor[ wId ][ hId ][ matrixId ][ x ][ y ] =
	ScalingList[ avgSizeId ][ matrixId ]
		[ Raster2DiagScanPos[ k ][ k ][ (((y << k) >> hId) << k) + ((x << k) >> wId) ] ]

## JVET-P0110 context

In the context of the QM specification proposed by JVET-P0110, the first equation found in “Scaling matrix derivation process” is changed as follows:

matrixId = 6 * sizeId + matrixTypeId			(6-1)
with log2TuWidth = log2(blkWidth) + ((cIdx > 0) ? log2(SubWidthC) : 0),
log2TuHeight = log2(blkHeight) + ((cIdx > 0) ? log2(SubHeightC) : 0),
sizeId = 6 – max( log2TuWidth, log2TuHeight )( log2TuWidth + log2TuHeight + 1 ) / 2, and
matrixTypeId = ( 2 * cIdx + ( predMode = =  MODE_INTRA ? 0 : 1 ) )

# References

[JVET-O2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Gothenburg, SE, July 2019

[JVET-M0114]	P. de Lagrange, E. François, F. Le Léannec, “CE7-related: implicit QP-offset based on block size,” JVET-M0114, Marrakech, MA, Jan. 2019

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).