| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0326 |
| --- | --- |

| Title: | AHG17: External reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Didier Doyen, Franck Galpin, Michel Kerdranvat 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com, michel.kerdranvat@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

It is proposed to signal in the reference-picture-list-struct whether some reference pictures are provided by an external means not specified in the Specification. It is also proposed to signal in the reference-picture-list-struct whether the motion information associated with one reference picture is zero (default) or not.

# Introduction

It is anticipated the need for future systems and/or applications, to be able to use reference pictures that are available through other (external) means, i.e. out of the scope of the current video standard.

For example:

1)	One picture of current layer may be predicted with a reconstructed picture decoded from another layer coded with different standard (e.g. layer-1 is VVC encoded, layer-0 is HEVC or AVC encoded). This special mode can be addressed with SHVC [2] in case of the base layer (layer-0) only and the other layers are coded with SHVC (see Annex F: “external base layer” in [2] ).

2)	It has been proposed in the literature the use of one additional reference picture generated with application-based modules. For example, L.Zhao [3] use an additional “virtual reference” frame computed with deep-learning based frame rate-up conversion algorithm, computed with two reference pictures of the DPB.

3) In the current specification [1], the semantics of the reference-picture-list-structure (RPLS) allow for duplicating one reference picture with different weighted prediction parameters to manage local illumination compensation or for increasing motion precision [1][4]. This principle could be extended with more elaborated illumination change models.

4)	Multiview content can provide parallax to immersive content. There are a lot of redundancies between the different views to allow this parallax. One view can be a good predictor for another one. There is no mechanism available in VVC to calculate a prediction of a view from texture and depth of another view. In order to exploit such promising possibility to predict one view from another one, there is a need to have a way to signal that one view in the DPB has been generated with external means to be used as a predictor.

# Proposals

## External reference picture signaling

1) On defines an external reference picture (ERP) as a reference picture provided by an external means not specified in this Specification. An ERP may be an inter layer reference picture ILRP or not.

It is proposed to signal with a flag sps_enabled_external_ref_pic_flag at the SPS whether the current sequence may contain external reference pictures.

sps_enabled_external_ref_pic_flag equal to 0 specifies that no external reference pictures is used for inter prediction of any coded picture in the CVS. sps_enabled_external_ref_pic_flag equal to 1 specifies that external reference pictures may be used for inter prediction of one or more coded pictures in the CVS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_enabled_external_ref_pic_flag | u(1) |
| … |  |

2) In the RPLS, a flag external_ref_pic_flag may be signaled for each reference picture, with the following semantics:

external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is provided by an external means not specified in this Specification. external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is a reconstructed picture specified by this Specification.

When external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is not present, it is inferred to be equal to 0.

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( sps_enabled_external_ref_pic_flag ) |  |
| external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| … |  |

## Multi-view support in VVC

Figure 1: multi-view system: disparity of one object in view-A versus view-B.

In case of multi-view system, if the viewport information of two views VA and VB (sensor position in the scene, field of view…) and if the view depth is known (depth information for each sample of the view), then one can derive the disparity DAB in-between the two views VA and VB. The 2D-disparity information DAB, which is the displacement between the position in view-A and the position in view-B of the same point in the scene, allows projecting VB samples into VA (Figure 1) (eq.1). In this case, the model is same as regular motion compensation applied on one reference picture (eq.2), and it turns out predicting VA texture from the VB texture using the disparity as the motion information (MV), where VB texture is the reference picture (eq.1):

proj-view-A[pocCur][ x ] = view-B[pocRef][ x + DAB ] 		(eq.1)

pred-Ima[pocCur][x] = Ima-ref[pocRef][ x + mv ]		(eq.2)

Figure 1: prediction of view-A from view-B and disparity used as motion.

In multi-view scalability, one can leverage on this relation by copying the disparity-info into the motion-info associated with the inter-layer reference picture (ILRP). If the ILRP is marked as “used as co-located reference”, then the (eq.2) allows TMVP providing a straightforward predictor candidate to view-A.

However, in some other scalability cases, the benefit of copying the motion-info associated with the ILRP may be considered as not significative and it may be preferred to use default values to avoid memory bandwidth burden.

One proposes to signal in the reference-picture-list-structure (RPLS) with a flag zero_mv_ref_flag whether the motion-info of the i-th reference picture is default (zero motion-vectors) or not. In that way, in case of multi-view scalability, one can set the flag to 1 and leverage on the disparity info if it is available. In other scalability cases, one may choose not to copy the motion-info into the ILRP by setting zero_mv_ref_flag to zero.

zero_mv_ref_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure has collocated motion vectors equal to default. zero_mv_ref_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure has collocated motion vectors that may be not equal to default.

CONFORMANCE NOTE: it is a requirement that the values of the reconstructed motion vectors are equal to zero when the reference has zero_mv_ref_flag equal to 1.

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( sps_enabled_external_ref_pic_flag ) { |  |
| external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if ( external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ¦¦ inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) |  |
| zero_mv_ref_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } |  |
| … |  |

# Summary of proposed reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( enabled_external_ref_pic_flag ) { |  |
| external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if ( external_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ¦¦ inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ]) |  |
| zero_mv_ref_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } |  |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } |  |
| else |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

# References

- Text of Versatile Video Coding (Draft 6), document JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- High efficiency video coding (HEVC), combined format range extensions (RExt), scalability (SHVC), and multi-view (MV-HEVC) extensions, ITU-T H.265, April 2015.

- L.Zhao et al. “Enhanced Motion-Compensated Video Coding with Deep Virtual Reference Frame Generation,” IEEE Transactions on Image Processing, Vol.28, No 10, October 2019.

- A.Tourapis, “Weighted prediction methods for improved motion compensation,” 2009 16th IEEE International Conference on Image Processing (ICIP).

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).