| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0330 |
| --- | --- |

| Title: | Non-CE5: CC-ALF filtering simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the filter of the Cross Component Adaptive Loop Filter (CC-ALF) that is studied in CE5-2.1. The proposed simplifications reduce the number of bits used in the filtering operations of CC-ALF to enable using smaller registers. It is also proposed to reduce the number of filter coefficients (from 18 to 16 or 8 coefficients) to reduce the number of operations per sample. Three distinct configurations are proposed for those simplifications, as alternative complexity versus coding gain trade off.

The average Y/Cb/Cr BDR gains of Configuration 1 over VTM-6.0 are 0.12%/-7.30%/-5.73% for the AI configuration, 0.15%/-10.06%/-9.46% for the RA configuration, 0.13%/-15.13%/-11.73% for the Low Delay B configuration, and 0.24%/-15.46%/-11.46% for the Low Delay P configuration.

The average Y/Cb/Cr BDR gains of Configuration 2 over VTM-6.0 are 0.12%/-7.43%/-5.87% for the AI configuration, 0.15%/-10.21%/-9.75% for the RA configuration, 0.17%/-15.04%/-11.41% for the Low Delay B configuration, and 0.21%/-15.40%/-11.21% for the Low Delay P configuration.

The average Y/Cb/Cr BDR gains of Configuration 3 over VTM-6.0 are 0.11%/-6.93%/-5.22% for the AI configuration, 0.17%/-9.53%/-9.00% for the RA configuration, 0.12%/-13.63%/-10.02% for the Low Delay B configuration, and 0.2%/-13.75%/-9.81% for the Low Delay P configuration.

# Introduction

During the 14th JVET Meeting [1], Sharp has proposed a new Cross-Component Adaptive Loop Filter (CCALF) for Chroma in JVET-O0636 [3], which is under study in CE5 [5].

Our contribution is focused on CE5-2.1 test, where CC-ALF is used as an in-loop filter. Similar conclusion should be obtained when simplifying CC-ALF as a post-processing filter (CE5-2.2 test), but with the proposed simplifications, CC-ALF could certainly be retained as an in-loop filter.

In CE5-2.1, the filter coefficients are represented and used a 7 bits fixed-point decimal precision. In addition, the limit on the range of values that the filter coefficients can take, is limited by 8 bits signed integer (7 bits precision + sign). So the filter coefficients can lie in the range [-1.0, 0.9921875]. (In JVET-O0636 [4], “the filter coefficients are transmitted in the APS, scaled by a factor of 210, and rounded for fixed point representation”, so there was 10 bits fixed-point decimal precision, and no specific limit on coefficients range).

The filtering process is using a linear combination between the filter coefficients and 18 Luma samples, for each filtered Chroma sample. So, there are 18 multiplications and 17 additions implied in the linear combination for the filtering of one Chroma sample.

With the current design, each multiplication operation of the linear combination is taking as inputs one unsigned integer that fits in – the internal bit-depth for Luma – (IBDL) bits (typically 10 bits) and one signed integer that fits in 8 bits (7 precision bits + 1 sign bit).

The results of each multiplication then fits in IBDL+8 bits (typically 18 bits). The linear combination implies the addition of 18 multiplication results, and so, 5 additional bits are required to store the intermediate results. To guaranty no overflow 22 bits + 1 sign bit are then required with a 10-bits internal bit-depth.

So, typically 32bits memory area and/or register will be used to store/compute the sum, and 16-bits by 16-bits multiplications with 32-bits results will be used for 10-bits internal bit-depth, while 8-bits signed by 8-bits unsigned multiplication with 16-bits result could be used for 8-bits internal bit-depth (the sum still being computed within 32 bits).

In this contribution, alternative filter designs are proposed to reduce the number of operations and/or the number of bits used in each operation.

In CE5-2.1, there is no consideration of the internal bit-depth that is used for Luma samples or for Chroma samples. In this contribution it is also suggested to take into account the difference of bit-depth between Luma channel and Chroma channels, to keep similar range of coefficient precision relatively to the filtered channel (Chroma channel), and keep efficient coding in that situation. It allows to improve the coding efficiency of CCALF in the cases where the internal bit-depth for Chroma is lower than the internal bit-depth for Luma.

# CC-ALF filter

In JVET-O0636 [3], a sample at a positionxc,yc, in Cb Chroma component, is filtered/corrected by adding to its value the amount:

Oxc,ycCb=f0Cb.Ix,y-2

+f1Cb.Ix-1,y-1+f2Cb.Ix,y-1+f3Cb.Ix+1,y-1

+f4Cb.Ix-2,y+f5Cb.Ix-1,y+f6Cb.Ix,y+f7Cb.Ix+1,y+f4Cb.Ix+2,y

+f4Cb.Ix-2,y+1+f8Cb.Ix-1,y+1+f9Cb.Ix,y+1+f10Cb.Ix+1,y+1+f4Cb.Ix+2,y+1

+f11Cb.Ix-1,y+2+f12Cb.Ix,y+2+f13Cb.Ix+1,y+2

+f0Cb.Ix,y+3.													(1)
where fiCb are filter coefficients with index i of the filter for Cb Chroma component. Figure 1 illustrates coefficients index positioning in the filter shape. The filter shape is positioned on Luma samples relatively to a reference sample (grey dotted background sample in Figure 1).

A similar filtering is applied to samples of Cr component, but using fiCr filter coefficients.

Figure 1 – CCALF filter shape in JVET-O0636 [3]

The notation is simplified by using the following formula notation to obtain the same filtering:

Oxc,ycCb=ifui,viCb.Ix+ui,y+vi.									(2)
where fui,viCb is the filter coefficient to apply with sample Ix+ui,y+vi in the filter shape at offset ui,vi of the reference sample (grey dotted background sample in Figure 1) which has a position x,y. In common test conditions (CTC) [3], x,y=2xc,2yc for YUV 4:2:0 content.

The implementation of equation (2) is performed using integer operations for approximation of real number operation with fixed point computation. Equation (2) can then be rewritten as:

Oxc,ycCb=if'ui,viCb.Ix+ui,y+vi+1≪B-1≫B,				(3)
where B is the number of bits for the decimal precision (7 bits in CE5-2.1) of the fixed point representation of the filter coefficients f'ui,viCb; and Ix+ui,y+vi is integer sample value with a integer precision equal to the internal bit depth for Luma (IBDL) samples (10 bits in CTC).

# Filter changes

## Handling different internal bit-depth

A first proposed change in the filter design is to consider the difference between the internal bit-depth for Luma (IBDL) channel, and the internal bit-depth for the Chroma (IBDC) channel that is being filtered. Then equation (3) is rewritten as:

Oxc,ycCb=if'ui,viCb.Ix+ui,y+vi+1≪B1-1≫B1,				(4)
where B1=B+IBDL-IBDC. Then, for a given Chroma channel content, the filter coefficient values would be the same for any bit depth scaling used on Luma samples. B is then the fixed point decimal precision, relatively to Luma channel samples that would be expressed with the same internal bit-depth as the Chroma channel samples. This is equivalent to using f''ui,viCbfilter coefficients in (3) instead of f'ui,viCb with f''ui,viCb=f'ui,viCb/2IBDL-IBDC; but encoding f'ui,viCb values in the bitstream, thus keeping better decimal precision for the filter coefficients, when needed.

## Filter coefficients precision

In comparison to JVET-O0636, the number of bits used for decimal precision (B) of fixed-point representation of filter coefficients can be reduced without affecting too much the coding gains of CCALF. Sometimes, it can either increase coding gains, because additional precision bits must be coded, but difference in filtering results is just noise. In CE5-2.1, (JVET-P0080 [6]), the fixed point decimal precision has then been reduced to 7 bits.

Using 7 bits precision may provide equivalent coding gains with common test conditions (CTC) [3], where both Luma and Chroma use same bit-depth (10 bits). However, with 10 bits internal bit-depth for Luma and 8 bits internal bit-depth for Chroma, it has been observed by the proponents of the current contribution that the coding gains of CCALF are reduced by around ~0.1% when using 7-bits decimal precision, compared to using 8-bits decimal precision (or more). While with 8-bits decimal precision, the CCALF coding gains are roughly equivalent to using 7-bits in CTC.

In this contribution, it is then recommended to use B=8 instead of B=7.

## Filter coefficients range

Rather than allowing to have any filter coefficient value representable more than 10 bits, as in JVET-O0636, with the fixed decimal precision (B), in this contribution it is recommended to limit the value in a smaller range, to reduce the number of bits to use in the multiplication operations. This is feasible without affecting too much the filtering/coding performances of CCALF, because optimal CCALF coefficients values are often small. With B=7, as is shown by the results of the CE5-2.1, it is possible to restrict the filter coefficients so that their fixed point representation fits in a signed 8-bits. In this contribution, as will be illustrated by the results obtained in section 5, it is possible to restrict the filter coefficients so that the fixed point representation fits in a signed 8 (or even less) bits integer values, even if B=8 is used, without affecting too much CCALF efficiency.

In the proposed configurations (see section 4) it is even suggested to reduce the coefficients’ range so that coefficients fit in 6 or 7 bits (5 or 6 bits plus one sign bit).

## Sample value precision

Rather than using the full bit-depth precision for the samples in the multiplication operations, it is possible to restrict the precision used for the sample values at the input of the multiplication operation, without affecting too much the coding gains of CC-ALF. This is because CC-ALF filter coefficients are small, and they are mostly meaningful with the highest bits of the sample values. As will be shown by the results obtained in section 5, it is possible to restrict the sample amplitude precision by only using the most significant bits and make them fit in an unsigned 8 (or even less) bits integer value.

In the proposed configurations (see section 4), it is suggested to reduce the samples precision in the multiplication so that it fits in 5 or 6 bits. This is particularly useful for decoder design, because whatever the internal bit-depth is, the same number of bits will always be used for the CC-ALF filter operations.

From the below experiments, it is recommended to restrict the number of bits to not less than 6 to not decrease CCALF efficiency. The number of bits can even be restricted to 5, but it may decrease CC-ALF BDR gains more significantly.

## Multiplication precision

By combining the restrictions on coefficients precision (section 3.2), on coefficients range (section 3.3) and on sample precision (section 3.4), it is possible to restrict the multiplication inputs so as to have a multiplication output that fits in signed 16-bits integer, or less, such that all filtering additions can also fit into signed 16-bits integer.

It is possible to use 8-bits inputs instead of 16-bits to simplify the filter design and use simpler operations than in JVET-O0636 [4] or in CE5-2.1 [5][6] without decreasing the coding efficiency of CCALF.

Note that it exists __m128i _mm_maddubs_epi16 (__m128i a, __m128i b)and __m256i _mm256_maddubs_epi16 (__m256i a, __m256i b) SSE3/AVX2 intrinsics SIMD instructions [6] to “Vertically multiply each unsigned 8-bit integer from ‘a’ with the corresponding signed 8-bit integer from ‘b’, producing intermediate signed 16-bit integers. Horizontally add adjacent pairs of intermediate signed 16-bit integers, and pack the saturated results in ‘dst’”. In the proposed configurations (see section 4) the output of the multiplication is always less than 15-bits so saturation of the addition will never occur and then it is possible to use such instructions, for instance, to achieve high throughput on CCALF filter operations on SSE3/AVX2 capable CPUs (16 multiplications and 8 additions at once for SSE3 instruction, and 32 multiplications and 16 additions at once for AVX2 instruction). The same instruction also exists in AVX-512 instruction set for double size registers and then allows 64 multiplications and 32 additions at once on architectures that supports that instruction set.

## Filter shape, number of operations and additions precision

In JVET-O0636 [4] and in CE5-2.1 [5][6], the filter uses 18 samples. Then, it is needed to reserve 5 additional bits to store the result of the sum of the multiplication outputs. In this contribution, it is proposed to reduce the size of the filter shape to 16 samples, to decrease that number of additional bits by one (and reduce the number of multiplications/additions by 2). Alternatively, it is proposed to reduce the size of the filter shape to 8 samples, to decrease the number of additional bits by two (and reduce the number of multiplications/additions by 10).

### Proposed 16-samples filter shape

By reducing the filter shape to use 16 samples, it is possible to save two multiplication operations and one bit for storing the result of the sum. It is then proposed to use the 16 samples filter shape of Figure 2 to simplify CCALF design while keeping similar coding efficiency as CCALF in CE5-2.1.

Figure 2 – proposed 16-samples CCALF filter shape

### Proposed 8-samples filter shape

Alternatively, it is proposed to use the 8 samples filter shape of Figure 3 to further simplify CCALF design by reducing the number of operation to 8 multiplications and 7 additions, and so further reducing by one the number of bits required to store the sum. But, using this shape also slightly decreases the coding gains of CCALF.

Figure 3 – proposed 8-samples CCALF filter shape

# Experiments

The results obtained with the CE5-2.1 tests (reference configuration) are compared with the results obtained with three different configurations proposed as complexity/coding gains trade-off.

## Reference configuration (CE5-2.1 results to compare with)

- Original 18 samples filter shape

- 10-bits fixed point decimal precision (B=10),

- Full sample precision (10 bits in CTC)

- 16-bits for coefficient value

- 18 multiplications (16-bits inputs, 32-bits output)

- 17 additions (32-bits)

- 32-bits filtering result

## Proposed configuration 1

- Original 18 samples filter shape

- 8-bits fixed point decimal precision (B=8),

- 5 (highest) bits kept for a Luma sample value in multiplication,

- 5-bits kept for a filter coefficient value, plus sign bit (coefficients value in range [-0.12109375, 0.12109375], i.e. integer representation of the fixed point values in the range [-31,31])

- 18 multiplications (8-bits inputs, 16-bits output)

- 17 additions (16-bits)

- 16-bits filtering result

## Proposed configuration 2

- 16-samples filter shape,

- 8-bits fixed point decimal precision (B=8),

- 6 (highest) bits kept for a Luma sample value in multiplication,

- 5-bits kept for a filter coefficient value, plus sign bit (coefficients value in range [-0.12109375, 0.12109375], i.e. integer representation of the fixed point values in the range [-31,31])

- 16 multiplications (8-bits inputs, 16-bits output)

- 15 additions (16-bits)

- 16-bits filtering result

## Proposed configuration 3

- 8-samples filter shape,

- 8-bits fixed point decimal precision (B=8),

- 6 (highest) bits kept for a Luma sample value in multiplication,

- 6-bits kept for a filter coefficient value, plus sign bit (coefficients value in range [-0.24609375, 0.24609375], i.e. integer representation of the fixed point values in the range [-63,63])

- 8 multiplications (8-bits inputs, 16-bits output)

- 7 additions (16-bits)

- 16-bits filtering result

# Results

The proposed configurations have been implemented on top of VTM-6.0 software, by modifying the implementation of CE5-2.1 Test [5][6]. Results of each configuration is reported using JVET common test conditions [3]. To illustrate the equivalent Luma gain, the YUV BDR measure in the results below has been computed with the following weights: YUV = (14*Y +U +V)/16.

## Results of Configuration 1

## Results of Configuration 2

## Results of Configuration 3

# Conclusion

Considering the coding gains of CC-ALF, and the proposed simplifications that do not affect its significant coding efficiency in Chroma, it is recommended to consider the CC-ALF as an in-loop filter for the VVC while including the proposed simplifications to perform the filtering by using 8-bits/16-bits operations instead of 16-bits/32-bits operations, and using filter shapes with at most 16 samples.

Different complexity/gains trade-off have been proposed regarding the size of the filter shape.

# References

- Meeting Report of the 15th JVET Meeting (Gothenburg, SE, 3–12 July 2019), JVET-O2000, G. J. Sullivan, J.-R. Ohm

- Versatile Video Coding (Draft 6), Document JVET-O2001_v7, B. Bross, J. Chen, S. Liu

- JVET common test conditions and software reference configurations for SDR video, JVET-N1010

- Cross-Component Adaptive Loop Filter for Chroma, JVET-O0636, K. Misra, F. Bossen, A. Segall

- Description of Core Experiment 5 (CE5): Loop filtering, JVET-O2025, C.-Y. Chen, A. Norkin

- CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter, JVET-P0080, K. Misra, F. Bossen and A. Segall.

- https://software.intel.com/sites/landingpage/IntrinsicsGuide/#text=maddubs

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).