| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P00332 |
| --- | --- |

| Title: | Non-CE5 : Parsability of ALF Chroma filter alternative signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

With the adoption at the last meeting around the signaling of alternative ALF chroma filters, it is asserted that one parsing issue has been introduced. In VTM6.0, an alternative chroma filter index can be encoded per chroma CTB. This chroma filter index depends on the number of available chroma filter alternatives, which is indicated in the APS. If the APS is lost, it is not possible to properly parse the alternative chroma filter index, because the number of available chroma filter alternatives is not known.

This contribution suggests to duplicate the information about the number of available chroma filter alternatives, in the slice header. The proposed changes leads to an average of 0.00% coding losses and 100% encoding/decoding time, in AI/LB/LP/RA configurations.

# Problem statement

During the 14th JVET Meeting [1], the use and signaling of alternative ALF chroma filters [3], which was studied in CE5-4.2 test [4], has been adopted. Alternative Chroma filters can be signaled in APS, and the index of a selected alternative Chroma filter can be signaled per CTB.

There is one parsability issue with the current design. As the alternative chroma filter index is encoded depending on the number of available chroma filter alternatives, if the APS is lost, a decoder may be lost when decoding alternative index, because the number of available chroma filter alternatives is not known.

# Proposed solution

To address this issue, it is suggested to duplicate in the slice header the information on the number of available chroma filter alternatives.

## Draft text

It is recommended to integrate the following highlighted text modifications in VVC draft text next to [2]

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) { |  |
| slice_alf_aps_id_chroma | u(3) |
| slice_alf_chroma_num_alt_filters_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

slice_alf_chroma_num_alt_filters_minus1 plus 1 specifies the number of alternative filters for chroma components. It must be set equal to alf_chroma_num_alt_filters_minus1 in the ALF APS that the chroma component of the slice refers to with slice_alf_aps_id_chroma.

Parsing process (Table 9-77 - Syntax elements and associated binarizations):

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| slice_data( ) | end_of_brick_one_bit | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
|  | alf_ctb_use_first_aps_flag | FL | cMax = 1 |
|  | alf_use_aps_flag | FL | cMax = 1 |
|  | alf_luma_fixed_filter_idx | TB | cMax = 15 |
|  | alf_luma_prev_filter_idx_minus1 | TB | cMax = slice_num_alf_aps_ids_luma − 2 |
|  | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = slice_alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |
| … |  |  |  |

# Results

Proposed changes have been implemented in VTM-6.0, and are compared to VTM-6.0 using JVET common test conditions [5]. The results of the proposed solution are provided below and shows that there is no impact on the coding efficiency.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.01% | 0.01% | 0.01% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.01% | 0.01% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 99% | 102% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 101% |

# Conclusion

It is recommended to fix the parsibility issue identified for the signalling of the ALF Chroma filter index signaling as proposed in this contribution since there is no impact on the coding efficiency.

# References

- Meeting Report of the 15th JVET Meeting (Gothenburg, SE, 3–12 July 2019), JVET-O2000, G. J. Sullivan, J.-R. Ohm

- Versatile Video Coding (Draft 6), Document JVET-O2001_v7, B. Bross, J. Chen, S. Liu

- CE5-4: alternative luma filter sets and alternative chroma filters for ALF, JVET-O0090, J. Taquet, P. Onno, C. Gisquet, G. Laroche

- CE5: Summary Report on Loop Filtering, JVET-O0025, A. Norkin

- JVET common test conditions and software reference configurations for SDR video, JVET-N1010

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).