| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0343-v2 |
| --- | --- |

| Title: | Flexible maximum MTT hierarchy depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, F. Galpin, T. Poirier, K. Naser, P. Delagrange | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VTM-6, the maximum multi-type tree (mtt) depth is signaled in the SPS and slice header as a fixed value, respectively for inter slice, intra luma slice and intra chroma slice (case of dual tree active for I slices). Thus, the maximum mtt hierarchy depth is fixed whatever the quad-tree leaf that serves as a root for a nested multi-type tree.  Moreover, the minimum QT size and maximum MTT depth are never changed in the JVET CTC, leading to the repetition of redundant data in the slice header.

This contribution includes two aspects. First aspect explicitly signals the overriding of the minimum QT size and maximum MTT depth in the slice header.

Second aspect proposes to increase the flexibility in the normative allocation of the maximum mtt hierarchy depth. This is achieved by signaling a maximum mtt hierarchy depth for each quad-tree level at which multi-type tree splitting is allowed.

It is shown that the proposed HLS modification (SPS and slice header) allows the VVC encoder to reach some BD-rate/enc. time trade-offs which are not reachable with VTM-6.

# Introduction

In VTM-6, four SPS syntax elements are dedicated to specifying the maximum multi-type tree depth:

- sps_log2_diff_min_qt_min_cb_intra_slice_luma

- sps_max_mtt_hierarchy_depth_intra_slice_luma and

- sps_log2_diff_max_bt_min_qt_intra_slice_luma.

- sps_log2_diff_max_tt_min_qt_intra_slice_luma.

These syntax elements are coded for the luma component of Intra slices. Their analogous parameters are specified for inter slices, and the chroma component of intra slices in case of dual tree coding.

These 4 SPS parameters can be overridden by their analogous syntax elements in the slice header:

- slice_log2_diff_min_qt_min_cb_luma

- slice_max_mtt_hierarchy_depth_luma

- slice_log2_diff_max_bt_min_qt_luma

- slice_log2_diff_max_tt_min_qt_luma

In the JVET CTC, among these parameter, the only parameters that is being overridden in the slice header are the syntax elements slice_log2_diff_max_bt_min_qt_luma and slice_log2_diff_max_tt_min_qt_luma. Other parameters are not modified.

This contribution includes two aspects.

- Test 1: first aspect consists in reducing the amount of redundant data coded in the slice headers, making the slice header syntax more compact.

- Test 2: Second proposes to specify, for each of the B, I-Luma and I-Chroma slice-components, it is proposed to specify several maximum mtt hierarchy depths by indexing them with the quad-tree depth value (or equivalently Log2QTSize value).

With respect to the intra slice luma case, for each quad-tree level that corresponds to a quad-tree node with log2 size comprised between MinQtLog2SizeIntraY and (1 << log2_diff_max_bt_min_qt_intra_slice_luma), a maximum MTT hierarchy depth is signaled, where:

log2_diff_max_bt_min_qt_intra_slice_luma = max ( sps_log2_diff_max_bt_min_qt_intra_slice_luma, sps_log2_diff_max_tt_min_qt_intra_slice_luma )

Concerning the case inter slice and intra slice chroma, similar specification is proposed.

# Test 1 description

The first aspect consists in adding a flag in the slice header, which signals if the Min QT size and maximum MTT depth are being overridden in the considered slice. This takes the form of the flag max_hierarchy_depth_override_flag of Table 1 below. The maximum BT size and maximum TT size information is signaled regardless the value of the maximum multi-type tree depth.

Moreover, the flag max_hierarchy_depth_override_flag also control the signaling of the minimum QT size and the maximum MTT depth for the chroma component, in case dual tree is active for the considered slice.

Table 1: slice header syntax table proposed in the first aspect of this contribution

| Slice_header( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| max_hierarchy_depth_override_flag | u(1) |
| If( max_hierarchy_depth_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| if( max_hierarchy_depth_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| …. |  |
| byte_alignment( ) |  |
| } |  |

# Test 2 description

The proposed second aspect is built onto the first one. It consists in signaling the maximum multi-type tree depth information as an array of maximum MTT depths, indexed by the quad-tree depth. The second aspect modified the VVC syntax both in the SPS and in the slice header. The modified SPS syntax is depicted by the following Table 2.

Table 2: SPS syntax table of VVC draft 6, modified according to second aspect.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_ctu_size_minus5 | u(2) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slices != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| for(i= start_qt_depth_inter_slice ; i <= max_qt_depth_inter_slice ; i++) { |  |
| sps_max_mtt_hierarchy_depth_inter_slice[i] | ue(v) |
| } |  |
| for(i= start_qt_depth_intra_slice_luma ; i <= max_qt_depth_intra_slice_luma ; i++) { |  |
| sps_max_mtt_hierarchy_depth_intra_slice_luma[i] | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if ( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| for(i= start_qt_depth_intra_slice_chroma; i <= max_qt_depth_intra_slice_chroma; i++) { |  |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma[i] | ue(v) |
| } |  |
| } |  |
| …. |  |
| rbsp_trailing_bits( ) |  |
| } |  |

In Table 2, start_qt_depth_inter_slice  is defined as:

start_qt_depth_inter_slice = CtbLog2SizeY-max(MaxBtLog2SizeY, MaxTtLog2SizeY )

where:

MaxBtLog2SizeY = ( MinQtLog2SizeInterY + sps_log2_diff_max_bt_min_qt_inter_slice)

MaxTtLog2SizeY = ( MinQtLog2SizeY + sps_log2_diff_max_bt_min_qt_inter_slice)

Moreover, max_qt_depth_inter_slice is defined as:

max_qt_depth_inter_slice = CtbLog2SizeY - MinQtLog2SizeInterY

Furthemore, start_qt_depth_intra_slice_luma and max_qt_depth_intra_slice_luma are defined similarly to start_qt_depth_inter_slice and max_qt_depth_inter_slice.

Finally, start_qt_depth_intra_slice_chroma, max_qt_depth_intra_slice_chroma are defined similarly to start_qt_depth_inter_slice and max_qt_depth_inter_slice.

The slice header syntax table is modified according to the proposed flexible max MTT depth signaling, exactly in the same way as for the SPS syntax table. This is depicted by Table 5.

Table 5: slice header syntax table of VVC draft 6, modified according to the proposed flexible maximum MTT depth.

| slice_header( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| max_hierarchy_depth_override_flag | u(1) |
| If( max_hierarchy_depth_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| for(i= start_qt_depth ; i <= max_qt_depth ; i++) { |  |
| sps_max_mtt_hierarchy_depth_luma[i] | ue(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| If( max_hierarchy_depth_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| for(i= start_qt_depth_chroma ; i <= max_qt_depth_chroma ; i++) { |  |
| sps_max_mtt_hierarchy_depth_chroma [i] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| …. |  |
| } |  |
| …. |  |
| byte_alignment( ) |  |
| } |  |

# Experimental Results

## Test 1 results

The following Table 3 shows the results of the proposed first aspect over the VTM6. It can be seen that up to 0.1% average bitrate reduction can be obtained in small resolution class D sequences thanks to the more compact slice header syntax proposed.

Table 3: results of test 1 over the VTM6

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.02% | -0.02% | 97% | 100% |
| Class C | -0.03% | -0.03% | -0.03% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.09% | -0.09% | -0.10% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test 2 on top of Test 1 results

The proposed method has been tested in VTM6 in JVET SDR coding test conditions [ 2]. Table 4 shows the impact of the proposed flexible signaling of the maximum multi-type tree depth information. It can be observed that the codec performances are almost unchanged inside the CTC.

Table 4: performances obtained with the proposed flexible maximum MTT hierarchy depth signaling in the JVET CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.02% | 0.02% | 98% | 99% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | 0.06% | 0.06% | 99% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | -0.03% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | -0.08% | -0.08% | 98% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

shows the different trade-offs between coding efficiency and encoding time that can be achieved by the VTM6 (blue) and by the proposed flexible max MTT depth specification.  As can be seen, the proposed flexibility increase allows providing a richer set of trade-offs, which are competitive compression-wise with the VTM6.

Figure 1: BD-rate trade-off achieved by the TM encoder with various maximum MTT depths configurations. Blue: trade-offs obtained with maximum MTT depth equal to 2, 3 and 4 with VTM-6. Red: exemplary BD-rate/enc.time trade-offs obtained with the proposed flexible MTT depth signaling.

# Conclusion

First aspect proposes to introduce a new flag in the slice header max_hierarchy_depth_override_flag which signals the overriding of several coding tree parameters: the minimum quad-tree leaf size and the maximum multi-type tree depth.

Second aspect proposes to signal the multi-type tree configuration with increased flexibility compared to VTM6. This is achieved thanks to the signaling of a maximum multi-type tree hierarchy depth for each quad-tree level, which corresponds to a quad-tree node size that can be used as a root for a nested multi-type tree. It is shown that a richer set of compression/encoder complexity trade-offs can be easily obtained with the proposed SPS and slice header signaling.

# References

[ 1] B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[ 2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, April 2019

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).