| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0346 |
| --- | --- |

| Title: | Non-CE6: SPS Clean-up of SBT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Tangi POIRIER Fabrice LE LEANNEC Frank GALPIN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Sub-Block Transform (SBT) is controlled by an SPS parameter (sps_sbt_enabled_flag) to activate/deactivate it. If activated, another SPS parameter (sps_sbt_max_size_64_flag) is coded to indicate if max SBT size is 64 or 32. However, due to the adoption of JVET-O0545, max SBT size is restricted to the maximum allowable transform size. Therefore, if the maximum transform size is set to 32, maximum SBT size is restricted to 32 regardless of the SPS parameter sps_sbt_max_size_64_flag. This contribution proposes to code sps_sbt_max_size_64_flag only if the maximum transform size is 64, otherwise max SBT size is inferred to be zero.

# Introduction

In VVC draft 6 specification, sps_sbt_enabled_flag and sps_sbt_max_size_64_flag  are coded as followes:

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |
| } |  |

Where sps_sbt_enabled_flag controls SBT (0 deactivated and 1 activated) and sps_sbt_max_size_64_flag specifies if the maximum width and height of CU’s allowing SBT are 64 or 32 (0 means 32 and 1 means 64).

According to VVC specification, the maximum SBT size is computed as:

MaxSbtSize = Min( MaxTbSizeY, sps_sbt_max_size_64_flag ? 64 : 32 )	(7-32)

Where sps_max_luma_transform_size_64_flag is a recently adopted flag ‎[1] to control the maximum transform size (0 means 32 and 1 means 64). When sps_max_luma_transform_size_64_flag is 0, MaxSbtSize is restricted to 32 even if sps_sbt_max_size_64_flag is signaled as 1. That is, the current specification codes redundant SPS parameter and also mandates the encoder and decoder to perform redundant minimum operation.

# Proposal

It is proposed to code sps_sbt_max_size_64_flag only if maximum transforom size is 64, and infer it be zero otherwise. The subsequent minimum function is also removed. The corresponding specification changes are shown below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag && sps_max_luma_transform_size_64_flag) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |
| } |  |

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples. When not present, the value of sps_sbt_max_size_64_flag is inferred to be equal to 0.

MaxSbtSize = Min( MaxTbSizeY, sps_sbt_max_size_64_flag ? 64 : 32 )	(7-32)

# References

- X. Zhao, X. Li, S. Liu, “Non-CE6: Configurable maximum transform size in VVC”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- O0545, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).