| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0349 |
| --- | --- |

| Title: | CE6 Related: MIP Adaptation On Top of CE6-1.1d |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Frank GALPIN Tangi POIRIER Ya CHEN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In CE6-1.1d test of JVET-P0041, a fast LFNST implementation is proposed by allowing only one LFNST kernel per intra-prediction mode, and alternate LFNST kernels among even and odd modes. It is reported that this method reduces the encoder time to 90% with AI luma loss of 0.11%. Since MIP is considered as planar mode when applying LFNST, MIP predicted blocks are only allowed to use LFNST kernel corresponding to even mode, therefore, less flexibility for the encoder compared with regular intra predicted blocks. This contribution proposes to extend the kernel alternation for MIP modes with the same even-odd distribution as in CE6-1.1d. It is reported that the encoder time is similar to CE6-1.1d while the AI luma loss is 0.10%.

# Introduction

CE6-1.1d proposes signalling only one flag (lfnst_idx) that enables LFNST for a CU. If lfnst_idx is true, LFNST is applied. LFNST transform matrix selection is derived from the intra prediction mode as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

The corresponding specification change is:

8.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set is specified as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

According to the spec, predModeIntra is set to INTRA_PLANAR if MIP is used:

### 8.7.4 Transformation process for scaled transform coefficients

#### 8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-965)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-966)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-967)

nLfnstSize = 1  <<  log2LfnstSize		(8-968)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-969)

- When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

That is, although the proposed method in CE6-1.1d tries to alternate the kernels between the intra prediction modes, predModeIntra is set to (INTRA_PLANAR = 0) when MIP is used and therefore the kernel selection becomes: lfnstIdx = ( 0 % 2 ) + 1 = 1. That is, always the first kernel. This reduces the flexibility of the encoder to test different kernels and leads to redundant computation of lfnstIdx when MIP is used.

# Proposal

It is proposed to consider the actual MIP prediction mode when calculating lfnstIdx rather and map the mode to planar mode for selecting the LFNST group. This can be done by moving the computation of lfnstIdx before the mode mapping. The specification change is as follows:

### 8.7.4 Transformation process for scaled transform coefficients

#### 8.7.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-965)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-966)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-967)

nLfnstSize = 1  <<  log2LfnstSize		(8-968)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-969)

- When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Lfnst index is modified as: lfnstIdx = ( predModeIntra % 2 ) + 1

When intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

# Experimental Results

The proposed methods have been implemented on top of VTM6.0. The tests were conducted under the CTC. The results are shown in the table below:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.77% | 1.05% | 91% | 100% |
| Class A2 | 0.03% | 0.73% | 0.75% | 89% | 101% |
| Class B | 0.08% | 0.71% | 0.80% | 90% | 100% |
| Class C | 0.11% | 0.40% | 0.69% | 91% | 103% |
| Class E | 0.13% | 0.65% | 0.68% | 92% | 101% |
| Overall | 0.10% | 0.65% | 0.79% | 90% | 101% |
| Class D | 0.04% | 0.44% | 0.58% | 92% | 107% |
| Class F | 0.10% | 0.32% | 0.38% | 92% | 96% |

# Conclusion

The proposed method extends the idea proposed in CE6-1.1d of alternating LFNST kernels among the intra prediction modes to MIP prediction modes. Instead of redundantly computing lfnst index when MIP is used, the method proposes to use the actual MIP prediction modes. Compared to CE6-1.1d, the AI luma loss due to using one kernel is reduced from 0.11% to 0.10%.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).