| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0356 |
| --- | --- |

| Title: | AHG17: Bitstream constraints on RPL and GDR |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution claims that seven bitstream constraints are needed to be added to VVC in order for reference picture lists (RPLs) and gradual decoding refresh (GDR) to operate properly. The constraints proposed to the VVC draft are as follows, shown in italic font:

- Active RPL entries shall not be “no reference picture”

  - No active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be equal to “no reference picture”.

- RPL entries of a CRA picture shall not precede in output order or decoding order any previous IRAP picture in decoding order

  - When the current picture is a CRA picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

- Active RPL entries of a trailing picture shall not be unavailable picture generated for the associated IRAP picture

  - When the current picture is a trailing picture there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture as specified in clause 8.3.4.

- Pictures that are recovered after a GDR picture shall not use unavailable pictures for reference that were generated for the GDR picture

  - When the current picture is a recovery point picture or a picture that follows the recovery point picture in decoding order, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the GDR picture of the recovery point picture as specified in clause 8.3.4.

- RPL entries of a trailing picture shall not precede the associated IRAP in output order or decoding order

  - When the current picture is a trailing picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order.

- RADL pictures shall not reference any RASL pictures, unavailable pictures or pictures that precede the associated IRAP picture in decoding order

  - When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

    - A RASL picture

    - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in clause 8.3.4

    - A picture that precedes the associated IRAP picture in decoding order

- GDR picture recovery shall result in the same output as if decoding started at an earlier IRAP picture or an earlier GDR picture

  - When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the associated GDR picture, the following applies:

    - It is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at the previous IRAP picture, if any,  preceding the associated GDR picture in decoding order.

    - It is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at any GDR picture, if any, for which its recovery point picture precedes the current picture in decoding order.

In addition, it is proposed to remove the following sentences from the notes on the definitions of RADL and RASL pictures since this is claimed to not be true according to an adoption at the previous meeting in Gothenburg when field_seq_flag is equal to 1:

- When present, all RADL pictures precede, in decoding order, all trailing pictures of the same associated IRAP picture.

- When present, all RASL pictures precede, in decoding order, all trailing pictures of the same associated CRA picture.

The proposed specification text relative to JVET-O2001-vE.docx is provided with the contribution.

# Proposal

In this contribution we propose to add six constraints related to reference picture lists and one constraint related to gradual decoding refresh (GDR).

## Constraint 1 – Active RPL entries shall not be “no reference picture”

| Proposed VVC text | No active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be equal to “no reference picture”. |
| --- | --- |
| Corresponding HEVC text | There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true: - The entry is equal to "no reference picture". - ... |

In section 8.3.2 of the VVC draft, RefPicList[ i ][ j ] is set to “no reference picture” for those cases that a picture included in an RPL is not present in the DPB. This may happen in the case a temporal sub-layer has been removed.

There is a note 4 in the VVC draft saying: “NOTE 4 – There may be one or more entries in RefPicList[ 0 ] or RefPicList[ 1 ] that are equal to "no reference picture" because the corresponding pictures are not present in the DPB. Each inactive entry in RefPicList[ 0 ] or RefPicList[ 0 ] that is equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is equal to "no reference picture".”. However, this is only a note and is therefore not normative.

We propose to require VVC bitstreams to not include active entries in RPL that results in RefPicList[ i ][ j ] being set to “no reference picture”.

## Constraint 2 – RPL entries of a CRA picture shall not precede any previous IRAP

| Proposed VVC text | When the current picture is a CRA picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present). |
| --- | --- |
| Corresponding HEVC text | When the current picture is a CRA picture, there shall be no picture included in the RPS that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present). |

It seems currently to be allowed in the VVC draft for an RPL of a CRA picture to include pictures that precede the IRAP picture preceding the CRA picture. This is not allowed in HEVC and we propose to disallow it also for VVC.

## Constraint 3 – Active RPL entries of a trailing picture shall not have been generated

| Proposed VVC text | When the current picture is a trailing picture there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture as specified in clause 8.3.4. |
| --- | --- |
| Corresponding HEVC text | When the current picture is a trailing picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that was generated by the decoding process for generating unavailable reference pictures as specified in clause 8.3.3. |

It seems currently to be allowed in the VVC draft for a trailing picture to reference an unavailable reference picture. Remember that unavailable reference pictures in VVC are generated when random access is done on a CRA picture and there are RASL pictures that reference pictures that precede the CRA in decoding order. We believe that VVC should only allow RASL picture to reference those unavailable reference pictures.

Note that we propose that trailing pictures are allowed to reference unavailable pictures that were generated for GDR pictures since allowing that is necessary for random access on GDR pictures to work.

## Constraint 4 – Pictures that are recovered after a GDR picture shall not use unavailable pictures for reference that were generated for the GDR picture

The current VVC draft does not include any RPL restrictions around generated unavailable pictures. Our opinion is that since the recovery point picture and the picture that follows the recovery point picture should be clean, there is no use allowing any such clean picture to use any unavailable picture for reference. Therefore, we propose to disallow it.

| Proposed VVC text | When the current picture is a recovery point picture or a picture that follows the recovery point picture in decoding order, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the GDR picture of the recovery point picture as specified in clause 8.3.4. |
| --- | --- |
| Corresponding HEVC text | [There is no corresponding HEVC text since normative recovery point pictures and GDR pictures are not part of HEVC] |

## Constraint 5 – RPL entries of a trailing picture shall not precede the associated IRAP

| Proposed VVC text | When the current picture is a trailing picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order. When present, all RADL pictures precede, in decoding order, all trailing pictures of the same associated IRAP picture. When present, all RASL pictures precede, in decoding order, all trailing pictures of the same associated CRA picture. |
| --- | --- |
| Corresponding HEVC text | When the current picture is a trailing picture, there shall be no picture in the RPS that precedes the associated IRAP picture in output order or decoding order. |

It seems currently to be allowed in the VVC draft for a trailing picture to reference a picture that precedes the associated IRAP picture in output order or decoding order.

### The decoding order part

The pictures preceding the IRAP picture in decoding order are not available if a decoder performs a random access and the IRAP picture is the first picture in decoding order it decodes. Therefore, we think that trailing pictures should not be using those for reference.

### The output order part

The pictures preceding the IRAP picture in output order but not in decoding order are the RASL and RADL pictures. We propose to disallow trailing pictures to reference those pictures since that enables a decoder to discard all RASL and RADL pictures and be guaranteed to be able to properly decode the IRAP and trailing pictures.

### A note on notes

There is a NOTE in “3.101 random access decodable leading (RADL) picture” that says:

NOTE – All RADL pictures are leading pictures. RADL pictures are not used as reference pictures for the decoding process of trailing pictures of the same associated IRAP picture. When present, all RADL pictures precede, in decoding order, all trailing pictures of the same associated IRAP picture.

This note assumes that the restriction regarding output order is already in the VVC draft but we were not able to find it. Also, the note says: “When present, all RADL pictures precede, in decoding order, all trailing pictures of the same associated IRAP picture.” This is not true, since the VVC draft contains the following:

“If field_seq_flag is equal to 0 and the current picture is a leading picture associated with an IRAP picture, it shall precede, in decoding order, all non-leading pictures that are associated with the same IRAP picture. Otherwise, let picA and picB be the first and the last leading pictures, in decoding order, associated with an IRAP picture, respectively, there shall be at most one non-leading picture preceding picA in decoding order, and there shall be no non-leading picture between picA and picB in decoding order.”

Similarly, there is a note on the definition of RASL pictures saying: “When present, all RASL pictures precede, in decoding order, all trailing pictures of the same associated CRA picture.”

We propose to remove those two sentences from the notes.

## Constraint 6 – RADL pictures shall not reference any RASL pictures, unavailable pictures or pictures that precedes the associated IRAP picture in decoding order

| Proposed VVC text | When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following: - A RASL picture - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in clause 8.3.4 - A picture that precedes the associated IRAP picture in decoding order |
| --- | --- |
| Corresponding HEVC text | When the current picture is a RADL picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is any of the following: - A RASL picture - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in clause 8.3.3 - A picture that precedes the associated IRAP picture in decoding order |

It seems currently to be allowed in the VVC draft for a RADL picture to use RASL pictures, unavailable pictures and pictures preceding the associated IRAP picture in decoding order for reference.

### RASL

We propose to disallow RADL pictures to use RASL pictures for reference. This seems to be the design intent in VVC since the VVC draft contains the following note texts:

RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures.

It is possible to perform random access at the position of an IRAP access unit by discarding all access units before the IRAP access unit (and to correctly decode the IRAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means not specified in this Specification) when it is referred.

### Unavailable pictures

Similar to the case with RASL pictures, it seems to be the intent according to the second note text that RADL pictures should be decodable when random access is performed on the IRAP picture. We agree with that design intent and propose that a restriction disallowing RADL pictures to use unavailable pictures for reference is added to the VVC draft.

Note that we propose to disallow it for unavailable pictures for both IRAP and GDR pictures. We believe that there is no use-case for a bitstream to use GDR pictures and leading pictures close to each other since GDR pictures are only useful for low-delay coding and leading pictures causes coding delay.

### Pictures preceding the associated IRAP picture in decoding order

For random access on an IRAP to work properly, the RADL picture can not be allowed to use any pictures preceding the associated IRAP picture in decoding order.

## Constraint 7 – GDR picture recovery shall result in the same output as if decoding started at an earlier IRAP picture or an earlier GDR picture

| Proposed VVC text | When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the previous associated GDR picture in decoding order for which there is no IRAP picure between the current picture and the previous GDR picture in decoding order, the following applies: - iIt is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at the previous IRAP picture, if any, preceding the associated GDRcurrent picture in decoding order. - It is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at any GDR picture, if any, for which its recovery point picture precedes the current picture in decoding order. |
| --- | --- |
| Corresponding HEVC text | [There is no corresponding HEVC text since GDR pictures are not part of HEVC] |

The current VVC specification contains the following text:

“When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the previous GDR picture in decoding order for which there is no IRAP picure between the current picture and the previous GDR picture in decoding order, it is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at the previous IRAP picture  preceding the current picture in decoding order.”

We see two problems with this text:

### Problem 1

The current VVC text uses the term “previous GDR picture in decoding order”. An example using overlapping GDR pictures may look as follows:

| POC | Picture type | recovery_point_cnt | RpPicOrderCntVal |
| --- | --- | --- | --- |
| 0 | IDR | - | - |
| 1 | TRAIL | - | - |
| 2 | GDR | 4 | 6 |
| 3 | TRAIL | - | - |
| 4 | GDR | 4 | 8 |
| 5 | TRAIL | - | - |
| 6 | GDR | 4 | 10 |
| 7 | TRAIL | - | - |
| 8 | GDR | 4 | 12 |

Looking at the text with the POC 6 picture as current picture we see the following:

| VVC text | Comment |
| --- | --- |
| When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to RpPicOrderCntVal of the previous GDR picture in decoding order for which there is no IRAP picure between the current picture and the previous GDR picture in decoding order, | PicOrderCntVal of the current picture is equal to 6. the previous GDR picture in decoding order for which there is no IRAP picure between the current picture and the previous GDR picture in decoding order is the picture with POC=4. RpPicOrderCntVal of the previous GDR picture is equal to 8 |
| it is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at the previous IRAP picture preceding the current picture in decoding order. | Since RpPicOrderCntVal is larger than PicOrderCntVal it is possible to interpret the VVC text as there is no requirements on any picture in the example that it shall be an exact match to anything. |

The solution to problem 1 that we propose is to replace “previous GDR picture in decoding order” by “the associated GDR picture” and the “current picture” by “associated GDR picture”. We then propose to remove the condition “for which there is no IRAP picure between the current picture and the previous GDR picture in decoding order” since the existance of an IRAP picture there doesn’t mean the pictures shall not be an exact match.

### Problem 2

The VVC requirement above does not cover the case where a bitstream contains no IRAP pictures at all but only GDR pictures. In this case, there is no previous IRAP picture. Consider the table below, where there are only TRAIL and GDR pictures. Assuming that the current picture is the picture with POC equal to 8, this picture is clean if random access is done at the POC 4 picture. The sample values of the current picture (POC equal to 8) should be identical if decoding started on one of the pictures with POC equal to 0 and 2.

| POC | Picture type | recovery_point_cnt | RpPicOrderCntVal |
| --- | --- | --- | --- |
| 0 | GDR | 4 | 4 |
| 1 | TRAIL | - | - |
| 2 | GDR | 4 | 6 |
| 3 | TRAIL | - | - |
| 4 | GDR | 4 | 8 |
| 5 | TRAIL | - | - |
| 6 | GDR | 4 | 10 |
| 7 | TRAIL | - | - |
| 8 | GDR | 4 | 12 |

We propose to add the following text to cover this case:

“It is a requirement of bitstream conformance that the current and subsequent decoded pictures shall be an exact match to the pictures produced by starting the decoding process at any GDR picture, if any, for which its recovery point picture precedes the current picture in decoding order.”

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).