| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0358-v3 |
| --- | --- |

| Title: | Non-CE3: Improved intra mode coding simplified |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Franck Galpin, Fabrice Urban, 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 gagan.rath@interdigital.com, fabrice.urban@interdigital.com |
| Source: | InterDigital. |  |  |

_____________________________

# Abstract

In this contribution, a simplification of the improved MPM generation in VTM 6.0 is proposed. It is proposed to keep all the design features the same but to simplify the candidate selection in the case when the prediction modes of the above and left neighboring blocks are both directional and unequal. In the first test, one conditional check is removed without losing any generality. In the second test, the remaining two checks are merged in a way to produce less number of checks for more likely selection paths.  It is reported that the proposed method yields 0% coding loss in AI over VTM 6.0 with 100% encoding and 99% decoding time, and that it yields 0% coding loss in RA over VTM 6.0 with 98% encoding and 94% decoding time.

# Introduction

In the JVET meeting held at Gothenburg in July 2019, the unified MPM (UMPM) list in VTM 5.0 [2] was improved by aligning the candidates with respect to the MIP adoption and was adopted into VTM 6.0 [3]. Since the MIP modes were observed to produce non-directional predictions like the DC mode, it was decided to remove the DC mode from the MPM list when at least one of the neighboring blocks (left/above) uses a directional prediction mode [3]. However, as the replacing mode is another directional mode at offset 2 from one of the directional modes, when the neighboring blocks have unequal directional modes, the adjacency check between the two modes became more complex than in the original UMPM. This contribution demonstrates that, in the case when the neighboring blocks have unequal directional modes, these conditional checks can be simplified without losing any generality.

# Problem definition

In VTM 6.0, an improved  MPM list is constructed using the intra modes of the left and above neighbor blocks, and some default modes like the Planar mode, the DC mode, the horizontal mode (H) and the vertical mode (V) [3].  Let the intra modes of the left and above neighboring blocks be denoted as Left and Above. The MPM list is constructed as follows:

- If both Left and Above are non-angular:

  - MPM list  {Planar, DC, V, H, V-4, V+4}.

- If one of  Left and Above is angular, and the other is non-angular:

  - MPM list  {Planar, Max, Max -1, Max +1, Max -2, Max + 2}, where Max = max(Left, Above)

- If Left and Above are both angular and are different:

Let Max ≡ max(Left, Above), Min ≡ min(Left, Above),

  - if Max – Min = 1

MPM list  {Planar, Left, Above, Min -1, Max +1, Min - 2}

  - Otherwise, if Max – Min >= 62

MPM list  {Planar, Left, Above, Min +1, Max -1, Min + 2}

  - Otherwise, if Max – Min = 2

MPM list  {Planar, Left, Above, Min +1, Min -1, Max + 1}

  - Otherwise

MPM list  {Planar, Left, Above, Min -1, Min +1, Max - 1}

- If Left and Above are both angular and equal:

  - MPM list  {Planar, Left, Left -1, Left +1, Left -2, Left + 2}

It is observed that, in the case the prediction modes of the left and above neighboring blocks are directional and unequal (case 3), there are three adjacency checks required to determine the non-planar MPM candidates.

# Proposal

First, comparing the cases 3.c and 3.d, it is observed that 4 of the non-planar candidates are common. There is no loss of generality if the 3rd, 4th and 5th non-planar candidates in 3.d case are taken the same as in 3.c. This will eliminate the conditional check in 3.c.  This will be referred to as Test 1.

- If Left and Above are both angular and are different:

Let Max ≡ max(Left, Above), Min ≡ min(Left, Above),

  - if Max – Min = 1

MPM list  {Planar, Left, Above, Min -1, Max +1, Min - 2}

  - Otherwise, if Max – Min >= 62

MPM list  {Planar, Left, Above, Min +1, Max -1, Min + 2}

  - Otherwise

MPM list  {Planar, Left, Above, Min +1, Min -1, Max + 1}

Second, comparing the cases 3.a and 3.b, it is noticed that the conditional checks are for adjacent or circularly adjacent modes. Therefore, the two check conditions can be merged into just one check condition. As most values of (Max - Min), which has value in the range [1-64], fall under the case 3.d, this change will also render the more likely case to just one check. Combined with Test1, this will be referred to as Test 2. Test 2 is the proposed method and is given below:

- If Left and Above are both angular and are different:

Let Max ≡ max(Left, Above), Min ≡ min(Left, Above),

  - if ( Max – Min + 2) % 64  < 4

swap(Max, Min) if  Max – Min > 1

MPM list  {Planar, Left, Above, Min -1, Max +1, Min - 2}

  - Otherwise

MPM list  {Planar, Left, Above, Min +1, Min -1, Max + 1}

The above proposal requires the following Specification text changes in section 8.4.2 of [1]:

- The candModeList[ x ] with x = 0..4 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-6)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-7)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-8)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-9)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-10)

- Otherwise, if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-11)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-12)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-13)

candModeList[ 1 ] = candIntraPredModeB	(8-14)

  - If ((maxAB – minAB + 2) % 64) is less than  4, the following applies:

  - when (maxAB –  minAB) is greater than 1 minAB and maxAB are swaped

  - If maxAB − minAB is equal to 1, inclusive, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-15)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-16)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-17)

  - Otherwise, if maxAB − minAB is greater than or equal to 62, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-18)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-19)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-20)

  - Otherwise, if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-21)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-22)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-23)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-24)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-25)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-26)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = maxAB		(8-27)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-28)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-29)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-30)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-31)

- Otherwise, the following applies:

candModeList[ 0 ] = INTRA_DC			(8-32)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-33)

candModeList[ 2 ] = INTRA_ANGULAR18	(8-34)

candModeList[ 3 ] = INTRA_ANGULAR46	(8-35)

candModeList[ 4 ] = INTRA_ANGULAR54	(8-36)

# Experimental Results

Simulations were performed according to CTC described in [4]. AI and RA results are reported for both Test 1 and Test 2. Test 2 is the proposal of the contribution.

Table 1. Results of Test 1 over VTM 6.0, AI configuration

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over | VTM-6.0 |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.03% | 101% | 101% |
| Class A2 | 0.00% | 0.02% | 0.00% | 102% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class C | 0.01% | -0.13% | 0.02% | 100% | 101% |
| Class E | -0.02% | 0.10% | -0.01% | 98% | 92% |
| Overall | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class D | -0.01% | -0.02% | -0.05% | 98% | 91% |
| Class F | 0.00% | 0.07% | -0.02% | 97% | 93% |

Table 2. Results of Test 1 over VTM 6.0, RA configuration

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over | VTM-6.0 |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.04% | 97% | 94% |
| Class A2 | 0.01% | 0.05% | 0.08% | 99% | 101% |
| Class B | 0.00% | 0.06% | -0.04% | 97% | 93% |
| Class C | 0.01% | 0.11% | -0.07% | 98% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | -0.03% | 98% | 93% |
| Class D | -0.05% | -0.44% | 0.01% | 98% | 91% |
| Class F | -0.02% | -0.12% | 0.09% | 98% | 95% |

Table 3. Results of Test 2 over VTM 6.0, AI configuration

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over | VTM-6.0 |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.02% | 100% | 101% |
| Class A2 | 0.00% | 0.03% | -0.02% | 101% | 103% |
| Class B | 0.00% | -0.02% | -0.04% | 101% | 100% |
| Class C | 0.01% | -0.12% | -0.02% | 100% | 100% |
| Class E | -0.01% | 0.05% | -0.01% | 98% | 91% |
| Overall | 0.00% | -0.02% | -0.02% | 100% | 99% |
| Class D | -0.01% | -0.08% | -0.01% | 98% | 90% |
| Class F | -0.01% | 0.10% | -0.04% | 96% | 91% |

Table 4. Results of Test 2 over VTM 6.0, RA configuration

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over | VTM-6.0 |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 97% | 94% |
| Class A2 | 0.03% | 0.14% | -0.01% | 99% | 101% |
| Class B | -0.01% | -0.05% | -0.03% | 97% | 93% |
| Class C | -0.02% | 0.03% | -0.01% | 98% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.01% | 98% | 94% |
| Class D | -0.07% | -0.51% | 0.11% | 98% | 90% |
| Class F | -0.01% | -0.14% | 0.07% | 98% | 96% |

# Conclusion

In this document, a simplification of the improved MPM-list construction for the intra mode coding, adopted at the last meeting, is proposed. The proposed method keeps  all the design features of the adopted MPM-list construction and only simplifies the candidate selection in the case when the prediction modes of the neighboring blocks are both directional and unequal. The proposed simplification leads to simpler software/hardware implementation and requires less Spec text for the MPM list generation process. Experimental results show the proposed simplification yields the same coding gain in both AI and RA configurations as VTM 6.0. It is proposed to adopt the method into the future VVC.

# References

- B. Bross et al., “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, Gothenburg, SE, July 2019.

- B. Wang et al., “CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, Geneva, CH, March 2019.

- J. Pfaff et al., “CE3-related: Improved intra mode coding based on unified MPM list generation”, JVET-O0484-r1, Gothenburg, SE, July 2019.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).