| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0359 |
| --- | --- |

| Title: | AHG17: An STSA_NUT restriction on PPS and APS availability |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution claims that the current VVC draft allows STSA pictures and pictures that follow an STSA picture in the decoding order to refer to a PPS or an APS of the same temporal layer that preceded the access unit containing the STSA picture in decoding order. It is proposed to add the following restriction text in the semantics of the nal_unit_type syntax element to prohibit usage of PPS and APS information that precedes the STSA access unit in decoding order in the same layer as the STSA access unit:

–	When the current access unit has a nal_unit_type equal to STSA_NUT, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit

–	For any following access unit that has TemporalId equal to that of the current access unit and follows the current access unit in decoding order, the following access unit shall not use a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order.

It is claimed in the contribution that this proposed restriction text ensures that necessary PPS and APS data is made available at temporal sub-layer up-switching points indicated by STSA pictures and that this is not ensured in the current VVC draft specification.

It was noted in the contribution that an alternative solution to the one proposed might be to make the sub-bitstream extraction process support removing some but not all NAL units of a certain temporal sub-layer, similar to what was proposed in JCTVC-H0568. However, the proponents believe that adding restriction text is a more straightforward solution to the claimed problem.

# Introduction and problem statement

The STSA_NUT in VVC indicates pictures to which temporal sub-layer up-switching is possible. For example, assuming a decoder is decoding temporal sub-layer n and lower. If there is an STSA picture belonging to temporal sub-layer (n+1) in the bitstream, the decoder can start decoding sub-layers (n+1) and lower.

The current VVC draft does not contain an activation process for parameter sets. Instead, there is the following text for the PPS and APS (shown in italic font):

#### 7.4.3.4 Picture parameter set RBSP semantics

A PPS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means, and the PPS NAL unit containing the PPS RBSP shall have nuh_layer_id equal to the nuh_layer_id of the coded slice NAL unit that refers it.

All PPS NAL units with a particular value of pps_pic_parameter_set_id within an access unit shall have the same content.

#### 7.4.3.5 Adaptation parameter set RBSP semantics

Each APS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

Let aspLayerId be the nuh_layer_id of an APS NAL unit. If the layer with nuh_layer_id equal to aspLayerId is an independent layer (i.e., vps_independent_layer_flag[ GeneralLayerIdx[ aspLayerId ] ] is equal to 1), the APS NAL unit containing the APS RBSP shall have nuh_layer_id equal to the nuh_layer_id of a coded slice NAL unit that referrs it. Otherwise, the APS NAL unit containing the APS RBSP shall have nuh_layer_id either equal to the nuh_layer_id of a coded slice NAL unit that referrs it, or equal to the nuh_layer_id of a direct dependent layer of the layer containing a coded slice NAL unit that referrs it.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within an access unit shall have the same content.

Assume an encoder generating a bitstream consisting of the following sequence of NAL units:

| AU number | NAL unit number | NAL unit type | TemporalId | PPS id | APS id | PPS id referred | APS id referred |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | DPS | 0 | - | - |  |  |
| 0 | 1 | VPS | 0 | - | - |  |  |
| 0 | 2 | SPS | 0 | - | - |  |  |
| 0 | 3 | PPS | 0 | 0 | - |  |  |
| 0 | 4 | APS | 0 | - | 0 |  |  |
| 0 | 5 | IDR_N_LP | 0 | - | - | 0 | 0 |
| 1 | 6 | PPS | 1 | 1 | - | - | - |
| 1 | 7 | APS | 1 | - | 1 | - | - |
| 1 | 8 | STSA | 1 | - | - | 1 | 1 |
| 2 | 9 | TRAIL | 0 | - | - | 0 | 0 |
| 3 | 10 | STSA | 1 | - | - | 1 | 1 |

If a decoder starts decoding temporal sub-layer 0 only and switches up at the STSA picture having NAL unit number 10, the PPS and APS NAL units with NAL unit numbers 6 and 7 may not be decoded.

However, we were unable to find that the current VVC draft prohibits an encoder to generate this sequence of NAL units.

The text above states that the parameter set “shall be available to the decoding process prior to it being referred“. This is fulfilled in the example above since all parameter sets are indeed available prior to them being referred to given that all temporal sub-layers are decoded.

It can be noted that the sub-bitstream extraction process in section 10 of the VVC draft requires that sub-bitstreams shall be conforming, but that sub-bitstreams are derived by removing all NAL units greater than tIdTarget. This means that it is required that the sub-bitstream consisting of only temporal layer 0 NAL units shall be conforming, but there is no requirement that any bitstream created by partial sub-layer extraction guided by STSA NAL unit types shall be conforming. Such a sub-bitstream extraction process and corresponding bitstream conformance text was proposed for HEVC in JCTVC-H0568_r1.doc section 3 but was not adopted into HEVC. We considered proposing a similar scheme for VVC but found adding restrictions to be preferred.

# Proposal

It is proposed to add the following restriction text in the semantics of the nal_unit_type syntax element:

–	When the current access unit has a nal_unit_type equal to STSA_NUT, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit

–	For any following access unit that has TemporalId equal to that of the current access unit and follows the current access unit in decoding order, the following access unit shall not use a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).