| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0359-v3 |
| --- | --- |

| Title: | AHG17: An STSA_NUT restriction on PPS and APS availability |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution claims that the current VVC draft allows STSA pictures and pictures that follow an STSA picture in the decoding order to refer to a PPS or an APS of the same temporal layer that preceded the access unit containing the STSA picture in decoding order. It is proposed to add the following restriction text in the semantics of the nal_unit_type syntax element to prohibit usage of PPS and APS information that precedes the STSA access unit in decoding order in the same layer as the STSA access unit:

–	When the current access unit has a nal_unit_type equal to STSA_NUT, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit

–	For any following access unit that has TemporalId equal to that of the current access unit and follows the current access unit in decoding order, the following access unit shall not use a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order.

It is claimed in the contribution that this proposed restriction text ensures that necessary PPS and APS data is made available at temporal sub-layer up-switching points indicated by STSA pictures and that this is not ensured in the current VVC draft specification.

It was noted in the contribution that an alternative solution to the one proposed might be to make the sub-bitstream extraction process support removing some but not all NAL units of a certain temporal sub-layer, similar to what was proposed in JCTVC-H0568. However, the proponents believe that adding restriction text is a more straightforward solution to the claimed problem.

Version 2 of this document provides three alternative solutions on top of the original proposed solution after it was pointed out in the HLS BoG that the proposed solution has issues with out of band transmission of PPSs and APSs. Alternative proposal 1 contains text for restrictions intended to be applied to APS and PPS sent in-band only. Alternative proposal 2 contains text for a solution with a self_contained_cvs_flag in SPS. Alternative proposal 3 contains text for an SEI message solution.

Version 3. After the contribution was revisited in the HLS BoG, it was agreed by the BoG to recommend the SEI message option of alternative proposal 3 for adoption, with the addition that the semantics and restrictions should all be included in the semantics of the SEI message. Version 3 of this document provides the text for the agreed alternative proposal 3 with the agreed modification.

# Introduction and problem statement

The STSA_NUT in VVC indicates pictures to which temporal sub-layer up-switching is possible. For example, assuming a decoder is decoding temporal sub-layer n and lower. If there is an STSA picture belonging to temporal sub-layer (n+1) in the bitstream, the decoder can start decoding sub-layers (n+1) and lower.

The current VVC draft does not contain an activation process for parameter sets. Instead, there is the following text for the PPS and APS (shown in italic font):

#### 7.4.3.4 Picture parameter set RBSP semantics

A PPS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means, and the PPS NAL unit containing the PPS RBSP shall have nuh_layer_id equal to the nuh_layer_id of the coded slice NAL unit that refers it.

All PPS NAL units with a particular value of pps_pic_parameter_set_id within an access unit shall have the same content.

#### 7.4.3.5 Adaptation parameter set RBSP semantics

Each APS RBSP shall be available to the decoding process prior to it being referred, included in at least one access unit with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

Let aspLayerId be the nuh_layer_id of an APS NAL unit. If the layer with nuh_layer_id equal to aspLayerId is an independent layer (i.e., vps_independent_layer_flag[ GeneralLayerIdx[ aspLayerId ] ] is equal to 1), the APS NAL unit containing the APS RBSP shall have nuh_layer_id equal to the nuh_layer_id of a coded slice NAL unit that referrs it. Otherwise, the APS NAL unit containing the APS RBSP shall have nuh_layer_id either equal to the nuh_layer_id of a coded slice NAL unit that referrs it, or equal to the nuh_layer_id of a direct dependent layer of the layer containing a coded slice NAL unit that referrs it.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within an access unit shall have the same content.

Assume an encoder generating a bitstream consisting of the following sequence of NAL units:

| AU number | NAL unit number | NAL unit type | TemporalId | PPS id | APS id | PPS id referred | APS id referred |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | DPS | 0 | - | - |  |  |
| 0 | 1 | VPS | 0 | - | - |  |  |
| 0 | 2 | SPS | 0 | - | - |  |  |
| 0 | 3 | PPS | 0 | 0 | - |  |  |
| 0 | 4 | APS | 0 | - | 0 |  |  |
| 0 | 5 | IDR_N_LP | 0 | - | - | 0 | 0 |
| 1 | 6 | PPS | 1 | 1 | - | - | - |
| 1 | 7 | APS | 1 | - | 1 | - | - |
| 1 | 8 | STSA | 1 | - | - | 1 | 1 |
| 2 | 9 | TRAIL | 0 | - | - | 0 | 0 |
| 3 | 10 | STSA | 1 | - | - | 1 | 1 |

If a decoder starts decoding temporal sub-layer 0 only and switches up at the STSA picture having NAL unit number 10, the PPS and APS NAL units with NAL unit numbers 6 and 7 may not be decoded.

However, we were unable to find that the current VVC draft prohibits an encoder to generate this sequence of NAL units.

The text above states that the parameter set “shall be available to the decoding process prior to it being referred“. This is fulfilled in the example above since all parameter sets are indeed available prior to them being referred to given that all temporal sub-layers are decoded.

It can be noted that the sub-bitstream extraction process in section 10 of the VVC draft requires that sub-bitstreams shall be conforming, but that sub-bitstreams are derived by removing all NAL units greater than tIdTarget. This means that it is required that the sub-bitstream consisting of only temporal layer 0 NAL units shall be conforming, but there is no requirement that any bitstream created by partial sub-layer extraction guided by STSA NAL unit types shall be conforming. Such a sub-bitstream extraction process and corresponding bitstream conformance text was proposed for HEVC in JCTVC-H0568_r1.doc section 3 but was not adopted into HEVC. We considered proposing a similar scheme for VVC but found adding restrictions to be preferred.

# Original proposal

It is proposed to add the following restriction text in the semantics of the nal_unit_type syntax element:

–	When the current access unit has a nal_unit_type equal to STSA_NUT, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit

–	For any following access unit that has TemporalId equal to that of the current access unit and follows the current access unit in decoding order, the following access unit shall not use a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order.

# Alternative proposal #1

During BoG discussions at the meeting, it was pointed out that the original proposed solution has issues with out of band transmission of PPSs and APSs. This alternative solution modifies the original proposal such that the restriction is only applied to the PPS and APS that are send in-band. The following text, highlighted in yellow, is proposed:

–	When the current access unit has a nal_unit_type equal to STSA_NUT, the following applies:

–	The current access unit shall not refer to a PPS or an APS that was not provided by external means and precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit

–	For any following access unit that has TemporalId equal to that of the current access unit and follows the current access unit in decoding order, the following access unit shall not refer to a PPS or an APS that was not provided by external means and has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order.

# Alternative proposal #2

During BoG discussions at the meeting, it was suggested to have a self_contained_cvs_flag as in HEVC to condition the restrictions. The following text, highlighted in yellow, is proposed for this alternative:

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| self_contained_cvs_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| } |  |

7.4.3.3	Sequence parameter set RBSP semantics

self_contained_cvs_flag equal to 1 indicates that each parameter set that is (directly or indirectly) referenced by any VCL NAL unit of the CVS that is not a VCL NAL unit of a RASL picture is present within the CVS at a position that precedes, in decoding order, any NAL unit that (directly or indirectly) references the parameter set. self_contained_cvs_flag equal to 0 indicates that this property may or may not apply.

7.4.2.2	NAL unit header semantics

nal_unit_type specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

…

When the current access unit has a nal_unit_type equal to STSA_NUT and self_contained_cvs_flag is equal to 1, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit.

- For any following access unit that has TemporalId equal to that of the current access unit and follows the 	current access unit in decoding order, the following access unit shall not refer to a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order

# Alternative proposal #3

During the BoG discussions at the meeting, it was suggested that an SEI message was used to carry the restriction. The following text, highlighted in yellow, is proposed for this alternative:

D.2 SEI payload syntax

D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) // Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7. |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 169 ) |  |
| referred_parameter_sets( payloadSize ) |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |

D.2.6 Referred parameter sets SEI message syntax

| referred_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| self_contained_cvs_flag | u(1) |
| } |  |

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The access unit containing the SEI message |
| Decoding unit information | The access unit containing the SEI message |
| Frame-field information | The access unit containing the SEI message |
| Referred parameter sets | The CVS containing the SEI message |

D.3.6 Referred parameter sets SEI message semantics

self_contained_cvs_flag equal to 1 indicates that each parameter set that is (directly or indirectly) referenced by any VCL NAL unit of the CVS that is not a VCL NAL unit of a RASL picture is present within the CVS at a position that precedes, in decoding order, any NAL unit that (directly or indirectly) references the parameter set. self_contained_cvs_flag equal to 0 indicates that this property may or may not apply.

7.4.2.2	NAL unit header semantics

nal_unit_type specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

When the current access unit has a nal_unit_type equal to STSA_NUT and there is no Referred parameter sets SEI message with self_contained_cvs_flag equal to 0 preceding the current access unit in the CVS, the following applies:

–	The current access unit shall not refer to a PPS or an APS that precedes the first NAL unit of the current access unit in decoding order and has TemporalId equal to the TemporalId of the current access unit.

- For any following access unit that has TemporalId equal to that of the current access unit and follows the 	current access unit in decoding order, the following access unit shall not refer to a PPS or an APS that has TemporalId equal to that of the current access unit that precedes the first NAL unit of the current access unit in decoding order

# Alternative proposal #3 with agreed modifications

After the contribution was revisited in the HLS BoG, it was agreed by the BoG to recommend the SEI message option of alternative proposal 3 for adoption, with the addition that the semantics and restrictions should all be included in the semantics of the SEI message. The following text, highlighted in yellow, reflects this agreement:

D.2 SEI payload syntax

D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) // Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7. |  |
| else if( payloadType = = 168 ) |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 169 ) |  |
| referred_parameter_sets( payloadSize ) |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |

D.2.6 Referred parameter sets SEI message syntax

| referred_parameter_sets( payloadSize ) { | Descriptor |
| --- | --- |
| self_contained_cvs_flag | u(1) |
| } |  |

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The access unit containing the SEI message |
| Decoding unit information | The access unit containing the SEI message |
| Frame-field information | The access unit containing the SEI message |
| Referred parameter sets | The CVS containing the SEI message |

D.3.6 Referred parameter sets SEI message semantics

self_contained_cvs_flag equal to 1 indicates that each parameter set that is (directly or indirectly) referenced by any VCL NAL unit of the CVS that is not a VCL NAL unit of a RASL picture is present within the CVS at a position that precedes, in decoding order, any NAL unit that (directly or indirectly) references the parameter set. self_contained_cvs_flag equal to 0 indicates that this property may or may not apply.

When an access unit A has a nal_unit_type equal to STSA_NUT and there is no Referred parameter sets SEI message with self_contained_cvs_flag equal to 0 preceding access unit A in the CVS, the following applies:

–	Access unit A shall not refer to a PPS or an APS that precedes the first NAL unit of access unit A in decoding order and has TemporalId equal to the TemporalId of access unit A.

–	For any following access unit that has TemporalId equal to that of access unit A and follows access unit A in decoding order, the following access unit shall not refer to a PPS or an APS that has TemporalId equal to that of access unit A that precedes the first NAL unit of access unit A in decoding order

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).