| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0361-v1 |
| --- | --- |

| Title: | On chroma quantization groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Wei-Jung Chien Han Huang Yu Han Bappaditya Ray Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to modify the derivation of chroma QP by specifying a chroma quantization group and aligning the definition of the chroma quantization group with that of the luma quantization group. It is asserted that the current chroma QP derivation may not be consistent for the dual tree architecture, where the luma and chroma splits are independent. It is asserted that the proposed method of aligning the definition of chroma and luma quantization groups results in simpler and cleaner QP derivation process.

# Introduction

The recent draft of VVC specifies luma quantization groups as a rectangular region of samples that have the same predicted QP values. The size of luma QGs are specified using the cu_qp_delta_subdiv syntax element specified in the PPS. For a luma QG, up to one QP adjustment parameter may be signalled.

In the 15th JVET meeting, chroma QP offsets were also adopted; the signalling of the chroma QP offset parameter is controlled by the cu_chroma_qp_offset_subdiv syntax element. The resultant region, referred to herein as chroma quantization group, has up to one chroma QP offset parameter specified for each component.

The chroma QP for a coded block is derived from the QP of a collocated luma block and adding the QP offset associated with the chroma component; this process is specified in Section 8.7.1 of JVET-O2001. When the chroma quantization group is equal to or smaller than the luma quantization group (i.e., in terms of the region covered by the chroma and luma QGs), the predicted QP derived for the chroma blocks in the QG is the same. However, when the chroma QG is larger than luma QG, different chroma coded blocks within a chroma may have different predicted QPs – this depends on the QP of the collocated luma QP. In typical content, the luma texture varies more than the chroma texture, and a finer control of QP may be necessary for luma than for chroma. Therefore, we may expect that it is quite common for chroma QG to be larger than the luma QG.

The current design makes it hard to keep the QP of the coding blocks in a chroma QG the same; for dual tree cases where the luma and chroma splits are independent, the correspondence of collocated luma QP as a predictor for chroma QP is not logical, particularly when it results in different QPs within a chroma QG. Even in the single tree case, QP within a chroma QG varies across the coding blocks.

Below are some examples of how the QP of the chroma coding blocks vary in single tree and dual tree for two different chroma QG sizes. For the larger chroma QG case, it is problematic to keep the QP the same for chroma coding blocks.

Figure 1: Example of chroma QP derived for two cases for chroma quantization groups in the single tree configuration. The chroma QPs derived for coding blocks within a chroma QG may not be the same. In this figure, red font indicates blocks with zero coefficient

Figure 2: Example of chroma QP derived for two cases for chroma quantization groups in the dual tree configuration. The chroma QPs derived for coding blocks within a chroma QG may not be the same. In this figure, red font indicates blocks with zero coefficients.

## Quantization group: same QP predictor, same QP adjustment or same QP?

By design, all the coding blocks in a luma quantization group have the same predicted QP value. For this discussion, we will split the coding blocks in a QG as leading coding blocks with zero coefficients (leading coding blocks), and rest of the coding blocks (trailing coding blocks). The trailing coding blocks follow the leading coding blocks in decoding order. The leading coding blocks, if any, have zero coefficients. The first coding block among the trailing coding blocks, if any, has non-zero coefficients.

When trailing coding blocks are present, a delta QP value is signalled and the QP for all the trailing coding blocks are set equal to the predicted QP plus the delta QP. The QP of the leading coding blocks is equal to the predicted QP.

So does the luma quantization group apply the same QP, the same QP predictor or same QP adjustment to all the coding blocks in the QG?

- Same QP predictor: all the coding blocks in a QG have the same QP predictor.

- Same QP adjustment: the leading coding blocks have no QP adjustment (or same QP adjustment of 0) and the trailing coding blocks have the same QP adjustment equal to delta QP.

- Same QP (dependent on the first two criteria): the leading coding blocks in the QG have the same QP (equal to the QP predictor); the trailing coding blocks have the same QP (equal to the QP predictor + delta QP)

The current chroma quantization group only satisfies the “same QP adjustment” criteria as the luma QG. Since the predicted QP of each chroma coding block is linked to the QP of the collocated luma, this may result in different QPs among the leading coding blocks, and different QPs among the trailing coding blocks. It is much cleaner to have an aligned definition of luma and chroma quantization groups.

# Proposal

It is proposed to specify a chroma QG as having the same predicted QP value. The QP value for the chroma is taken as QP of the collocated luma samples of the top-left chroma sample in the QG. The proposal is applied for both the dual tree and the single tree in order to align the chroma QP derivation and align the definition of chroma and luma QGs.

The impact of the proposed change to the examples in Section 1 are illustrated in the figures below; the QPs of the leading coding blocks are the same and the QPs for the trailing coding blocks are the same for both single tree and dual tree architectures. Moreover, the predicted QPs for all the CBs in the chroma QG are the same, thus aligning the definition of luma and chroma QGs.

Figure 3: Chroma QP derived using the proposed method for the single-tree case; it is easier to control the chroma QP for the coding blocks within a chroma QG.

Figure 4: Chroma QP derived using the proposed method for the dual-tree case; it is easier to control the chroma QP for the coding blocks within a chroma QG.

# Changes to the specification

The proposed changes to the specification are highlighted below.

The following changes are made to the coding tree syntax.

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOnY && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= cu_chroma_qp_offset_subdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuChromaQgTopLeftX = x0 |  |
| CuChromaQgTopLeftY = y0 |  |
| } |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |
| … |  |

The following changes are made to the implicit dual tree syntax structure.

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= cu_chroma_qp_offset_subdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuChromaQgTopLeftX = x0 |  |
| CuChromaQgTopLeftY = y0 |  |
| } |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| … |  |

The following changes are made to the Section 8.7.1 Derivation process of the quantization parameters.

### 8.7.1	Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current luma quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables CuQgTopLeftX and CuQgTopLeftY.

The chroma location ( xChromaQg, yChromaQg ), specifies the top-left luma sample of the current chroma quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xChromaQg and yChromaQg are set equal to CuChromaQgTopLeftX and CuChromaQgTopLeftY, respectively.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a brick.

–	The current quantization group is the first quantization group in a CTB row of a brick and entropy_coding_sync_enabled_flag is equal to 1.

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma coding unit in the previous quantization group in decoding order.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	The CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to the CTB containing the current luma coding block at ( xCb, yCb ), i.e. all of the following conditions are true:

–	( xQg − 1 )  >>  CtbLog2SizeY is not equal to ( xCb )  >>  CtbLog2SizeY

–	( yQg )  >>  CtbLog2SizeY is not equal to ( yCb )  >>  CtbLog2SizeY

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	The CTB containing the luma coding block covering the luma location ( xQg, yQg − 1 ) is not equal to the CTB containing the current luma coding block at ( xCb, yCb ), i.e. all of the following conditions are true:

–	( xQg )  >>  CtbLog2SizeY is not equal to ( xCb )  >>  CtbLog2SizeY

–	( yQg − 1 )  >>  CtbLog2SizeY is not equal to ( yCb )  >>  CtbLog2SizeY

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

- If all the following conditions are true, then qPY_PRED is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ):

- availableB is equal to TRUE.

- the current quantization group is the first quantization group in a CTB row within a brick

- Otherwise, qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(8-932)

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal + 64 + 2 * QpBdOffsetY )%( 64 + QpBdOffsetY ) ) − QpBdOffsetY	(8-933)

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffsetY		(8-934)

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, tThe variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2xChromaQg, yChromaQg ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiChroma = Clip3( −QpBdOffsetC, 63, QpY )		(8-935)

qPiCb = ChromaQpTable[ 0 ][ qPiChroma ]		(8-936)

qPiCr = ChromaQpTable[ 1 ][ qPiChroma ]		(8-937)

qPiCbCr = ChromaQpTable[ 2 ][ qPiChroma ]		(8-938)

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb =  Clip3( −QpBdOffsetC, 63, qPCb + pps_cb_qp_offset + slice_cb_qp_offset +CuQpOffsetCb )
	 + QpBdOffsetC		(8-939)

Qp′Cr = Clip3( −QpBdOffsetC, 63, qPCr + pps_cr_qp_offset + slice_cr_qp_offset +CuQpOffsetCr )
	 + QpBdOffsetC		(8-940)

Qp′CbCr = Clip3( −QpBdOffsetC, 63, qPCbCr + pps_cbcr_qp_offset + slice_cbcr_qp_offset +CuQpOffsetCbCr )
	 + QpBdOffsetC		(8-941)

# References

[1] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4 and 4x2 chroma CBs, JVET-O0050, Gothenburg, June 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).