| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0363-v2 |
| --- | --- |

| Title: | AHG17: On NAL unit type table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the NAL unit type (NUT) table in the current version of the VVC specification:

- Switch position of RADL and RASL in the table to align with the order in HEVC.

- Add VPS_NUT, which is missing, and order the parameter set NUTs in hierarchical order as in HEVC, in the order of DPS_NUT, VPS_NUT, SPS_NUT, PPS_NUT, APS_NUT.

- Add FD_NUT, which is missing, between EOB_NUT and PREFIX_SEI_NUT as in HEVC.

- Group reserved non-IRAP VCL NUTs within one code range instead of two.

- Allocate 5 codes instead of 6 for reserved non-IRAP VCL NUTs and 3 codes instead of 2 for reserved IRAP VCL NUTs.

Proposed specification text relative to JVET-O2001-vE.docx is provided with the contribution.

In version 2 of this contribution, FD_NUT was added.

# Introduction

The NUT tables in the HEVC and VVC specifications provide information about the NUT codes and NUT classes. In the current version of the VVC specification (JVET-O2001-vE) the NUT table is not aligned with the NUT table in the HEVC (V6) specification. The proponents believe that aligning the NUT tables in VVC and HEVC (V6) as far as possible prevents confusion. Since HEVC uses 64 and VVC uses 32 NUT codes, it is not possible to use the same NUT codes but maintaining the same order for the NUTs would be useful. The following issues have been identified with the NUT table in VVC:

- RADL and RASL are in a different order compared to HEVC

- VPS_NUT is missing

- FD_NUT, which was adopted at the 14-th JVET meeting in Geneva, is missing

- The parameter set NUTs are not in hierarchical order, which they are in HEVC

- There are two ranges of reserved non-IRAP VCL NUTs which seems to be a remnant from previous versions of the NUT table in VVC

- There are 6 codes for reserved non-IRAP VCL NUTs and 2 codes for reserved IRAP VCL NUTs, which seems unbalanced

For convivence, the NUT tables in the current version of the VVC specification (JVET-O2001vE) and in the HEVC (V6) specification have been added to annexes A and B of this contribution.

# Proposal

The following changes to the NAL unit type (NUT) table are proposed to the VVC specification:

- Switch position of RADL and RASL in the table to align with the order in HEVC.

- Add VPS_NUT, which is missing, and order the parameter set NUTs in hierarchical order as in HEVC, in the order of DPS_NUT, VPS_NUT, SPS_NUT, PPS_NUT, APS_NUT.

- Add FD_NUT, which is missing, between EOB_NUT and PREFIX_SEI_NUT as in HEVC.

- Group reserved non-IRAP VCL NUTs within one code range instead of two

- Allocate 5 codes instead of 6 for reserved non-IRAP VCL NUTs and 3 codes instead of 2 for reserved IRAP VCL NUTs.

## Proposed changes to the VVC specification

The proposed changes on top of the VVC draft (JVET-O2001-vE) are marked in yellow with removed text in red strike-through.

- Definitions

…

  - associated VCL NAL unit: The preceding VCL NAL unit in decoding order for a non-VCL NAL unit with nal_unit_type equal to EOS_NUT, EOB_NUT, SUFFIX_SEI_NUT, RSV_NVCL_27, RSV_NVCL_23, or in the range of UNSPEC30..UNSPEC31; or otherwise the next VCL NAL unit in decoding order.

…

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 2 | RASDL_NUT | Coded slice of a RASDL picture slice_layer_rbsp( ) | VCL |
| 3 | RADSL_NUT | Coded slice of a RADSL picture slice_layer_rbsp( ) | VCL |
| 4..78 | RSV_VCL_4.. RSV_VCL_78 | Reserved non-IRAP VCL NAL unit types | VCL |
| 89 910 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 101 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) | VCL |
| 112 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 123..135 | RSV_IRAP_VCL123.. RSV_IRAP_VCL135 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | VPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 168 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 179 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1820 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 1921 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 202 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 213 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 24 | FD_NUT | Filler data filler_data_rbsp() | non-VCL |
| 225, 236 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 25..27 | RSV_NVCL25.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

…

When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_VCL135, inclusive, TemporalId shall be equal to 0.

…

7.4.2.4	Order of NAL units and association to coded pictures, layer access units, access units, and coded video sequences

…

The order of the coded pictures and non-VCL NAL units within a layer access unit or an access unit shall obey the following constraints:

–	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, APS NAL units, prefix SEI NAL units, NAL units with nal_unit_type in the range of RSV_NVCL_25.. RSV_NVCL_26, or NAL units with nal_unit_type in the range of UNSPEC28..UNSPEC29 are present in a layer access unit, they shall not follow the last VCL NAL unit of the layer access unit.

–	NAL units having nal_unit_type equal to SUFFIX_SEI_NUT or RSV_NVCL_27, or in the range of UNSPEC30..UNSPEC31 in a layer access unit shall not precede the first VCL NAL unit of the layer access unit.

# Annex A – Table 7.1 in VVC

Table 7-1 – NAL unit type codes and NAL unit type classes in VVC

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_4.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) | VCL |
| 11 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 12 13 | RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 25..27 | RSV_NVCL25.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

# Annex B – Table 7.1 in HEVC

Table ‎7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP SLNR VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).