| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0366 |
| --- | --- |

| Title: | AHG17: Signaling absence of NAL unit types in DPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to add flags to signal the absence of some of the NAL unit types in the bitstream in the general_constraint_info() struct in  DPS and VPS. In a first option the following constraint flags are proposed to be added to the general_constraint_info():

- no_trail_constraint_flag

- no_stsa_constraint_flag

- no_rasl_constraint_flag

- no_radl_constraint_flag

- no_idr_constraint_flag

- no_cra_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

In a second option, constraint flags are only proposed for the NAL unit and picture types the proponents assess are the most useful. Another difference compared to the first option is that the no_idr_constraint_flag and the no_cra_constraint_flag are grouped into a no_irap_constraint_flag. The following constraint flags are proposed to be added to the general_constraint_info() in the second option:

- no_stsa_constraint_flag

- no_irap_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

It is proposed to add one of option 1 or option 2 to the VVC specification, or a mixture of the two.

# Introduction

The current version of VVC provides signaling a set of constraints in the general_constraint_info() struct in DPS and optionally in VPS. The constraints inform the decoder what to expect from the bitstream, including whether certain coding tools are enabled in the bitstream or not, the maximum bitdepth and chroma format for the bitstream, etc. Given these restrictions, the decoder may then adapt the allocation and usage of resources.

However, these constraints do not include any information about which NAL unit types that may be expected in the bitstream. Below are some examples when it may be useful for the decoder to know that certain NAL unit types will not be present in the bitstream:

- If it is known that no STSA pictures will be present in the bitstream, the decoder does not need to scan higher temporal sublayers than what it is currently decoding, nor store any PPSs or APSs in the higher sublayers that may otherwise be needed when switching up at an STSA picture.

- If it is known that no CRA or IDR pictures will be present in the bitstream, the decoder may conclude that there may not be any bitrate spikes from IRAP pictures and may adapt its output timing accordingly.

- If it is known that no GDR pictures will be present in the bitstream, the decoder does not need to generate any unavailable pictures to decode the stream (It is well known that CRA random access can be done without generating unavailable pictures since any RASL picture is discarded).

- If it is known that there will be no APSs in the bitstream, the decoder does not need to allocate memory to store any potential APSs.

# Proposal

It is proposed to add flags to signal the absence of some of the NAL unit types in the bitstream in the general_constraint_info() struct in  DPS and VPS. In a first option the following constraint flags are proposed to be added to the general_constraint_info():

- no_trail_constraint_flag

- no_stsa_constraint_flag

- no_rasl_constraint_flag

- no_radl_constraint_flag

- no_idr_constraint_flag

- no_cra_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

In a second option, constraint flags are only proposed for the NAL unit and picture types the proponents assess are the most useful. Another difference compared to the first option is that the no_idr_constraint_flag and the no_cra_constraint_flag are grouped into a no_irap_constraint_flag. The following constraint flags are proposed to be added to the general_constraint_info() in the second option:

- no_stsa_constraint_flag

- no_irap_constraint_flag

- no_gdr_constraint_flag

- no_aps_constraint_flag

It is proposed to add one of option 1 or option 2 to the VVC specification, or a mixture of the two.

## Proposed changes to the VVC specification

The proposed changes on top of the current VVC draft (JVET-O2001-vE) for option 1 and option 2 are shown below.

### Option 1

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| … |  |
| } |  |

no_trail_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type TRAIL_NUT shall be present in the bitstream. no_trail_constraint_flag equal to 0 does not impose a constraint.

no_stsa_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type STSA_NUT shall be present in the bitstream. no_stsa_constraint_flag equal to 0 does not impose a constraint.

no_rasl_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type RASL_NUT shall be present in the bitstream. no_rasl_constraint_flag equal to 0 does not impose a constraint.

no_radl_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type RADL_NUT shall be present in the bitstream. no_radl_constraint_flag equal to 0 does not impose a constraint.

no_idr_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type IDR_W_RADL or IDR_N_LP shall be present in the bitstream. no_idr _constraint_flag equal to 0 does not impose a constraint.

no_cra_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type CRA_NUT shall be present in the bitstream. no_cra_constraint_flag equal to 0 does not impose a constraint.

no_gdr_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type GDR_NUT shall be present in the bitstream. no_gdr_constraint_flag equal to 0 does not impose a constraint.

no_aps_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type APS_NUT shall be present in the bitstream. no_aps_constraint_flag equal to 0 does not impose a constraint.

### Option 2

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_stsa_constraint_flag | u(1) |
| no_irap_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| … |  |
| } |  |

no_stsa_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type STSA_NUT shall be present in the bitstream. no_stsa_constraint_flag equal to 0 does not impose a constraint.

no_irap_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type IDR_W_RADL, IDR_N_LP or CRA_NUT shall be present in the bitstream. no_irap _constraint_flag equal to 0 does not impose a constraint.

no_gdr_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type GDR_NUT shall be present in the bitstream. no_gdr_constraint_flag equal to 0 does not impose a constraint.

no_aps_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that no NAL unit of NAL unit type APS_NUT shall be present in the bitstream. no_aps_constraint_flag equal to 0 does not impose a constraint.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).