| : 1. Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0370 |
| --- | --- |

| Title: | Non-CE7: Signaling of coded_sub_block_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In VVC 6, coded_sub_block_flag is always context coded even after the number of context coded bins reached to the maximum limit. Since coded_sub_block_flag is always context coded, it is possible that even after reaching the limit of number of the context coded bins in a TU, CABAC module still has to switch between by-pass and context coding within the TU. This contribution proposes two methods to avoid context/by-pass switching after reaching the maximum number of context coded bins. Method 1 applies bypass coding to this flag when maximum number of context coded bins is reached in a TU, and method 2 re-positions the coded_sub_block_flag of the CGs towards the beginning of the TU. Following results are reported.

- Test 1.1: bypass coding of coded_sub_block_flag after reaching context coded bin limit. The maximum number of context coded bins are same as VVC 6.

  - 0.00% (AI), 0.00% (RA), 0.02% (LB) luma BD-rate

- Test 1.2: bypass coding of coded_sub_block_flag after reaching context coded bin limit. The maximum number of context coded bins are adjusted (VVC6 context coded bins + number of sub-blocks)

  - 0.00% (AI), -0.01% (RA), -0.02% (LB) luma BD-rate

- Test 2: Reposition the coded_sub_block_flag. The maximum number of context coded bins are same as VVC 6.

  - 0.00% (AI), 0.00% (RA), 0.00% (LB) luma BD-rate

# Problem statement

In VVC 6, both transform and transform skip residual coefficients of a transform block are coded using non-overlapped coefficient groups (CGs). The selection of CG sizes depends on the transform block (TB) size. For each CG, coded_sub_block_flag is signaled at the beginning of the CG. The coded_sub_block_flag = 0 means all of the coefficients of that CG are zero, therefore, coding of the transform coefficients of that CG can be skipped.

In VVC 6, coded_sub_block_flag is always context coded even after the number of context coded bins reached the maximum limit for a TB. Figure 1 shows the example of the bit-stream of a residual block. The blue portion of the bit-stream represents context coded bins and white portion represents the by-pass coded bins.

Figure 1: An example of a bit-stream of a residual block of VVC 6

Since coded_sub_block_flag is always context coded, as shown in Figure 1, it is observed that even after reaching the context coded bins to maximum limit, CABAC module still has to switch between by-pass and context coding which ultimately impacts the throughput of the CABAC decoder.

# Proposed method

## Method 1: By-pass coding of coded_sub_block_flag after reaching the maximum context bin limit

In the proposed method, before coding of each coded_sub_block_flag, the CABAC module checks if the context coded bins are within maximum limit or not. If the number of context coded bins are within the maximum limit, coded_sub_block_flag is context coded, otherwise, the coded_sub_block_flag is by-pass coded. In this way, after reaching the maximum number of context coded bins, all of the remaining coded_sub_block_flags are by-pass coded. Figure 2 shows an example of the bit-stream when coded_sub_block_flags are by-pass coded after reaching to the maximum limit.

Figure 2: An example of the bit-stream when coded_sub_block_flag is bypass coded after reaching to the maximum limit

## Method 2: reposition coded_sub_block_flag

In method 2, we propose to reposition the coded_sub_block_flags of all CGs at the beginning of the residual block in case of transform skip mode. In case of transform block, we proposed to code all of the coded_sub_block_flags right after coding of the last significant position. The proposed method guarantees that after the number of context coded bins reaches the maximum limit, all of the remaining bins of that residual block are by-pass coded. Figure 3 shows an example of the bit-stream of the transform skip residual block of the proposed method. The maximum number of context coded bins of this method is same as VVC 6.

Figure 3: An example of the bit-stream of method 2 where all of the coded_sub_block flags are reposition at the beginning

# Simulation results

The proposed methods are tested under CTC test settings. Following three tests are reported.

- Test 1.1 (Method 1): The maximum number of context coded bins are same as VVC 6, as follows

  - if (transform_skip_flag)

MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight )

  - else

MaxCcbs  = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2

- Test 1.2 (Method 1): The maximum number of context coded bins are adjusted

  - if (transform_skip_flag)

MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight )+          number_of_subblock

  - else

MaxCcbs  = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 +  number_of_subblock

- Test 2 (Method 2): Reposition the coded_sub_block_flag. The maximum number of context coded bins are same as VVC 6.

Note that Test 1.2 does not change the  worst case scenario in terms of the number of context coded bins, since VVC draft 6 does not count the coded_sub_block_flag toward the bin budget. In comparison, the test 1.1 and test 1.2 always count coded_sub_block_flag toward the context bin budget. Table 1, Table 2 and Table 3 shows the results of proposed methods under CTC test conditions.

Table 1: Results of test 1.1 ( CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.05% | 99% | 100% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class C | 0.02% | -0.12% | 0.01% | 99% | 103% |
| Class E | -0.01% | 0.00% | -0.05% | 100% | 103% |
| Overall | 0.00% | -0.03% | 0.00% | 99% | 102% |
| Class D | 0.01% | -0.12% | -0.01% | 98% | 100% |
| Class F | 0.04% | 0.01% | -0.02% | 101% | 102% |
| Class SCC | 0.05% | 0.05% | 0.02% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.04% | 100% | 99% |
| Class A2 | 0.02% | 0.17% | 0.08% | 100% | 99% |
| Class B | -0.01% | 0.04% | 0.02% | 100% | 99% |
| Class C | 0.00% | 0.00% | -0.08% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.01% | 100% | 99% |
| Class D | -0.01% | -0.29% | -0.16% | 100% | 93% |
| Class F | 0.02% | 0.07% | 0.01% | 100% | 103% |
| Class SCC | 0.04% | 0.01% | 0.04% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.16% | 0.21% | 100% | 100% |
| Class C | -0.02% | 0.08% | -0.15% | 100% | 101% |
| Class E | 0.11% | -0.65% | -0.96% | 101% | 101% |
| Overall | 0.02% | -0.07% | -0.20% | 100% | 101% |
| Class D | 0.05% | 0.03% | -0.03% | 100% | 106% |
| Class F | 0.07% | -0.01% | 0.42% | 101% | 101% |
| Class SCC | -0.03% | -0.03% | -0.04% | 101% | 105% |

Table 2: Results of test 1.2 (CTC QP) (Run time is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 100% | 108% |
| Class A2 | 0.00% | 0.01% | 0.02% | 101% | 107% |
| Class B | 0.00% | 0.01% | 0.02% | 101% | 107% |
| Class C | 0.02% | -0.16% | 0.02% | 100% | 108% |
| Class E | 0.00% | 0.01% | -0.06% | 101% | 110% |
| Overall | 0.00% | -0.03% | 0.00% | 100% | 108% |
| Class D | -0.01% | -0.06% | 0.00% | 98% | 109% |
| Class F | -0.01% | 0.02% | -0.01% | 101% | 103% |
| Class SCC | -0.03% | -0.01% | -0.02% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.02% | 101% | 112% |
| Class A2 | 0.04% | -0.03% | 0.00% | 101% | 111% |
| Class B | -0.04% | 0.02% | -0.04% | 101% | 113% |
| Class C | -0.01% | 0.02% | -0.07% | 101% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.02% | -0.03% | 101% | 111% |
| Class D | -0.02% | 0.00% | 0.04% | 101% | 108% |
| Class F | 0.01% | 0.05% | 0.03% | 101% | 103% |
| Class SCC | 0.03% | -0.04% | -0.04% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.06% | -0.18% | 101% | 112% |
| Class C | 0.00% | -0.37% | -0.41% | 102% | 112% |
| Class E | -0.07% | 0.21% | 0.23% | 102% | 121% |
| Overall | -0.02% | -0.10% | -0.15% | 101% | 114% |
| Class D | 0.04% | -0.21% | 0.01% | 101% | 115% |
| Class F | -0.05% | -0.02% | 0.00% | 101% | 103% |
| Class SCC | -0.10% | -0.03% | 0.02% | 103% | 106% |

Table 3: results of test 2 ( CTC QP) ( runtime is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 110% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 109% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class SCC | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 111% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 110% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 112% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 111% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 108% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class SCC | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 109% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 110% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 119% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 112% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 114% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class SCC | 0.00% | 0.00% | 0.00% | 102% | 104% |

# Conclusion

In order to improve the CABAC throughput, this contribution proposed two methods of coded_sub_block_flag signaling to guarantee by-pass coding of a residual block after reaching the maximum limit of the context coded bins. Method 1 proposed by-pass coding of the coded_sub_block_flag after reaching the limit, whereas, method 2 proposed to reposition the coded_sub_block_flag at the beginning of the residual block. The proposed methods can improve the CABAC throughput by reducing the context/by-pass switching without noticeable coding loss.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).