| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0372-v1 |
| --- | --- |

| Title: | CE5-related: Joint chroma cross-component adaptive loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He, Hongyu Li 4 Research Way 3rd Floor, Princeton, NJ 08540, USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com Hongyu.Li@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Cross-Component Adaptive Loop Filter (CCALF) applies a linear diamond shaped filter on luma samples and uses the output signal to refine each chroma component. In this proposal, to reduce the complexity of CCALF, we propose joint chroma CCALF (JC-CCALF), where only one set of CCALF filter coefficients will be trained and used to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component. It reduces the complexity of existing CCALF roughly by half. Our JC-CCALF is implemented based on CCALF in CE5-2.1 (i.e. In-loop CCALF with chroma gain shifted to luma). Simulation result shows that: compared to VTM-6.0, the proposed method yields -0.68%/+0.84%/+3.18% Y/Cb/Cr BD-rate change for AI test of 100 frames, while the original CCALF yields -0.99%/+1.54%/+2.83%.

# Introduction

Cross-Component Adaptive Loop Filter (CCALF) is a newly proposed coding tool [1] and tested in CE5-2 [2]. It makes use of luma sample values to refine each chroma component. For each chroma component sample, a CCALF filter is applied to a diamond region centered at the collocated position in luma component, the filter output as a refinement is added to chroma sample value.

The filter coefficients are transmitted in APS. The application of the filters is controlled on a variable block size and signalled by a context-coded flag received for each block of samples. The block size along with an CCALF enabling flag is received at the slice-level for each chroma component. The supported block sizes (in chroma samples) are 16x16, 32x32, 64x64, 128x128. At the encoder side, the same filter coefficient training functions used in existing ALF of VTM-6.0 are used for CCALF as well.

Currently, CCALF yields ~1% BD-rate RA luma coding gain, while incurring 50% computation complexity increase over the existing ALF (for the case that CCALF and ALF are applied to all samples). Note that ALF yields ~5% RA coding gain. The complexity of the current CCALF is a little high considering its relative gain over ALF.

# Joint chroma CCALF

The inherent correlation between Cb and Cr components has been successfully exploited in the joint chroma residual coding tool of VVC. Herein, to reduce complexity of CCALF, we propose joint chroma CCALF (JC-CCALF), where only one CCALF filter will be used to generate one CCALF filtered output as the chroma refinement signal for one color component only, while a properly weighted version of the same chroma refinement signal will be applied to the other color component. In this way, the complexity of existing CCALF is reduced roughly by half. The proposed method is illustrated in Figure 1.

SAO Luma

ALF Luma

SAO Cb

SAO Cr

Joint chroma

CC ALF

ALF Chroma

Y

Cb

Cr

w

Figure 1. The proposed joint chroma CCALF method workflow

Our current implementation is based on CCALF CE5-2.1 in-loop filtering with chroma gain shifted to luma [2]. The weight values are {+/−2, +/−1, +/−1/2}, which are selected at the encoder side when training for the ALF filter coefficients. In syntax, it is coded into two variables, i.e. the sign of the weight and the weight index.

The block-level on/off control of ALF filtering for Cb and Cr is the same, and thus, only one set of block-level on/off control map will be coded. Currently, the Cb, Cr on/off control block sizes are the same, and thus, only one block size variable is coded. When making the on/off decision for a block at the encoder side, the total RD cost of both Cb and Cr is considered.

At the encoder side, to train for the optimal set of filter coefficients, originally for the existing CCALF, the ideal target refinement signal is just the residual of each chroma components (denoted as resCb, resCr, respectively). Take the Cb component for example, the optimization problem for joint chroma CCALF is as follows.

Find tgt_ccalf, s.t.

minorgCb-tgt_ccalf+alfCb2+orgCr-tgt_ccalf*w+alfCr2. 	  (1)

Herein, tgt_ccalf, orgCb/Cr, alfCb/Cr represent the ideal target output refinement signal, the original input video signal, and the ALF output signal, respectively. w represents the weighting factor. Denote the chroma residual signals as resCb/Cr as follows.

resCb=orgCb-alfCb, resCr=orgCr-alfCr  			  (2)

To solve Eq (1), take the derivative of it and make it zero. We have:

-2orgCb-tgt_ccalf+alfCb-2w*orgCr-tgt_ccalf*w+alfCr=0	  (3)

Solving Eq (3) and plugging in Eq (2), we have:

tgt_ccalf=(resCb+w*resCr)/(1+w2) 				  (4)

The similar iterative training algorithm of CCALF is used in the proposed method as well.

# Simulation results

CTC tests are performed based on VTM-6.0 as the anchor [3]. So far, we only has result of AI test of 100 frames available. The full CTC test result will be provided later on.

## JC-CCALF based on CE5-2.1 (in loop filter with chroma gain shifted to luma)

Performance of the proposed method is as follows.

Table 1. Performance of the proposed JC-CCALF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | 8.27% | 4.85% | 118% | 101% |
| Class A2 | -2.07% | 4.24% | 1.83% | 105% | 100% |
| Class B | -0.80% | 1.50% | 6.33% | 104% | 101% |
| Class C | -0.79% | 4.72% | 6.10% | 101% | 93% |
| Class E | -0.58% | -2.55% | 6.49% | 104% | 94% |
| Overall | -1.04% | 3.13% | 5.31% | 106% | 98% |
| Class D | -0.75% | 5.01% | 7.80% | 100% | 92% |
| Class F | -0.81% | -1.50% | 0.22% | 101% | 96% |
| Class TGM | -1.31% | -4.85% | -4.08% | 101% | 100% |

For comparison reference, the performance of the original CCALF as in CE5-2.1 in-loop filtering with chroma gain shift is as follows.

Table 2. Performance of existing CCALF in CE5-2.1 with in-loop filtering with chroma gain shifted to luma

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | 7.52% | 3.84% | 104% | 102% |
| Class A2 | -2.02% | -0.28% | -0.33% | 99% | 103% |
| Class B | -0.76% | -0.20% | 1.59% | 100% | 101% |
| Class C | -0.74% | 3.75% | 4.57% | 99% | 94% |
| Class E | -0.49% | -2.68% | 4.75% | 100% | 96% |
| Overall | -0.99% | 1.54% | 2.83% | 100% | 99% |
| Class D | -0.72% | 5.09% | 5.81% | 99% | 92% |
| Class F | -0.72% | -2.38% | -1.70% | 100% | 96% |
| Class TGM | -1.18% | -4.98% | -4.60% | 100% | 102% |

## JC-CCALF with luma gain shifted to chroma

To compare easily with CCALF, we shift luma gain to chroma via reducing the chroma QP offset by 1 when calculating the Lagrangian multiplier λ for the chroma components. Performance of the modified method is as follows.

Table 3. Performance of the proposed JC-CCALF with luma gain shifted to chroma

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | 4.92% | 2.55% | 119% | 102% |
| Class A2 | -1.35% | 1.61% | -0.78% | 106% | 102% |
| Class B | -0.54% | -0.84% | 4.17% | 106% | 102% |
| Class C | -0.55% | 3.09% | 4.14% | 103% | 101% |
| Class E | -0.37% | -4.25% | 4.82% | 107% | 101% |
| Overall | -0.68% | 0.84% | 3.18% | 107% | 102% |
| Class D | -0.53% | 3.28% | 5.97% | 102% | 100% |
| Class F | -0.46% | -2.80% | -1.33% | 102% | 101% |
| Class TGM | -0.67% | -5.16% | -4.58% | 102% | 102% |

# Conclusion

To reduce complexity of CCALF, we propose joint chroma CCALF which conducts ALF filtering only once and applies the output refinement signal for both chroma components, i.e. directly applied to Cb, and weighted and applied to Cr, respectively. In this way, the computation complexity of CCALF is roughly reduced by half. On VTM-6.0, the proposed method yields -0.68%/+0.84%/+3.18% Y/U/V BD-rate change for AI test of 100 frames, while the original CCALF yields -0.99%/+1.54%/+2.83%.

# References

- K. Misra, F. Bossen, A. Segall, “Cross-Component Adaptive Loop Filter for chroma”, JVET-O0636, 15th meeting, July 2019, Gothenburg, Sweden.

- C.-Y. Chen, A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering”, JVET-O2025, 15th meeting, July 2019, Gothenburg, SE.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,"JVET common test conditions and software reference configurations for SDR video", JVET-O2010, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).