| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0373-v3 |
| --- | --- |

| Title: | CE7-related: Decouple level mapping from transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

The newly adopted level mapping process in transform skip (TS) residual coding requires inverse mapping to be performed during parsing, because the Rice parameter derivation depends on the inverse mapped level value. It is asserted that this could affect the CABAC throughput. In order to improve CABAC throughput, this contribution proposes to derive the Rice parameter from the mapped value so that the inverse mapping process can be untied from the parsing process.  The proposed method also unifies the processing flow of the transform-skip mode and BDPCM mode. It is reported that the BD-rate impact of the proposed method is 0.00% (AI), 0.05% (RA), 0.01% (LB)  for the SCC sequences under CTC test conditions, which is negligible.

# Problem statement

In VVC6, a level mapping mechanism is applied to TS residual coding. Level mapping uses the top and left neighboring coefficient levels to predict the current coefficient level in order to reduce signaling cost. For a given residual position, denote absCoeff as the absolute coefficient level before mapping and absCoeffMod as the coefficient level after mapping. Let X0 denote the absolute coefficient level of the left neighbouring position and let X1 denote the absolute coefficient level of the above neighbouring position. The level mapping is performed as follows:

pred = max(X0, X1);

if (pred == 0 | | absCoeff == 0 | | bdpcm )

absCoeffMod = absCoeff

else

{

if (absCoeff == pred)

absCoeffMod = 1;

else

absCoeffMod = (absCoeff < pred) ? absCoeff + 1 : absCoeff;

}

At the decoder, since the Rice parameter derivation process depends on the actual level, computation of actual level (which requires inverse mapping) need to be implemented within the CABAC parsing loop. Such interleaved manner of parsing and level decoding may be undesirable as it reduces the throughput for decoder hardware implementation.

# Proposed method

In order to improve the CABAC throughput of TS residual parsing, the proposed method derives the Rice parameter based on the mapped level value instead of the inverse mapped level value. In the proposed method, the context models and Rice parameters both depend on the mapped value (whereas, in VVC 6, context models are derived from mapped value, but the Rice parameters are derived from inverse mapped level value), it is not required to perform inverse mapping operations during residual parsing process. This allows the inverse mapping to be de-coupled from the residual parsing process; in other words, the inverse mapping may be executed after the residuals of the entire transform block have been parsed. Table 1 shows the part of the residual coding syntax table of the proposed method.

Table 1: Syntax table of the proposed residual parsing ( there is no inverse level mapping)

| /* remainder scan pass */ |  |
| --- | --- |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 4 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| if( intra_bdpcm_flag = = 0 ) { |  |
| absRightCoeff = abs( TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC − 1 ][ yC ] ) |  |
| absBelowCoeff = abs( TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC − 1 ] ) |  |
| predCoeff = Max( absRightCoeff, absBelowCoeff ) |  |
| if( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] = = 1 && predCoeff > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * predCoeff |  |
| else if( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] <= predCoeff ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] − 1) |  |
| else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } else |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| } |  |
| } |  |

In the proposed method, the inverse mapping is performed after residual parsing process and before inverse quantization. Assume TransCoeffLevel [xC][yC] is the coefficient value of the (xC, yC) position after residual parsing and TransCoeffLevelInvMapped [xC][yC] is the coefficient value of the (xC, yC) position after inverse mapping.

for (int yC = 0; yC < height; yC++){

for (int xC = 0; xC < width; xC++){

TransCoeffLevelInvMapped [xC][yC] = TransCoeffLevel [xC][yC];

if (TransCoeffLevel [xC][yC])

{

topPos = abs (TransCoeffLevelInvMapped [xC][yC-1]);

leftPos = abs(TransCoeffLevelInvMapped [xC - 1][yC]);

if (topPos | | leftPos)

{

int absMappedLevel = abs(TransCoeffLevel [xC][yC]);

int sign = TransCoeffLevel [xC][yC] < 0;

int pred1 =max(topPos, leftPos);

if (absMappedLevel == 1)

TransCoeffLevelInvMapped [xC][yC]= pred1;

else

TransCoeffLevelInvMapped [xC][yC] = absMappedLevel - (absMappedLevel <= pred1);

TransCoeffLevelInvMapped [xC][yC] = sign ? - TransCoeffLevelInvMapped [xC][yC]

: TransCoeffLevelInvMapped [xC][yC];

}

}

}

}

The proposed decoupling method also unify the pipeline/processing flow between transform-skip mode and BDPCM mode. In VVC6, when transform skip mode is selected, there is no BDPCM prediction but there is level mapping. Similarly, when BDPCM mode is selected there is no level mapping but there is BDPCM prediction. Figure 1 shows the processing flow of transform-skip mode and BDPCM mode of VVC6. In the proposed method, inverse level mapping is decoupled from the residual coding unit. The processing flow of the proposed method, shown in Figure 2, shows that with the proposed method, the processing flow of transform-skip mode and BDPCM mode becomes unified.

Figure 1: Processing flow of transform-skip and BDPCM mode of VVC 6

Figure 2: processing flow of transform-skip and BDPCM mode of proposed method

# Simulation results

The proposed methods are tested under both CTC and low QP (2, 7, 12, 17) settings.

Table 1 shows the results of proposed method under CTC test conditions whereas Table 2 shows the simulation results of the proposed method with low QP setting.

Table 2: Results of the proposed method ( CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | -0.01% | -0.01% | 101% | 102% |
| Class C | 0.00% | 0.01% | 0.01% | 100% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | -0.01% | 0.00% | 99% | 106% |
| Class F | 0.00% | -0.02% | -0.01% | 100% | 101% |
| Class SCC | 0.01% | -0.01% | 0.00% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.00% | 101% | 101% |
| Class C | 0.01% | 0.01% | -0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.08% | 0.10% | 101% | 100% |
| Class F | 0.00% | 0.06% | -0.01% | 101% | 103% |
| Class SCC | 0.05% | -0.02% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.15% | -0.02% | 100% | 100% |
| Class C | -0.01% | 0.01% | -0.02% | 101% | 104% |
| Class E | -0.01% | -0.01% | 0.01% | 101% | 102% |
| Overall | 0.00% | 0.06% | -0.01% | 101% | 102% |
| Class D | 0.04% | 0.14% | 0.00% | 101% | 106% |
| Class F | 0.14% | 0.16% | 0.71% | 100% | 100% |
| Class SCC | 0.01% | 0.02% | -0.04% | 101% | 105% |

Table 3: results of the proposed method at low QP ( RA results of the camera capture contents will be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.01% | 0.00% | 98% | 100% |
| Class E | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.01% | 0.00% | 101% | 102% |
| Class F | 0.03% | 0.03% | 0.00% | 100% | 100% |
| Class SCC | 0.12% | 0.08% | 0.08% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.03% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.02% | 0.00% | 100% | 101% |
| Class F | -0.07% | -0.18% | -0.18% | 101% | 100% |
| Class SCC | 0.19% | 0.13% | 0.13% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.01% | 101% | 98% |
| Class C | -0.01% | 0.00% | -0.01% | 101% | 100% |
| Class E | 0.00% | 0.01% | 0.04% | 100% | 98% |
| Overall | -0.01% | 0.00% | 0.00% | 101% | 98% |
| Class D | -0.03% | 0.04% | 0.02% | 100% | 100% |
| Class F | -0.02% | 0.00% | 0.06% | 101% | 100% |
| Class SCC | 0.19% | 0.17% | 0.18% | 101% | 98% |

# Additional results

The proposed method is also tested with reduced number of context coded bins for TS residual coding. The maximum context coded bins is set to 1.75 bins per coefficient. Table 4 and Table 5 show the results under CTC and low QP, respectively.

Table 4: results of the proposed method with 1.75 max ctx bins per coefficient CTC QP ( Anchor is VTM-6.0 with 2 bins per coefficient)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.10% | 0.13% | 0.19% | 102% | 98% |
| Class SCC | 0.27% | 0.22% | 0.24% | 102% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC | 0.23% | 0.14% | 0.11% | 101% | 88% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.11% | 0.01% | 0.39% | 102% | 89% |
| Class SCC | 0.05% | 0.09% | 0.12% | 102% | 90% |

Table 5: results of the proposed method with 1.75 bins per coefficient low QP ( Anchor: VTM-6.0 with maximum ctx bin 2 bins per coefficient)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.33% | 0.22% | 0.20% | 100% | 97% |
| Class SCC | 0.59% | 0.48% | 0.48% | 100% | 97% |

# Conclusion

In order to improve the CABAC throughput, this contribution proposes to decouple level mapping from the CABAC parsing process. The proposed method not only improves the CABAC parsing throughput but also unifies the pipeline of transform-skip and BDPCM mode. Simulation results shows the coding performance is similar to VTM-6.0 anchor.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).