| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0374-v3 |
| --- | --- |

| Title: | Non-CE3: One directional Intra planar mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He 4 Research Way 3rd Floor, Princeton, NJ 08540 USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The current Intra planar mode is essentially an equally weighted combination of both the horizontal and the vertical prediction components. We propose that for coding blocks with unequal width and height, only one direction prediction component from the larger dimension is used in Intra planar mode. Simulation result shows that: Test 1: applying one direction planar mode for blocks with unequal width and height yields +0.12%/+0.08%/+0.03% Y BD-rate change for AI/RA/LDB; Test 2: applying one direction planar mode for blocks with aspect ratio ≥ 4 yields +0.04%/+0.04%/+0.02% Y BD-rate change for AI/RA/LDB, both with similar coding time.

# Introduction

Intra planar mode is an important and commonly used Intra mode in practice. The current Intra planar prediction is essentially an equal weight average of the horizontal and vertical prediction component, as illustrated in Figure 3.

Vertical

prediction

Horizontal

prediction

Bottom left reference sample repeated padding to generate the bottom row

Top right corner reference sample repeated padding to generate the right column

Figure 1. The existing Intra planar prediction mode

The vertical prediction component is a weighted averge of the top row and bottom row reference samples, where the weight used is actually the inverse of the respective prediction distance. Similarly, the horizontal prediction component is also an inverse prediction distance weighted average over the left column and the right column of reference samples. Herein, the bottom row and the right column reference samples are actually padded by the left bottom and the top right corner reference sample, respectively.

However, in practice, if a coding block is of unequal width and height, one direction of prediction component may have much more dominant impact than the other. In this case, equal weights used in the current planar mode may not be appropriate or efficient.

# One directional Intra planar mode

We propose that in case of coding blocks with unequal width and height, either one of the horizontal or vertical prediction component, but not both, is used as the Intra planar prediction. In that case, if W > H, vertical planar prediction will be used. Otherwise, i.e. H > W, horizontal planar prediction will be used.

# Simulation results

CTC test result with VTM-6.0 is provided [1].

## Test 1: One direction planar mode if W ≠ H

In the first test, we apply one direction Intra planar mode for coding blocks with unequal width and height, i.e. with aspect ratio ≥ 2. The aspect ratio is defined as max( W / H, H / W ).

Table 1. Performance of the proposed one directional Intra planar mode for blocks with W≠H

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.20% | 0.09% | 101% | 100% |
| Class A2 | 0.03% | 0.10% | 0.09% | 101% | 99% |
| Class B | 0.14% | 0.29% | 0.21% | 100% | 99% |
| Class C | 0.10% | 0.07% | 0.21% | 100% | 101% |
| Class E | 0.16% | 0.21% | 0.18% | 100% | 101% |
| Overall | 0.12% | 0.18% | 0.17% | 100% | 100% |
| Class D | 0.09% | 0.23% | 0.25% | 100% | 99% |
| Class F | 0.05% | 0.05% | -0.01% | 100% | 99% |
| Class TGM | -0.10% | -0.12% | -0.09% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.09% | 0.12% | 101% | 101% |
| Class A2 | 0.06% | -0.02% | 0.02% | 100% | 100% |
| Class B | 0.09% | 0.08% | -0.03% | 100% | 99% |
| Class C | 0.08% | 0.02% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.05% | 0.02% | 100% | 100% |
| Class D | -0.02% | -0.19% | -0.04% | 100% | 101% |
| Class F | 0.04% | -0.03% | 0.09% | 100% | 100% |
| Class TGM | -0.03% | -0.07% | -0.05% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.02% | -0.13% | 100% | 104% |
| Class C | 0.00% | -0.08% | -0.12% | 100% | 97% |
| Class E | 0.03% | 1.04% | -0.08% | 100% | 98% |
| Overall | 0.03% | 0.23% | -0.12% | 100% | 100% |
| Class D | 0.02% | 0.46% | 0.56% | 100% | 99% |
| Class F | 0.11% | 0.00% | 0.39% | 100% | 99% |
| Class TGM | -0.05% | -0.04% | -0.09% | 100% | 98% |

## Test 2: One direction planar mode if aspect ratio ≥ 4

In the 2nd test, we apply one direction Intra planar mode for coding blocks with aspect ratio ≥ 4.

Table 2. Performance of the proposed one directional Intra planar mode for aspect ratio ≥ 4 coding blocks

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.08% | -0.01% | 100% | 99% |
| Class A2 | -0.01% | 0.03% | -0.01% | 101% | 99% |
| Class B | 0.06% | 0.16% | 0.07% | 100% | 100% |
| Class C | 0.04% | -0.06% | 0.11% | 100% | 100% |
| Class E | 0.06% | 0.17% | 0.05% | 100% | 101% |
| Overall | 0.04% | 0.08% | 0.05% | 100% | 100% |
| Class D | 0.05% | -0.05% | -0.09% | 100% | 99% |
| Class F | 0.04% | -0.03% | -0.08% | 100% | 101% |
| Class TGM | -0.08% | -0.09% | -0.09% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.12% | 0.10% | 101% | 101% |
| Class A2 | 0.04% | -0.06% | 0.03% | 101% | 100% |
| Class B | 0.02% | 0.09% | 0.05% | 100% | 100% |
| Class C | 0.04% | 0.14% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.08% | 0.07% | 100% | 100% |
| Class D | -0.04% | -0.33% | 0.09% | 100% | 101% |
| Class F | 0.03% | 0.00% | 0.07% | 100% | 101% |
| Class TGM | -0.06% | -0.05% | -0.09% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.01% | -0.18% | 101% | 103% |
| Class C | -0.01% | -0.05% | -0.18% | 100% | 99% |
| Class E | 0.01% | 0.59% | -0.30% | 100% | 100% |
| Overall | 0.02% | 0.13% | -0.21% | 100% | 101% |
| Class D | 0.04% | 0.14% | 0.00% | 100% | 100% |
| Class F | 0.01% | -0.15% | 0.40% | 100% | 101% |
| Class TGM | -0.09% | -0.06% | -0.01% | 101% | 100% |

# Conclusion

We propose to use either the horizontal or the vertical prediction component as calculated in the current Intra planar mode to be the Intra planar prediction for coding blocks with unequal width and height. Simulation result shows that with aspect ratio ≥ 2, the method yields +0.12%/+0.08%/+0.03% Y BD-rate change for AI/RA/LDB, while for aspect ratio ≥ 4, the performance becomes +0.04%/+0.04%/+0.02%.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,"JVET common test conditions and software reference configurations for SDR video", JVET-O2010, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).