| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0377-v1 |
| --- | --- |

| Title: | AHG12: Cleanups on syntax design of sub-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Jizheng Xu, Na Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With the current syntax design of sub-pictures, the sub-picture boundaries may not coincide with CTU boundaries. Besides it is possible to signal sub-pictures overlapped with each other, and all the sub-pictures may not cover the whole picture.

In this contribution, two aspects are proposed to cleanup the syntax design of sub-pictures.

Aspect #1: It is required that the sub-picture grid must be times of the CTU size.

Aspect #2: It is required that sub-pictures cannot be overlapped with each other, and all the sub-pictures must cover the whole picture.

# Introduction

With the current syntax design of sub-pictures in JVET-O2001-vE, the locations and dimensions of sub-pictures are derived as below:

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { |  |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) /
		( subpic_grid_col_width_minus1 * 4 + 4 )	(7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) /
		( subpic_grid_row_height_minus1 * 4 + 4 )	(7-6)

subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits.

The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows:

NumSubPics = 0
for( i = 0; i. < NumSubPicGridRows; i++ ) {
	for( j = 0; j < NumSubPicGridCols; j++ ) {
		if ( i = = 0)
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0
		else if( subpic_grid_idx[ i ][ j ]  !=  subpic_grid_idx[ i – 1 ][ j ] ) {
			SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i
			SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ]
		}
		if ( j = = 0)
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0	(7-7)
		else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) {
			SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j
			SubPicWidth[ subpic_grid_idx[ i ][ j - 1] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ]
		}
		if ( i = = NumSubPicGridRows − 1)
			SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1			if (j = = NumSubPicGridRows − 1)
			SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1
		if( subpic_grid_idx[ i ][ j ] > NumSubPics)
			NumSubPics = subpic_grid_idx[ i ][ j ]
	}
}

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

There are two problems in the current design.

#1: The grid unit is 4, which may result in sub-pictures not CTU-aligned. A CU may cover more than one sub-picture.

#2: Sub-pictures may be overlapped with each other and all the sub-pictures may not cover the whole picture.

For example, if subpic_grid_idx is signaled as: {{ 0, 1}, {1, 1}}, then sub-picture 0 will be a rectangle from (0, 0) to (G-1, G-1), where G represents the grid length. And sub-picture 1 will be a rectangle from (0, 0) to (2G-1, 2G-1), which contains sub-picture 0.

In addition, all sub-pictures may not cover the whole picture. For example, if subpic_grid_idx is signaled as: {{ 0, 1}, {1, 0}}, then sub-picture 0 will be a rectangle from (G, G) to (2G-1, 2G-1), where G represents the grid length. And sub-picture 1 will be a rectangle from (0, G) to (G-1, 2G-1). The rectangle from (0, 0) to (2G-1, G-1) is not covered by any sub-pictures.

# Proposed methods

To address the problems, two aspects are proposed as below.

Aspect #1: The sub-picture grid must be times of the CTU size.

subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY) ) bits.

The variable NumSubPicGridCols is derived as follows:

NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY-1) /
		( subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY)	(7-5)

subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY) ) bits.

The variable NumSubPicGridRows is derived as follows:

NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY-1 ) /
		( subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY)	(7-6)

Aspect #2: A conformance constrain is added to guarantee that sub-pictures cannot be overlapped with each other, and all the sub-pictures must cover the whole picture.

Any subpic_grid_idx[ i ][ j ] must be equal to idx if the following conditions are both satisfied:

- i >= SubPicTop[ idx ] and i < SubPicTop[ idx ] + SubPicHeight[ idx ].

- j >= SubPicLeft[ idx ] and j < SubPicLeft[ idx ] + SubPicWidth[ idx ].

Any subpic_grid_idx[ i ][ j ] must be different to idx if the following conditions are not both satisfied:

- i >= SubPicTop[ idx ] and i < SubPicTop[ idx ] + SubPicHeight[ idx ].

- j >= SubPicLeft[ idx ] and j < SubPicLeft[ idx ] + SubPicWidth[ idx ].

# Conclusions

Since the syntax design for sub-pictures can be cleaned up, it is recommended that the proposed methods be adopted into VVC.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).