| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0381-v1 |
| --- | --- |

| Title: | Non-CE1: A fix on reference sample offset for RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With RPR in VVC, a reference sample position is derived assuming that the offset between the top-left corner of the conformance window in the current picture and that in the reference picture is zero. However, the offset may be signalled to be non-zero. In this contribution, it is proposed to derive the reference sample position considering the non-zero offsets. Experimental results reportedly show that the proposed method achieves 16.91%, (PSNR1), 18.53%(PSNR2) BD-rate reductions for scaling ratio 1.5 and 14.89% (PSNR1), 15.25% (PSNR2) BD-rate reductions for scaling ratio 2.0, when the top-left corner of the conformance window in low-resolution pictures is at (64, 64).

# Introduction

In the current design of VVC, the reference sample position (xr, yr) is derived based on the current sample position (xc, yc) and a ratio between the Wr × Hr conformance window in the reference picture and the Wc × Hc conformance window in the current picture, formulated as

xr = xc ×Wr /Wc and yr = yc ×Hr /Hc.

However, when the top-left corner of the conformance window in the current picture and that in the reference picture are not at the same position, the derivation may result in a wrong position, as shown in Figure 1(a).

# Proposed methods

It is proposed to derive the reference sample position considering the non-zero offset between the two conformance windows as shown in Figure 1(b), formulated as

xr = (xc - OffXc) ×Wr /Wc+ OffXr and yr = (yc - OffYc) ×Hr /Hc + OffYr,

wherein (OffXc, OffYc) and (OffXr, OffYr) represent the top-left corners of conformance windows in the current picture and in the reference picture, respectively.

# Experimental Results

Simulations are conducted under the RPR test conditions required by CE1 [1].

To verify the proposed method, the top-left corner of the conformance window in a W × H low-resolution picture is set to be (64, 64) at both the anchor and the tested encoders. Therefore, the dimensions of the low-resolution picture to be coded is (W+64, H+64). Samples out of the conformance window are filled to be zero.

Table 1 and Table 2 demonstrates the coding performance of the proposed method, compared with VTM-6.1, with the same conformance window setting. Significant coding BD-rate reductions ( 16.91%, 18.53% for scaling ratio 1.5 and 14.89%, 15.25% for scaling ratio 2.0) can be achieved since the position offset error is fixed.

(a)

(b)

Figure 1 The positions of the current sample and its reference sample in JVET-O2001-vE (a) and the proposed method (b). Dashed rectangles represent the conformance windows.

Table 2: Results for scaling ratio = 1.5

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -16.12% | -21.00% | -21.71% | 95% | 100% | -16.32% | -19.13% | -19.52% | 95% | 100% |
| Class C | -9.21% | -10.13% | -9.02% | 98% | 101% | -8.28% | -8.51% | -7.32% | 98% | 101% |
| Class E | -28.48% | -47.97% | -48.31% | 89% | 101% | -35.87% | -45.12% | -46.30% | 89% | 101% |
| Overall | -16.91% | -24.12% | -24.13% | 94% | 101% | -18.53% | -22.09% | -22.15% | 94% | 101% |
| Class D | -10.12% | -13.76% | -12.75% | 97% | 101% | -9.18% | -13.70% | -12.12% | 97% | 101% |

Table 3: Results for scaling ratio = 2.0

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | -16.66% | -24.33% | -24.91% | 97% | 101% | -14.48% | -20.38% | -20.50% | 97% | 101% |
| Class C | -7.58% | -14.56% | -13.91% | 99% | 99% | -6.63% | -10.73% | -9.91% | 99% | 99% |
| Class E | -21.67% | -47.18% | -47.74% | 92% | 99% | -28.01% | -40.05% | -42.26% | 92% | 99% |
| Overall | -14.89% | -26.79% | -26.95% | 96% | 100% | -15.25% | -22.08% | -22.41% | 96% | 100% |
| Class D | -6.46% | -14.61% | -14.17% | 98% | 102% | -5.75% | -13.62% | -12.12% | 98% | 102% |

# Changes on the Working Draft

The working draft specified in JVET-O2001-v14 are changed as below.

##### General

…

The variable fRefWidth is set equal to the PicOutputWidthL of the reference picture in luma samples.

The variable fRefHeight is set equal to PicOutputHeightL of the reference picture in luma samples.

The vairable fRefLeftOff is set equal to the conf_win_left_offset of the reference picture in luma samples.

The vairable fRefTopOff is set equal to the conf_win_top_offset of the reference picture in luma samples.

The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

- If cIdx is equal to 0, the following applies:

  - The scaling factors and their fixed-point representations are defined as

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntL, ySbIntL ) is set equal to ( xSb + ( mvLX[ 0 ]  >>  4 ), ySb + ( mvLX[ 1 ]  >>  4 ) ).

  - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

refxSbL = (( ( xSb - conf_win_left_offset) <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp 	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) +( fRefLeftOff<<10) + 32 )  >>  6	 (8-756)

refySbL = ( ( (ySb - conf_win_top_offset) <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp 	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) +( fRefTopOff<<10)+ 32 )  >>  6	(8-758)

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = refxL  >>  4		(8-759)

yIntL = refyL  >>  4		(8-760)

xFracL = refxL & 15		(8-761)

yFracL = refyL & 15		(8-762)

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight and ( xSb, ySb ) as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntC, ySbIntC ) is set equal to ( (xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5), ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) ).

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- Let (refxSbC, refySbC) and ( refxC, refyC ) be chroma locations pointed to by a motion vector (mvLX[ 0 ], mvLX[ 1 ]) given in 1/32-sample units. The variables refxSbC, refySbC, refxC and refyC are derived as follows:

refxSbC =  ( (( xSb - conf_win_left_offset) / SubWidthC  <<  5 ) + mvLX[ 0 ] ) * hori_scale_fp	 (8-763)

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 ) 	+ xC * ( ( hori_scale_fp + 8 )  >>  4 ) ) + ( fRefLeftOff/ SubWidthC <<10) + 16 )  >>  5	 (8-764)

refySbC =  ( ( (ySb - conf_win_top_offset) / SubHeightC  <<  5 )  + mvLX[ 1 ] ) * vert_scale_fp	 (8-765)

refyC = ( ( Sign( refySbC ) * ( ( Abs( refySbC ) + 256 )  >>  9 ) + yC* ( ( vert_scale_fp + 8 )  >>  4 ) ) + ( fRefTopOff/ SubHeightC <<10) + 16 )  >>  5 	(8-766)

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows:

xIntC = refxC  >>  5		(8-767)

yIntC = refyC  >>  5		(8-768)

xFracC = refyC & 31		(8-769)

yFracC = refyC & 31		(8-770)

# Conclusions

Since the offset error of the reference sample derivation in RPR can be fixed, it is recommended that the proposed methods be adopted into VVC.

# References

- D. Luo, V. Seregin, W. Wan, “Description of Core Experiment 1 (CE1): Reference picture resampling filters,” JVET-O2021, Jul. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).