| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0382-v1 |
| --- | --- |

| Title: | Non-CE1: Simplified motion compensation in RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Zhipin Deng, Jizheng Xu, Na Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With RPR in VVC, the bandwidth and computational complexity demanded by motion compensation is higher when the reference picture is larger than the current picture. In this contribution, it is proposed to apply the 6-tap interpolation filters used in the affine motion compensation on a luma M × N block satisfying certain conditions when the scaling ratio is larger than 1. Two tests are conducted with different conditions as below:

Test #1: The luma block is bi-predicted.

Test #2: The luma block is bi-predicted and (M × N <= 64 || M == 4 || N== 4)

For both tests with the scaling ratio equal to 2, the required bandwidth of motion compensation is asserted to be reduced to 82%, and the required number of multiplications is asserted to be reduced to 67% in the worst case. Experimental results reportedly show that the BD-rate changes are 0.00%, 0.00% for scaling ratio 1.5 and 0.11%, 0.06% for scaling ratio 2.0 in test #1, and 0.00%, 0.00% for scaling ratio 1.5 and 0.00%, 0.00% for scaling ratio 2.0 in test #2, respectively, under the CTC for RPR.

# Introduction

In the current design of VVC, the reference sample position (xr, yr) is derived based on the current sample position (xc, yc) and a scaling ratio between the Wr × Hr conformance window in the reference picture and the Wc × Hc conformance window in the current picture, formulated as

xr = xc ×Wr /Wc and yr = yc ×Hr /Hc.

In the cases when the scaling ratio is larger than one, a higher bandwidth and more computations are required to conduct the MC for a M × N block than in the cases when the scaling ratio is equal to one. Theoretically, ( M + 7 ) × ( N + 7 ) integer samples and 8 × ( M × ( N + 7 ) + M × N ) multiplications are required when the reference picture has the same resolution as the current picture, while ( 2M + 7 ) × ( 2N + 7 ) integer samples and 8 × ( M × ( 2N + 7 ) + M × N ) multiplications are required when the scaling ratio is equal to two. Table 1 summarizes the integer samples and multiplications per sample required by luma blocks with different dimensions when the scaling ratio is equal to 1 and 2. It is found that the required bandwidth in the worst case is increased to 231%, and the required multiplications in the worst case are increased to 127%, when the scaling ratio is equal to 2. The increasement could be obtained by dividing the highlighted numbers under scaling ratio equal to 2 by that under scaling ratio equal to 1.

# Proposed methods

To reduce the required bandwidth and computations, it is proposed to apply the 6-tap interpolation filters used in the affine motion compensation on a luma M × N block satisfying certain conditions when the scaling ratio is larger than 1. Two tests are conducted with different conditions as below:

Test #1: The luma block is bi-predicted.

Test #2: The luma block is bi-predicted and (M × N <= 64 || M == 4 || N== 4)

Table 2 summarizes the integer samples and multiplications per sample required by luma blocks with different dimensions when the scaling ratio is equal to 1 and 2 with the proposed method. With the both the two tests, the required bandwidth in the worst case is decreased to 82%, the required multiplications in the worst case are decreased to 67%, when the scaling ratio is equal to 2.

Table 1 Bandwidth/multiplications required in VTM-6.1

| M | M | integer samples/sample |  |  |  | multiplications/sample |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ratio = 1 |  | ratio = 2 |  | ratio = 1 |  | ratio = 2 |  |
|  |  | Uni | Bi | Uni | Bi | Uni | Bi | Uni | Bi |
| 4 | 8 | 5.16 | - | 10.78 | - | 23.00 | 46.00 | 31.00 | 62.00 |
| 4 | 16 | 3.95 | 7.91 | 9.14 | 18.28 | 19.50 | 39.00 | 27.50 | 55.00 |
| 4 | 32 | 3.35 | 6.70 | 8.32 | 16.64 | 17.75 | 35.50 | 25.75 | 51.50 |
| 4 | 64 | 3.05 | 6.10 | 7.91 | 15.82 | 16.88 | 33.75 | 24.88 | 49.75 |
| 8 | 4 | 5.16 | - | 10.78 | - | 30.00 | - | 38.00 | - |
| 16 | 4 | 3.95 | 7.91 | 9.14 | 18.28 | 30.00 | 60.00 | 38.00 | 76.00 |
| 32 | 4 | 3.35 | 6.70 | 8.32 | 16.64 | 30.00 | 60.00 | 38.00 | 76.00 |
| 64 | 4 | 3.05 | 6.10 | 7.91 | 15.82 | 30.00 | 60.00 | 38.00 | 76.00 |
| 8 | 8 | 3.52 | 7.03 | 8.27 | 16.53 | 23.00 | 46.00 | 31.00 | 62.00 |
| 8 | 16 | 2.70 | 5.39 | 7.01 | 14.02 | 19.50 | 39.00 | 27.50 | 55.00 |
| 8 | 32 | 2.29 | 4.57 | 6.38 | 12.76 | 17.75 | 35.50 | 25.75 | 51.50 |
| 8 | 64 | 2.08 | 4.16 | 6.06 | 12.13 | 16.88 | 33.75 | 24.88 | 49.75 |
| 16 | 8 | 2.70 | 5.39 | 7.01 | 14.02 | 23.00 | 46.00 | 31.00 | 62.00 |
| 32 | 8 | 2.29 | 4.57 | 6.38 | 12.76 | 23.00 | 46.00 | 31.00 | 62.00 |
| 64 | 8 | 2.08 | 4.16 | 6.06 | 12.13 | 23.00 | 46.00 | 31.00 | 62.00 |
| 16 | 16 | 2.07 | 4.13 | 5.94 | 11.88 | 19.50 | 39.00 | 27.50 | 55.00 |

Table 2 Bandwidth/multiplications required in the proposed method

| M | M | integer samples/sample |  |  |  | multiplications/sample |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ratio = 2/Test 1 |  | ratio = 2/Test 2 |  | ratio = 2/Test 1 |  | ratio = 2/Test 2 |  |
|  |  | Uni | Bi | Uni | Bi | Uni | Bi | Uni | Bi |
| 4 | 8 | 10.78 | - | 10.78 | - | 23.00 | - | 31.00 | - |
| 4 | 16 | 9.14 | 15.03 | 9.14 | 15.03 | 19.50 | 39.75 | 27.50 | 39.75 |
| 4 | 32 | 8.32 | 14.02 | 8.32 | 14.02 | 17.75 | 37.88 | 25.75 | 37.88 |
| 4 | 64 | 7.91 | 13.51 | 7.91 | 13.51 | 16.88 | 36.94 | 24.88 | 36.94 |
| 8 | 4 | 10.78 | - | 10.78 | - | 30.00 | - | 38.00 | - |
| 16 | 4 | 9.14 | 15.03 | 9.14 | 15.03 | 30.00 | 51.00 | 38.00 | 51.00 |
| 32 | 4 | 8.32 | 14.02 | 8.32 | 14.02 | 30.00 | 51.00 | 38.00 | 51.00 |
| 64 | 4 | 7.91 | 13.51 | 7.91 | 13.51 | 30.00 | 51.00 | 38.00 | 51.00 |
| 8 | 8 | 8.27 | 13.78 | 8.27 | 13.78 | 23.00 | 43.50 | 31.00 | 43.50 |
| 8 | 16 | 7.01 | 12.14 | 7.01 | 14.02 | 19.50 | 39.75 | 27.50 | 55.00 |
| 8 | 32 | 6.38 | 11.32 | 6.38 | 12.76 | 17.75 | 37.88 | 25.75 | 51.50 |
| 8 | 64 | 6.06 | 10.91 | 6.06 | 12.13 | 16.88 | 36.94 | 24.88 | 49.75 |
| 16 | 8 | 7.01 | 12.14 | 7.01 | 14.02 | 23.00 | 43.50 | 31.00 | 62.00 |
| 32 | 8 | 6.38 | 11.32 | 6.38 | 12.76 | 23.00 | 43.50 | 31.00 | 62.00 |
| 64 | 8 | 6.06 | 10.91 | 6.06 | 12.13 | 23.00 | 43.50 | 31.00 | 62.00 |
| 16 | 16 | 5.94 | 10.70 | 5.94 | 11.88 | 19.50 | 39.75 | 27.50 | 55.00 |

# Experimental Results

Simulations are conducted under the RPR test conditions required by CE1 [1].

Table 3/Table 4 and Table 5/Table 6 demonstrate the coding performance of the proposed method, compared with VTM-6.1 for test #1 and test #2, respectively. Negligible BD-rate changes (0.00%, 0.00% for scaling ratio 1.5 and 0.11%, 0.06% for scaling ratio 2.0 in test #1 and 0.00%, 0.00% for scaling ratio 1.5 and 0.00%, 0.00% for scaling ratio 2.0 in test #2) are reported.

Table 3: Results in test #1 for scaling ratio = 1.5

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.00% | 0.11% | -0.02% | 102% | 101% | -0.01% | 0.15% | 0.03% | 100% | 100% |
| Class C | -0.02% | -0.31% | -0.21% | 100% | 99% | -0.03% | -0.20% | -0.18% | 99% | 98% |
| Class E | 0.03% | 0.40% | 0.74% | 100% | 95% | 0.06% | 0.55% | 0.55% | 100% | 97% |
| Overall | 0.00% | 0.04% | 0.11% | 101% | 99% | 0.00% | 0.13% | 0.09% | 100% | 99% |
| Class D | -0.02% | -0.07% | 0.01% | 102% | 100% | 0.00% | 0.13% | 0.29% | 101% | 100% |

Table 4: Results in test #1 for scaling ratio = 2.0

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.09% | 0.27% | 0.16% | 99% | 100% | 0.02% | 0.22% | 0.14% | 99% | 100% |
| Class C | -0.01% | -0.01% | -0.12% | 98% | 97% | -0.01% | -0.01% | 0.01% | 98% | 97% |
| Class E | 0.32% | 0.10% | 0.13% | 100% | 96% | 0.22% | 0.14% | 0.01% | 100% | 96% |
| Overall | 0.11% | 0.13% | 0.06% | 99% | 98% | 0.06% | 0.12% | 0.06% | 99% | 98% |
| Class D | -0.01% | 0.14% | -0.21% | 100% | 96% | -0.04% | 0.06% | 0.01% | 100% | 96% |

Table 5: Results in test #2 for scaling ratio = 1.5

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.02% | 0.25% | 0.00% | 101% | 99% | 0.02% | 0.27% | -0.01% | 101% | 99% |
| Class C | 0.00% | 0.00% | -0.04% | 99% | 98% | -0.01% | 0.00% | -0.03% | 99% | 98% |
| Class E | -0.04% | 0.31% | 0.55% | 100% | 99% | -0.03% | 0.31% | 0.51% | 100% | 99% |
| Overall | 0.00% | 0.18% | 0.12% | 100% | 99% | 0.00% | 0.19% | 0.11% | 100% | 99% |
| Class D | -0.02% | 0.49% | -0.12% | 99% | 98% | -0.03% | 0.59% | -0.26% | 99% | 98% |

Table 6: Results in test #2 for scaling ratio = 2.0

|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.01% | 0.30% | 0.24% | 100% | 99% | -0.02% | 0.28% | 0.19% | 100% | 99% |
| Class C | -0.10% | 0.07% | -0.07% | 97% | 97% | -0.03% | 0.02% | 0.05% | 97% | 97% |
| Class E | 0.11% | -0.07% | 0.13% | 100% | 99% | 0.07% | -0.10% | -0.01% | 100% | 99% |
| Overall | 0.00% | 0.13% | 0.11% | 99% | 99% | 0.00% | 0.10% | 0.09% | 99% | 99% |
| Class D | 0.04% | -0.62% | -0.25% | 99% | 97% | 0.03% | -0.39% | -0.19% | 99% | 97% |

# Changes on the Working Draft

The working draft specified in JVET-O2001-v14 are changed as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

#### 8.5.6.3 Fractional sample interpolation process

##### 8.5.6.3.1 General

…

  - using6TapFlag is set to be 1 if all the conditions below are satisfied:

    - cbWidth[0][xSb][ySb] <=4 | | cbHeight[0][xSb][ySb] <=4 | | cbWidth[0][xSb][ySb] * cbHeight[0][xSb][ySb] <=64.

    - PredFlagL0[xSb][ySb] == 1 && PredFlagL1[xSb][ySb] == 1.

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight and ( xSb, ySb ) and using6TapFlag as inputs.

...

##### 8.5.6.3.2 Luma sample interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- a luma location in full-sample units ( xSbIntL, ySbIntL ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left luma sample of the reference picture,

- the luma reference sample array refPicLXL,

- the half sample interpolation filter index hpelIfIdx,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture,

- a flag using6TapFlag specfiying whether the 6-tap interpolation filter used.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

- The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

- The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If at least one of the following conditions are satisfied, the luma interpolation filter coefficients fL[ p ] are specified in Table 8-12.

  - MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4,

  - using6TapFlag is equal to 1.

…

# Conclusions

Since both the bandwidth and computations can be reduced when the scaling ratio is larger than 1 for RPR coding with a negligible performance drop, it is recommended that the proposed test #2 be adopted into VVC.

# References

- D. Luo, V. Seregin, W. Wan, “Description of Core Experiment 1 (CE1): Reference picture resampling filters,” JVET-O2021, Jul. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).