# Changes on the Working Draft

The working draft specified in JVET-O2001-v14 are changed as below. The newly added parts are highlighted in yellow. The removed parts are striked.

#### 8.5.5.3 Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- …

Outputs of this process are:

- ….

…

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to tempMv[ 0 ], rightShift set equal to 4, and leftShift set equal to 0 as inputs and the rounded tempMv[ 0 ] as output.

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to tempMv[ 1 ], rightShift set equal to 4, and leftShift set equal to 0 as inputs and the rounded tempMv[ 1 ] as output.

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth + sbWidth / 2	(8-551)

ySb  =  yCb + ySbIdx * sbHeight + sbHeight / 2	(8-552)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

    - The following applies:

yColSb = Clip3( yCtb,
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ), 	ySb + ( tempMv[1]   >>  4 ) ) 	(8-553)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xColSb = Clip3( xCtb, 
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), 	xSb + ( tempMv[0] >>  4 ) )	 (8-554)

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xColSb = Clip3( xCtb,
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xSb + ( tempMv[0] >>  4 ) )	(8-555)

…

#### 8.5.5.4 Derivation process for subblock-based temporal merging base motion data

Inputs to this process are:

- ...

Outputs of this process are:

- ...

The variable tempMv is set as follows:

tempMv[ 0 ] = 0		(8-558)

tempMv[ 1 ] = 0		(8-559)

The variable currPic specifies the current picture.

When availableFlagA1 is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMv is set equal to mvL0A1:

  - predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0A1]) is equal to 0,

- Otherwise, if all of the following conditions are true, tempMv is set equal to mvL1A1:

  - slice_type is equal to B,

  - predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1A1]) is equal to 0.

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to tempMv[ 0 ], rightShift set equal to 4, and leftShift set equal to 0 as inputs and the rounded tempMv[ 0 ] as output.

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to tempMv[ 1 ], rightShift set equal to 4, and leftShift set equal to 0 as inputs and the rounded tempMv[ 1 ] as output.

The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows.

  - The following applies:

yColCb = Clip3( yCtb,
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ), yColCtrCb + ( tempMv[1] >>  4 ) ) 	(8-560)

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xColCb = Clip3( xCtb,
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xColCtrCb + ( tempMv[0] >>  4 ) )	 (8-561)

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to o, the following applies:

xColCb = Clip3( xCtb,
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ), xColCtrCb + ( tempMv[0] >>  4 ) ) 	(8-562)