| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0386-v1 |
| --- | --- |

| Title: | Non-CE8/Non-CE5: Deblocking for IBC blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei) | Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

IBC virtual dedicated buffer concept is adopted in 15th Gothenburg JVET meeting. With the IBC virtual dedicated buffer concept, the block vectors are first clip using modulo operation before referencing, however the non-clip block vectors are used to derive the boundary strength to deblocking to adjacent IBC blocks, which lead to wrong deblocking decision. In this contribution, a method is proposed to fix the mentioned problem. With the proposed method, a ClassF luma performance changing of 0.00%/0.00%/0.00% for AI/RA/LDB, a TGM420 luma performance changing of 0.00%/0.00%/0.00% for AI/RA/LDB  and a SCC(CE8 condition) luma performance changing of 0.00%/0.00%/0.00% for AI/RA/LDB are observed. No encoding and decoding time changes are observed.

# Introduction

## Virtual IBC buffer and block vector modulo

In 15th Gothenburg JVET meeting, the bitstream conformance with a virtual IBC buffer concept is adopted [1]. With the virtual IBC buffer concept, the IBC predictor is referencing from: ibcVirBuf[ (x + bv[0]>>4)& (wIbcVirBuf – 1)] [ (y + bv[1]>>4) & (ctbSize – 1) ], where ibcVirBuf is is updated and refreshed based on the following rules:

1)	At the beginning of decoding each CTU row, refresh the whole ibcVirBuf with value (-1).

2)	At the beginning of decoding a VPDU (xVPDU, yVPDU) relative to the top-left corner of the picture, set the ibcVirBuf[ x ][ y ] = -1, with x = xVPDU%wIbcVirBuf, …, xVPDU% wIbcVirBuf + Wv – 1; y = yVPDU%ctbSize, …, yVPDU%ctbSize + Wv – 1.

3)	After decoding a CU contains (x, y) relative to the top-left of the picture, set ibcVirBuf[ x & (wIbcVirBuf-1) ][ y & (ctbSize-1) ] = recSample[ x ][ y ]

With this virtual IBC buffer concept, several different block vectors can pointing is possible pointing to the same IBC reference blocks as shown in Figure 1. Since after the modulo operation ((x + bv[0])& (wIbcVirBuf-1) , (y + bv[1]) & (ctbSize-1)), all 3 BVs are referencing from the block located in the left CTB.

Figure 1 different BVs referring same IBC refereeing block

## Current boundary strength derivation for IBC blocks in VVC-6.0

Based on current VVC-6.0 drafted Text [2], if two adjacent blocks are both coded with IBC mode, the boundary strength for deblocking is derived by difference of the two block vectors. More specific:

- Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1: - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

# Problem Statement

With the IBC BV modulo operation, the block vectors used during IBC referencing and block vectors stored for deblocking adjacent IBC blocks can be different, which lead to wrong deblocking boundary strength decision. For example in. Figure 2, block 1 and block 2 are adjacent with vertical edge, BV1 and BV2 are the stored (signaled) block vectors of block 1 and blocks to be used in deblocking. Since the different between BV1 and BV2 are larger than the threshold, based on the boundary strength derivation for IBC blocks in VVC-6.0, deblocking is triggered.

However, in Figure 3, the real referencing blocks in the virtual IBC buffer is illustrated. BV1’ and BV2’ are BV1 and BV2 after modulo opetration for block 1 and bock 2 referencing. Since the referencing blocks are also adjacent, deblocking process should not be triggered for this vertical edge.

Figure 2 block vectors for adjacent IBC blocks

Figure 3 IBC referencing blocks for figure 2 case in the IBC virtual buffer

In addition, since the block vector in VVC-6.0 for IBC is stored in fractional. If two fractional block vector are accidently parsed for two adjacent IBC block and the different greater or equal 8 but smaller than 16. Based on current specification, this case also deblocked. But since IBC only using integer block vectors, this case should not be deblocked.

# Proposal

In this proposal, modulo and right-shift operations are applied before deblocking filtering same as before IBC motion compensation. The proposed fix solves the problem of wrong boundary strength decision for deblocking adjacent IBC blocks.

As shown in the following highlighted (yellow part is removed and green part is added) specification changes:

- Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1: - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples. at least one of the following conditions are true: - ( mvp0[ 0 ] >> 4 ) & ( IbcBufWidthY – 1 ) is not equal to ( mvq0[0] >> 4 ) & ( IbcBufWidthY – 1 ) - ( mvp0[ 1 ] >> 4 ) & ( CtbSizeY – 1 ) is not equal to ( mvq0[1] >> 4 ) & ( CtbSizeY – 1 )

For two adjacent IBC blocks boundary strength derviation, the block vectors of these two IBC blocks is shifted down to integer precision and moduloed by the size of IBC virtual buffer. If any of the moduloed block vectors component is different, deblocking process is triggered; otherwise (all component of the moduloed block vectors are same), deblocking process is not triggered (Bs is set as 0).

With the proposed method, as one example of Figure 2, the BV1 and BV2 after modulo are equal both in horizontal and vertical component. The deblocking process for this two adjacent IBC blocks are not triggered.

# Simulation Results

The Table 1 illustrated the simulation results of the proposed method on top of VTM-6.0 for ClassF, TGM420 and 444SCC (CE8 condition). Detailed results can be found in the attached excel document.

It’s noted that the performance of the proposed method compared with VTM-6.0 has no changes, since the VTM-6.0 encoder restricted the block vector search area in the current or left CTU partially.

Table 1 simulation results of the proposed method

| Class | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| ClassF | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| TGM420 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| SCC(OverAll) | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |

# Conclusion

This contribution proposed a method to fix the wrong boundary strength derivation for two IBC blocks. The simulation results on top of VTM-6.0 shows negligible coding performance or complexity changes. It’s proposed to be adopted in the next version of VVC draft.

# References

- H. Gao, S. Esenlik, E. Alshina, A.Karabutov, S. Ikonin, B. Wang, A. M. Kotra “Bitstream conformance with a virtual IBC buffer concept”, JVET-O1171, Gothenburg, SE, 3–12 July 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, SE, 3–12 July 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).