| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0390-v2 |
| --- | --- |

| Title: | CE1-related: Anti-aliasing motion compensation interpolation downsampling filters for reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Jill Boyce Pavel Frolov Vasily Aristarkhov | Email: | alexander.alshin@intel.com jill.boyce@intel.com pavel.frolov@intel.com vasily.aristarkhov@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

VTM6.0 supports reference picture resampling, which is being studied in CE1. In VTM6, existing sub-pel interpolation filters for motion compensation are used to resample the reference picture for inter-picture prediction. Using these conventional “all pass” interpolation filters may encounter the well-known restrictions of Nyquist–Shannon sampling theorem and cause aliasing effect if the reference picture size is larger than the current picture size.  This contribution proposes to reduce the aliasing effect by switching to a different set of filter coefficients, maintaining the 8-tap filter size and the filter coefficient sum of 64, when the reference picture size exceeds the current picture size by a 2x ratio per dimension or more.

The -v2 version provides additional experimental results.

# Proposal

The VTM6 specification defines 16 filters for luma with 8-taps, and 32 filters for Chroma with 4-taps to perform sub-pel motion compensated interpolation. VTM6 uses the those existing motion compensation filters to resample reference pictures when the reference picture resampling (RPR) feature is used. CE1 provides test conditions for resampling filters for RPR.

The sub-pel motion compensation interpolation filters are sub-optimal when the reference picture is significantly larger than current picture size, because they have an “all pass” rather than “low pass” characteristic.  Ideal filters for resampling would be based upon the particular resampling ratio, but it would require more complex implementation for the codec to support many sets of filter coefficients.

This contribution proposes a new set of motion compensation filter coefficients to be used when the reference picture is 2x larger or more than the current picture in a particular dimension, or more precisely when the number of luma samples is 4x or more higher in the reference picture than in the current picture. For small picture size ratios, the existing VTM6 filter coefficients are used. The proposed motion compensation filter coefficients are 8-tap for luma,, the same number of taps as for the existing VTM6 design. Sum of integer coefficients are equal 64.

## Filter Design of Low pass MC filters for scale>=2.0

An 8-tap filter can be represented by coefficients  a1, a2,a3,a4,a5,a6,a7,a8,

Application this filter to function eiωx

ρeiωδ=18akeiωxk    (1)

The current VVC draft specification filters are “all pass” . All frequencies are transmitted without significant cut off. For 2.0x scaling, the cut off frequency is smaller and optimal MC filters have noticeable decay in high frequency area

The following filter coefficients in Table 1 are proposed, which were derived using trigonometric (DCT) interpolation with high frequency decay. Using DCT basis for interpolation has one clear advantage. It significantly reduces contribution of quantization error to final MSE of interpolation.

In our proposal the sum of filter coefficients is equal 64. The conventional MC filters in VVC also have 64 as the sum of filter coefficients. Maintaining the 8 tap filter with the same sum of filter coefficients allows reusing of regular MC filtering implementation and avoids increasing the number of required gates to support a larger number of bits for intermediate stages if the sum of filter coefficients is increased.

Table 1. Proposed Motion compensation interpolation filter coefficients for 2x scaling

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | -7 | 5 | 21 | 26 | 21 | 5 | -7 | 0 |
| 1 | -7 | 4 | 20 | 27 | 22 | 6 | -8 | 0 |
| 2 | -6 | 3 | 18 | 27 | 24 | 7 | -9 | 0 |
| 3 | -4 | 0 | 13 | 35 | 21 | 1 | -2 | 0 |
| 4 | −4 | 0 | 11 | 35 | 23 | 2 | -2 | −1 |
| 5 | −3 | 0 | 10 | 32 | 25 | 3 | -2 | −1 |
| 6 | −3 | -1 | 9 | 32 | 26 | 4 | -2 | −1 |
| 7 | −3 | -1 | 7 | 32 | 28 | 5 | -2 | −2 |
| 8 | −2 | -1 | 6 | 29 | 29 | 6 | -1 | −2 |
| 9 | −2 | -2 | 5 | 28 | 32 | 7 | -1 | −3 |
| 10 | −1 | -2 | 4 | 26 | 32 | 9 | -1 | −3 |
| 11 | −1 | -2 | 3 | 25 | 32 | 10 | 0 | −3 |
| 12 | −1 | -2 | 2 | 23 | 35 | 11 | 0 | −4 |
| 13 | 0 | -2 | 1 | 21 | 35 | 13 | 0 | −4 |
| 14 | 0 | -9 | 7 | 24 | 27 | 18 | 3 | -6 |
| 15 | 0 | -8 | 6 | 22 | 27 | 20 | 4 | -7 |

# Test results

The CE1 experimental conditions in JVET-O02021 were used in the tests. If fractional position  is equal zero VTM s/w uses copying (m_filterCopy). It is necessary to substitute function m_filterCopy for lowpass filtration. We implemented our modifications on top reference s/w VTM-6.1_CE1. The results are presented below.  We also checked our modifications on top  of reference s/w VTM-6.1_CE1 with VTM-6.1_RPR_affine_patch.diff.

## 2x scaling ratio VTM-6.1_CE1 without VTM-6.1_RPR_affine_patch.diff

| PSNR1 | PSNR2 |
| --- | --- |
| Low delay B Main10. Over VTM-6.0_CE1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -2.92%; -1.55%; -1.48%; 104%; 106%. Class C; -5.93%; -3.69%; -3.57%; 98%; 100%. Class E; -8.31%; -3.01%; -3.34%; 100%; 106%. Overall; -5.27%; -2.63%; -2.64%; 101%; 104%. Class D; -5.46%; -2.62%; -2.61%; 97%; 93%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!. | Low delay B Main10. Over VTM-6.0_CE1. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -1.83%; -0.97%; -0.81%; 104%; 106%. Class C; -3.53%; -2.09%; -2.20%; 98%; 100%. Class E; -4.34%; -1.19%; -2.33%; 100%; 106%. Overall; -3.02%; -1.40%; -1.66%; 101%; 104%. Class D; -2.94%; -1.87%; -1.80%; 97%; 93%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!. |

## 2x scaling ratio VTM-6.1_CE1 with VTM-6.1_RPR_affine_patch.diff

| PSNR1 | PSNR2 |
| --- | --- |
| Low delay B Main10. Over VTM-6.1_patch. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -2.80%; -1.11%; -1.59%; 97%; 103%. Class C; -6.09%; -3.48%; -3.39%; 100%; 113%. Class E; -8.35%; -3.01%; -3.14%; 97%; 99%. Overall; -5.28%; -2.38%; -2.58%; 98%; 105%. Class D; -5.50%; -3.32%; -3.67%; 92%; 96%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!. | Low delay B Main10. Over. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -1.73%; -0.87%; -1.22%; 97%; 103%. Class C; -3.53%; -2.05%; -2.05%; 100%; 113%. Class E; -4.10%; -1.20%; -2.11%; 97%; 99%. Overall; -2.93%; -1.35%; -1.72%; 98%; 105%. Class D; -2.89%; -2.48%; -2.81%; 92%; 96%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!. |

# References

- D. Luo, V. Seregin, W. Wan, JVET-O2021 Description of Core Experiment 1 (CE1): Reference picture resampling filters Gothenburg 2019

- F. Bossen. J.Chen, S. Liu. Versatile Video Coding (Draft 5). JVET-O2001. Gothenburg 2019

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).