| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0391-v3 |
| --- | --- |

| Title: | Non-CE6: A clean-up of SPS maximum SBT size signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In VVC6, when sps_sbt_enabled_flag = 1, sps_sbt_max_size_64_flag is signalled regardless of the value of maximum transform block size specified by sps_max_luma_transform_size_64_flag. This contribution proposes to signal sps_sbt_max_size_64_flag only if the maximum transform block size is 64. If the maximum transform block size is 32, sps_sbt_max_size_64_flag is inferred to be 0. As an alternative approach, this contribution also proposes to remove the maximum SBT size signaling and maximum SBT size is inferred to be equal to maximum transform block size.

In v3, the simulation results of the method 2 is added.

# Problem statement

In VVC draft 6, sub-block transform (SBT) is used for an inter-predicted coding unit (CU). In this transform mode, only a sub-part of the residual block is coded for the CU. An SPS level syntax, sps_sbt_enabled_flag, is used to specify whether SBT is enabled or disabled. If sps_sbt_enabled_flag = 1, another SPS syntax sps_sbt_max_size_64_flag specifies the maximum CU width and height for which SBT is allowed. sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing SBT is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing SBT is 64 luma samples. The variable MaxSbtSize that specifies the max allowed CU size for SBT is computed as follows

MaxSbtSize = Min( MaxTbSizeY, sps_sbt_max_size_64_flag ? 64 : 32 )

Where MaxTbSizeY is the maximum allowed transform block (TB) size and can be derived from another SPS level syntax, sps_max_luma_transform_size_64_flag.

MaxTbSizeY = sps_max_luma_transform_size_64_flag  ?  64  :  32

From the above equations, if the value of sps_max_luma_transform_size_64_flag  = 0, MaxSbtSize is always 32, regardless of the value of sps_sbt_max_size_64_flag. Therefore, it is unnecessary to signal sps_sbt_max_size_64_flag when sps_max_luma_transform_size_64_flag is zero. Such syntax redundancy is unnecessary and confusing.

# Proposed method

## Method 1: conditionally signal maximum SBT size

The proposed method signals sps_sbt_max_size_64_flag only when both sps_max_luma_transform_size_64_flag   and sps_sbt_enabled_flag are 1. Table 1 shows the part of SPS syntax table of the proposed method.  As shown in Table 1, sps_sbt_max_size_64_flag is signaled only if both sps_max_luma_transform_size_64_flag  and sps_sbt_enabled_flag are 1. If sps_max_luma_transform_size_64_flag is 0, sps_sbt_max_size_64_flag is inferred to be zero, that is, the maximum CU width and height that allows SBT is 32 (in units of luma samples).

Table 1: part of the SPS syntax table of the proposed method 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_luma_transform_size_64_flag | u(1) |
| . |  |
| . |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag && sps_max_luma_transform_size_64_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |

## Method 2: Remove maximum SBT size signaling

As an alternative method, it is also proposed to remove sps_sbt_max_size_64_flag  syntax. In that case, maximum allowed CU width and height of SBT directly depends on the sps_max_luma_transform_size_64_flag. If sps_max_luma_transform_size_64_flag is equal to 0, the maximum CU width and height for which SBT is allowed is 32 luma samples. If sps_max_luma_transform_size_64_flag is equal to 1, the maximum CU width and height for which SBT is allowed is 64 luma samples. In other words, MaxSbtSize is set equal to MaxTbSizeY. Table 2 shows the part of the SPS syntax of this proposed method. Table 3 shows the part of coding unit (CU) syntax table of the proposed method. Here MaxTbSizeY is computed as follows, which is the same as in VVC draft 6:

MaxTbSizeY = sps_max_luma_transform_size_64_flag  ?  64  :  32

Table 2: part of SPS syntax table of method 2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_max_luma_transform_size_64_flag | u(1) |
| . |  |
| . |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |

Table 3: part of CU syntax table of method 2

| if( cu_cbf ) { |  |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize MaxTbSizeY && cbHeight <= MaxSbtSize MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| } |  |

# Simulation results

The proposed method (method 2) is implemented on top of VTM6.0 software. The test is performed under CTC condition. In CTC condition, sps_max_luma_transform_size_64_flag is equal to 1. In VTM6.0 anchor software, if the width of the source image is equal to or larger than 1920, the maximum SBT size is set to 64; otherwise the maximum SBT size is set to 32. In the proposed method (method 2), signaling of the sps_sbt_max_size_64_flag is removed and maximum SBT size is same as maximum TB size.

In CTC test condition, the performance differences between the proposed method 2 and VTM6.0 anchor only appeared if the image width is less than 1920. Therefore, only class C, D and E sequences are tested.Table 4 shows the simulation results of the proposed method.

Table 4: simulation results of the proposed method 2 (results will be updated)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class C | -0.04% | 0.02% | 0.07% | 103% | 100% |
| Class E |  |  |  |  |  |
| Class D | -0.08% | -0.08% | -0.09% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.04% | 0.38% | 103% | 107% |
| Class F |  |  |  |  |  |
| Class SCC |  |  |  |  |  |

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).