| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0398-v1 |
| --- | --- |

| Title: | Non-CE3: MIP cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Tobias Hinz, Philipp Helle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document, several cleanups for MIP are proposed. First, it is proposed that all MIP modes, i.e. also the 0th MIP mode, can be used in a transposed way where the parity of the mode determines the transposed condition. Second, it is proposed to reduce the number of MIP modes for MipSizeId=0 from 35 to 32 and to reduce the number of MIP modes for MipSizeId =1 from 19 to 16. Thus, it is proposed to code the MIP-mode with a fixed-length code for small blocks. Third, it is proposed to remove the restriction that MIP may not be used on blocks whose aspect ratio is greater or equal to four. Additionally, it is proposed to introduce a separate context for the first bin of tu_mts_idx depending on the MIP flag. 
It is reported that the proposed method yields -0.06% BD-rate change over the VTM-6.0 in the AI configuration and -0.04% BD-rate savings over the VTM-6.0 in the RA configuration if the proposed context is not added. Here, the encoder runtime is reported to be 101% for the AI configuration and to be 100% for the RA configuration. The decoder runtime is reported to be 100% for the AI configuration and to be 99% for the RA configuration. If furthermore the proposed context is added, it is reported that the proposed method yields -0.09% BD-rate change over the VTM-6.0 in the AI configuration and -0.05% BD-rate change over the VTM-6.0 in the RA configuration. Here, the encoder runtime is reported to be 100% for the AI configuration and to be 100% for the RA configuration. The decoder runtime is reported to be 101% for the AI configuration and to be 99% for the RA configuration.

# Description of the method

In the current VVC working draft [2], for each block on which MIP may be used, the number of MIP modes is an odd integer N=2n+1. Here, the 0-th mode may not be transposed while all other modes may be transposed: For i>n, MIP mode i is the transposed of MIP mode i-n. It is asserted that the restriction that the 0-th mode may not be transposed had originally been made due to the fact that in the first adopted version of MIP from the 15-th JVET meeting in Geneva, the MIP modes were signalled by an MPM (most-probable mode) scheme with 3 MPMs, [1]. For this scheme, the total number of MIP modes needed to be an odd integer of the form N=2k +3, where k is an integer. However, at the 16-th JVET meeting in Gothenburg, the latter signalization scheme for the MIP modes was replaced by a truncated binary code, [2]. It is asserted that as a consequence, the number of MIP modes does not need to be odd anymore and that thus the special treatment of the 0-th MIP mode regarding transposed can be removed. 
Thus, it is proposed that in Clause 8.4.5.2.1 of the current VVC-draft, for an MIP-mode predModeIntra, the transposed flag isTransposed which determines whether the mode is transposed is defined as

isTransposed=predModeIntra&1.

Consequently, it is proposed that in Clause 8.4.5.2.1, the index modeId which determines which matrix is to be used for the given MIP mode predModeIntra is defined as

modeId=predModeIntra>>1. 
It is asserted that defining the transposed condition via the parity of the MIP-mode also ensures that the signalling of a MIP mode requires as many bits as the signalling of its transposed mode.  	
Furthermore, is it proposed to reduce the number of MIP modes from 35 to 32 for MipSizeId=0 and from 19 to 16 for MipSizeId=1. For MipSizeId=2, it is proposed to use 12 instead of 11 MIP modes, since it is proposed that also the 0-th MIP mode can be transposed. Consequently, in the proposed method, 16 instead of 18 MIP matrices are needed for MipSizeId=0 and 8 instead of 10 MIP matrices are needed for MipSizeId=1. For MipSizeId=2, as before, 6 MIP matrices are needed.

Next, in the current VVC working draft, MIP is excluded for blocks whose aspect ratio is greater or equal to four. It is asserted that this restriction is unnecessary since the MIP of the current VVC works for these blocks without any additional design changes. It is asserted that the restriction for MIP regarding the block shape was mainly made to save encoder runtime and that it is thus undesirable to be normative. Thus, it is proposed to remove the latter restriction on the usage of MIP and to allow MIP for all block shapes.

Additionally, it is proposed to use a separate context for the first bin of tu_mts_idx if the corresponding coding unit is using MIP. It is asserted that there is a statistical difference between the usage of MTS for MIP and for non-MIP blocks which can thus be exploited by separating the corresponding context.

Note, that the draft specification in the attached spec text also contains editorial cleanups regarding the MaxTbSizeY restriction and the MipSizeId derivation.

# Experimental results

In this section, experimental results are reported according to the common test conditions [3] and the test conditions of the CE for intra, [4]. In Table 1 and Table 2, results are reported for the proposed cleanup but without the addition of the extra context for the MTS index. In Table 3 and Table 4, results are reported for the proposed cleanup with the proposed addition of the extra context for the MTS index. No optimizations at the encoder were made in comparison to the VTM-6.0 anchor in any of the tests.

Table 1. Result of the incorporated MIP changes: Reference is VTM-6.0 anchor, test is VTM-6.0 with the proposed cleanup and without the addition of the extra context, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,08% | -0,03% | -0,03% | 101% | 101% |
| Class A2 | -0,06% | -0,04% | -0,06% | 101% | 101% |
| Class B | -0,07% | -0,08% | -0,12% | 100% | 101% |
| Class C | -0,03% | -0,18% | -0,01% | 101% | 97% |
| Class E | -0,09% | 0,08% | 0,02% | 100% | 99% |
| Overall | -0,06% | -0,06% | -0,05% | 101% | 100% |
| Class D | -0,02% | -0,04% | 0,07% | 101% | 99% |
| Class F | -0,03% | -0,05% | -0,01% | 100% | 100% |

Table 2. Result of the incorporated MIP changes: Reference is VTM-6.0 anchor, test is VTM-6.0 with the proposed cleanup and without the addition of the extra context, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,03% | -0,11% | -0,12% | 100% | 100% |
| Class A2 | -0,02% | 0,12% | -0,07% | 101% | 97% |
| Class B | -0,05% | 0,03% | -0,12% | 100% | 99% |
| Class C | -0,03% | 0,08% | -0,01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,04% | 0,03% | -0,08% | 100% | 99% |
| Class D | -0,04% | -0,12% | -0,12% | 100% | 100% |
| Class F | -0,01% | -0,05% | 0,00% | 101% | 101% |

Table 3. Result of the incorporated MIP changes: Reference is VTM-6.0 anchor, test is VTM-6.0 with the proposed cleanup and with the addition of the extra context, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,12% | -0,16% | -0,10% | 99% | 100% |
| Class A2 | -0,08% | -0,10% | -0,08% | 100% | 101% |
| Class B | -0,08% | -0,07% | -0,12% | 100% | 103% |
| Class C | -0,04% | -0,18% | -0,07% | 100% | 97% |
| Class E | -0,13% | -0,07% | -0,05% | 101% | 101% |
| Overall | -0,09% | -0,11% | -0,08% | 100% | 101% |
| Class D | -0,02% | 0,05% | -0,09% | 100% | 97% |
| Class F | -0,03% | -0,10% | -0,12% | 100% | 99% |

Table 4. Result of the incorporated MIP changes: Reference is VTM-6.0 anchor, test is VTM-6.0 with the proposed cleanup and with the addition of the extra context, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,05% | -0,05% | -0,08% | 100% | 99% |
| Class A2 | -0,03% | -0,02% | 0,01% | 100% | 99% |
| Class B | -0,07% | 0,01% | -0,04% | 100% | 100% |
| Class C | -0,03% | 0,07% | -0,08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,05% | 0,01% | -0,05% | 100% | 99% |
| Class D | -0,05% | -0,11% | 0,00% | 100% | 98% |
| Class F | -0,06% | 0,06% | -0,03% | 99% | 99% |

# Conclusion

In this contribution, an alignment of the transposed usage of the MIP modes and of the number of MIP modes with the newly adopted signalling of the MIP mode is proposed. Moreover, a cleanup regarding an unnecessary restriction for the usage of MIP is proposed. Finally, a separate context for the MTS index depending on the MIP flag is proposed. It is proposed to adopt all these changes into the next working draft of the VVC.

# References

- B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, Geneva, Switzerland, March 2019

- B. Bross et al., “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, Sweden, July 2019

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

- G. Van der Auwera et al.,“Description of Core Experiment 3: Intra Prediction and Mode Coding”,  JVET-O2023, Gothenburg, Sweden, July 2019

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).