| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0399-v1 |
| --- | --- |

| Title: | Non-CE8: On escape sample coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document proposes to signal escape samples using QP dependent fixed-length binarization method. Three aspects are modified: 1) limit the lowest QP for escape samples; 2) fixed-length coding is applied; 3) reconstruction by left shift only. Accordingly, the dequantization process for escape samples is also modified which only has a QP dependent shifting operation. The proposed method is implemented and evaluate with VTM-6.0 following the CE8 test conditions. Results are summarized as below.

CTC QPs = {22, 27, 32, 37}

Dual tree on:

TGM1080P: AI: -0.07%, 102%, 101%; RA: - 0.14%, 102%, 102%; LDB: -0.08%, 101%, 100%

Dual tree off:

TGM1080P: AI: -0.22%, 100%, 102%; RA: 0.30%, 100%, 101%; LDB: -0.10%, 100%, 100%.

Low QPs = {2, 7, 12, 17}

Dual tree on:

TGM1080P: AI: -2.04%, 102%, 101%; RA: -1.62%, 102%, 98%; LDB: -1.27%, 103%, 102%

Dual tree off:

TGM1080P: AI: -3.33%, 101%, 99%; RA-2.89%, 100%, 100%; LDB: -1.79%, 101%, 101%.

# Introduction

Escape samples are employed to the handle the outlier case in the palette mode.

The binarization of escape sample is EG3 in the current VTM. However, for a uniform distribution signal, the fixed length binarization could be better than EG3 with both distortion and bitrate measurements.

# Proposed Method

This document proposes to use the fixed length binarization for escape samples and the quantization and dequantization process are also modified accordingly.

The maximal bit depth of an escape sample in the proposed method depends on the quantization parameter and derived as follows.

max (1, bitDepth – (max (QpPrimeTsMin, Qp) - 4)/6)                           (1)

where the bitDepth is the internal bit depth, Qp is the current quantization parameter (QP), QpPrimeTsMin is the minimal QP for transform skip blocks and max is an operation to get a larger value between two inputs.

Also, only a shifting operation is needed in the dequantization process for an escape sample. Let escapeVal be the decoded escape value and recon be the reconstructed value of an escape sample. The value is derived as follows.

shift = min (bitDepth – 1, (max (QpPrimeTsMin, Qp) - 4)/6)                (2)

recon = (escapeVal << shift)                                       (3)

It is to guarantee that the distortion of the reconstructed value is always smaller or equal to that of the current design, i.e. ((escapeVal*levelScale [qP%6]) <<(qP/6)+32)>>6.

At the encoder, the quantization is implemented as follows:

escapeVal = (p + (1 << (shift – 1))) >> shift				(4)

escapeVal = clip3(0, (1 << bitDepth) – 1, escapeVal)	                	(5)

where p is the original value of a escape sample.

Compared with the current design using EG3 and quantization with a dequantization table, one addition, one multiplication and two shifting operations, the proposed method is much simpler, which needs just one shifting operation.

# Simulation Results

The proposed method is evaluated with VTM-6.0. The simulation follows the CE 8 test condition [1]. The detailed results are given in Table 1-6. More detailed results could be found in the attached excel sheets.

Table 1.  Coding performance of the proposed scheme under AI configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.07% | -0.08% | -0.02% | 102% | 101% |
| TGM 720p | -0.05% | -0.17% | 0.00% | 101% | 101% |
| Animation | -0.01% | 0.02% | 0.04% | 101% | 100% |
| Mixed content | 0.05% | -0.10% | -0.08% | 102% | 101% |
| Camera-Captured content | -0.01% | 0.03% | 0.00% | 102% | 102% |
| Overall | -0.03% | -0.07% | -0.01% | 101% | 101% |

Table 2. Coding performance of the proposed scheme under RA configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.14% | -0.15% | -0.10% | 102% | 102% |
| TGM 720p | 0.02% | -0.16% | -0.26% | 103% | 105% |
| Animation | 0.13% | 0.07% | 0.05% | 102% | 102% |
| Mixed content | 0.05% | -0.02% | 0.07% | 102% | 101% |
| Camera-Captured content | -0.02% | 0.02% | -0.02% | 100% | 101% |
| Overall | 0.00% | -0.06% | -0.07% | 102% | 102% |

Table 3. Coding performance of the proposed scheme under LDB configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.08% | -0.15% | -0.22% | 101% | 100% |
| TGM 720p | 0.20% | -0.22% | 0.49% | 99% | 98% |
| Animation | 0.03% | 0.00% | -0.10% | 100% | 100% |
| Mixed content | -0.19% | -0.02% | 0.06% | 102% | 101% |
| Camera-Captured content | 0.02% | -0.04% | -0.14% | 101% | 100% |
| Overall | 0.00% | -0.10% | 0.04% | 100% | 100% |

Table 4. Coding performance of the proposed scheme under AI configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.22% | -0.32% | -0.36% | 100% | 102% |
| TGM 720p | 0.06% | 0.01% | -0.17% | 100% | 100% |
| Animation | -0.02% | -0.06% | -0.07% | 99% | 97% |
| Mixed content | -0.13% | -0.27% | -0.27% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Overall | -0.07% | -0.14% | -0.20% | 100% | 100% |

Table 5. Coding performance of the proposed scheme under RA configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.30% | -0.30% | -0.28% | 100% | 101% |
| TGM 720p | -0.05% | -0.03% | 0.09% | 101% | 102% |
| Animation | 0.01% | -0.02% | -0.10% | 100% | 100% |
| Mixed content | -0.17% | -0.09% | -0.23% | 101% | 101% |
| Camera-Captured content | 0.00% | -0.01% | 0.00% | 99% | 102% |
| Overall | -0.11% | -0.10% | -0.11% | 101% | 101% |

Table 6. Coding performance of the proposed scheme under LDB configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.10% | -0.10% | -0.23% | 100% | 100% |
| TGM 720p | 0.10% | -0.23% | 0.14% | 101% | 99% |
| Animation | 0.00% | 0.04% | 0.03% | 101% | 96% |
| Mixed content | 0.25% | 0.27% | -0.06% | 102% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.05% | -0.03% | -0.03% | 101% | 99% |

Table 7. Coding performance of the proposed scheme under AI configuration with dual tree on under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.04% | -3.55% | -3.55% | 102% | 101% |
| TGM 720p | -0.64% | -1.14% | -1.17% | 102% | 99% |
| Animation | -0.27% | -0.47% | -0.56% | 103% | x102% |
| Mixed content | -1.72% | -2.85% | -2.88% | 101% | 101% |
| Camera-Captured content | 0.00% | -0.02% | -0.01% | 100% | 102% |
| Overall | -1.04% | -1.80% | -1.83% | 102% | 101% |

Table 8. Coding performance of the proposed scheme under RA configuration with dual tree on under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.62% | -2.45% | -2.45% | 102% | 98% |
| TGM 720p | -0.61% | -0.99% | -1.19% | 100% | 96% |
| Animation | -0.17% | -0.51% | -0.43% | 101% | 101% |
| Mixed content | -1.40% | -0.80% | -0.87% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.01% | -0.01% | 101% | 101% |
| Overall | -0.85% | -1.10% | -1.15% | 101% | 99% |

Table 9. The performance of the proposed scheme versus VTM 6 under LDB with dual tree on under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.27% | -1.73% | -1.74% | 103% | 102% |
| TGM 720p | -0.06% | -0.69% | -0.58% | 102% | 99% |
| Animation | -0.62% | -0.47% | -0.48% | 103% | 103% |
| Mixed content | -0.50% | -1.05% | -0.97% | 103% | 102% |
| Camera-Captured content | 0.01% | 0.00% | 0.02% | 102% | 102% |
| Overall | -0.54% | -0.89% | -0.85% | 102% | 101% |

Table 10. The performance of the proposed scheme versus VTM 6 under AI with dual tree off under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -3.33% | -4.60% | -4.66% | 101% | 99% |
| TGM 720p | -0.99% | -1.54% | -1.56% | 101% | 103% |
| Animation | -0.43% | -0.93% | -0.91% | 101% | 101% |
| Mixed content | -1.47% | -2.00% | -2.13% | 102% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | -1.44% | -2.09% | -2.13% | 101% | 101% |

Table 11. The performance of the proposed scheme versus VTM 6 under RA with dual tree off under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.89% | -3.06% | -3.08% | 100% | 100% |
| TGM 720p | -1.05% | -1.24% | -1.55% | 100% | 103% |
| Animation | -0.33% | -0.33% | -0.38% | 100% | 100% |
| Mixed content | -1.36% | -1.72% | -1.87% | 101% | 102% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 101% | 101% |
| Overall | -1.30% | -1.46% | -1.58% | 100% | 101% |

Table 12. The performance of the proposed scheme versus VTM 6 under LDB with dual tree off under low QPs

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -1.79% | -2.30% | -2.23% | 101% | 101% |
| TGM 720p | -0.34% | -0.65% | -0.89% | 100% | 101% |
| Animation | -0.31% | -0.57% | -0.66% | 101% | 100% |
| Mixed content | -0.66% | -1.70% | -1.88% | 101% | 100% |
| Camera-Captured content | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Overall | -0.71% | -1.16% | -1.25% | 101% | 100% |

# Text changes

The text changes below are on top of JVET-O2001-vE[2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

Table 9-77 – Syntax elements and associated binarizations

| Syntax element |  |  |
| --- | --- | --- |
|  | Process | Input parameters |
| palette_escape_val | EG3 FL | max(1, bitDepth - (Max(QpPrimeTsMin, Qp′Y ) – 4)/6) |

#### 8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location (xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max(0, Qp′Cr )		(8-274)

- The variables bitDepth is derived as follows:

bitDepth = (cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 )  >>  6	(8-276)

shift = min(bitDepth - 1, (max(QpPrimeTsMin, qP) - 4)/6)

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ]  << shift )

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )	(8-277)

# Conclusion

It is proposed to employ a QP dependent fixed length binarization and one shifting for the reconstruction for escape samples. We recommend adopting the proposed method to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).