| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0407 |
| --- | --- |

| Title: | AHG15: cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv syntax dependency removal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ru-Ling Liao, Jiancong Luo, Yan Ye Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In VVC draft 6, cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv are signaled in PPS, while the range of these two syntax elements depends both on SPS and slice level information, which causes a syntax dependency issue. This contribution proposes to remove the syntax dependency by moving cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv to slice header.

Problem statement

In VVC draft 6, cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv are signaled in PPS as shown in Table 1 and the semantics are given below.

Table 1 Current PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| … |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| …. |  |

cu_qp_delta_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_qp_delta_abs and cu_qp_delta_sign_flag. The value range of cu_qp_delta_subdiv is specified as follows:

- If slice_type is equal to I, the value of cu_qp_delta_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_qp_delta_subdiv shall be in the range of 0 to 2* ( log2_ctu_size_minus2 − log2_min_qt_size_inter_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of cu_qp_delta_subdiv is inferred to be equal to 0.

cu_chroma_qp_offset_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_chroma_qp_offset_flag. The value range of cu_chroma_qp_offset_subdiv is specified as follows:

- If slice_type is equal to I, the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2* ( log2_ctu_size_minus2 − log2_min_qt_size_inter_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of cu_chroma_qp_offset_subdiv is inferred to be equal to 0.

As according to the VVC draft 6, the range of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv depdend on the following

- Slice type

- log2_ctu_size_minus2 (it is changed to log2_ctu_size_minu5 in VVC draft 6) which is signaled in SPS

- log2_min_qt_size_intra_slice_minus2 and log2_min_qt_size_inter_slice_minus2 which are not signalled in the bitstream (instead, sps_log2_diff_min_qt_min_cb_intra_slice_luma and sps_log2_diff_min_qt_min_cb_inter_slice are signaled in VVC draft 6)

- MaxMttDpethY which is derived in slice level based on slice_max_mtt_hierarchy_depth_luma. (If slice_max_mtt_hierarchy_depth_luma is not present, the value of slice_max_mtt_hierarchy_depth_luma is inferred to be either sps_max_mtt_hierarchy_depth_intra_slice_luma or sps_max_mtt_hierarchy_depth_inter_slice_luma, based on the slice type.)

Among the above four factors, 2) is signaled in SPS level, 1) is decided in slice level, 3) and 4) are derived from both SPS and slice level information. Thus, a syntax dependency issue exits in the VVC draft 6. It is noted that the VVC draft 6 includes editor notes regarding this syntax dependency issue.

Proposed methods

In order to solve the syntax dependency issue, it is proposed to simply move cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv to slice header as shown in Table 2 and the semantics are given below the table. The proposed changes are highlighted in yellow

Table 2 proposed syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| .. |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| … |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |

cu_qp_delta_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_qp_delta_abs and cu_qp_delta_sign_flag. The value range of cu_qp_delta_subdiv is specified as follows:

- If slice_type is equal to I, the value of cu_qp_delta_subdiv shall be in the range of 0 to 2 * ( CtbLog2SizeY −  MinQtLog2SizeIntraY + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_qp_delta_subdiv shall be in the range of 0 to 2* (  CtbLog2SizeY − MinQtLog2SizeInterY + MaxMttDepthY ), inclusive.

When not present, the value of cu_qp_delta_subdiv is inferred to be equal to 0.

cu_chroma_qp_offset_subdiv specifies the maximum cbSubdiv value of coding units that convey cu_chroma_qp_offset_flag. The value range of cu_chroma_qp_offset_subdiv is specified as follows:

- If slice_type is equal to I, the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2 * ( CtbLog2SizeY − MinQtLog2SizeIntraY + MaxMttDepthY ), inclusive.

- Otherwise (slice_type is not equal to I), the value of cu_chroma_qp_offset_subdiv shall be in the range of 0 to 2* ( CtbLog2SizeY − MinQtLog2SizeInterY + MaxMttDepthY ), inclusive.

When not present, the value of cu_chroma_qp_offset_subdiv is inferred to be equal to 0.

In the above, the range itself of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv is not changed. The different range calculation equation is only to replace the non-existing syntax elements log2_min_qt_size_intra_slice_minus2 and log2_min_qt_size_inter_slice_minus2 with the variables already defined in VVC draft to correctly reflect the design intention. Only syntax position of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv is proposed to be changed. All other parsing process and decoding process related to cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv are kept the same as before.

With the proposed method, the syntax element dependency issue of cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv is removed and the typos regarding missing syntax elements are corrected.

Conclusion

In this contribution, it is proposed to move cu_qp_delta_subdiv and cu_chroma_qp_offset_subdiv from PPS to slice header to solve the syntax dependency issue. It is suggested to adopt this contribution to next version of VVC draft.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).