| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0408 |
| --- | --- |

| Title: | CE4-related: On PROF on/off control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ru-Ling Liao, Jiancong Luo, Yan Ye Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

Prediction refinement with optical flow (PROF) was adopted to VVC draft 6 with only SPS level controlling flag. This contribution proposes to add a slice level controlling flag for PROF.

Introduction

PROF was adopted to VVC in Gothenburg meeting. In VVC draft 6, DMVR/BDOF have both SPS level controlling flag and slice level control flag. While for PROF, there is only a SPS level controlling flag. To give more flexibility for on/off control of PROF, it is proposed to add a slice level disabling flag for PROF so that each slice can enable/disable PROF individually.

Proposed methods

Considering the fact that BDOF and DMVR share the same slice level controlling flag，it is proposed to use the same flag to control PROF as well so that there is no need to signaling additional syntax element. The proposed syntax changes as Table 1 and Table 2. The proposed changes are highlighted with yellow

Table 1 Proposed SPS syntax changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| … |  |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag ¦¦ sps_affine_prof_enabled_flag) |  |
| sps_bdof_dmvr_affine_prof_slice_present_flag | u(1) |

Table 2 Proposed slice header syntax change

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_bdof_dmvr_slice_present_flag sps_bdof_dmvr_affine_prof_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag slice_disable_bdof_dmvr_affine_prof_flag | u(1) |
| … |  |

sps_bdof_dmvr_affine_prof_slice_present_flag equal to 1 specifies that slice_disable_bdof_dmvr_affine_prof_flag is present in slice headers referring to the SPS. sps_bdof_dmvr_affine_prof_slice_present_flag equal to 0 specifies that slice_disable_bdof_dmvr_affine_prof_flag is not present in slice headers referring to the SPS. When sps_bdof_dmvr_affine_prof_slice_present_flag is not present, the value of sps_bdof_dmvr_affine_prof_slice_present_flag is inferred to be equal to 0.

slice_disable_bdof_dmvr_affine_prof_flag equal to 1 specifies that none of bi-directional optical flow inter prediction, decoder motion vector refinement based inter bi-prediction, and prediction refinement with optical flow is enabled in the current slice. slice_disable_bdof_dmvr_affine_prof_flag equal to 0 specifies that bi-directional optical flow inter prediction or decoder motion vector refinement based inter bi-prediction or prediction refinement with optical flow may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_affine_prof_flag is not present, the value of slice_disable_bdof_dmvr_affine_prof_flag is inferred to be equal to 0.

- 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

The variable cbProfFlagLX is derived as follows:

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

  - sps_affine_prof_enabled_flag is equal to 0.

  - slice_disable_bdof_dmvr_affine_prof_flag is equal to 1.

  - fallbackModeTriggered is equal to 1.

  - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

  - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

- Otherwise, cbProfFlagLX set equal to TRUE.

- 8.5.6.3.1 General

The prediction block border extension size brdExtSize is derived as follows:

brdExtSize = ( bdofFlag | | ( inter_affine_flag[ xSb ][ ySb ] && sps_affine_prof_enabled_flag && ! slice_disable_bdof_dmvr_affine_prof_flag ) ) ? 2 : 0	(8-752)

The variable fRefWidth is set equal to the PicOutputWidthL of the reference picture in luma samples.

The variable fRefHeight is set equal to PicOutputHeightL of the reference picture in luma samples.

The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

- If cIdx is equal to 0, the following applies:

  - The scaling factors and their fixed-point representations are defined as

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntL, ySbIntL ) is set equal to ( xSb + ( mvLX[ 0 ]  >>  4 ), ySb + ( mvLX[ 1 ]  >>  4 ) ).

  - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

refxSbL = ( ( xSb  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-756)

refySbL = ( ( ySb  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-758)

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = refxL  >>  4		(8-759)

yIntL = refyL  >>  4		(8-760)

xFracL = refxL & 15		(8-761)

yFracL = refyL & 15		(8-762)

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE and slice_disable_bdof_dmvr_affine_prof_flag is FALSE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight and ( xSb, ySb ) as inputs.

Conclusion

In this contribution, the slice level controlling flag for RPOF is proposed to give more flexibility of PROF controlling. It is proposed to reuse BDOF and DMVR slice level controlling flag to control PROF as well. So there is no need to additionally signal a flag in slice header.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).