| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0410 |
| --- | --- |

| Title: | AHG15: chroma QP mapping table syntax variant with less bits |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Philippe Bordes, Karam Naser, Edouard François | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes changes in the luma to chroma QP mapping table syntax adopted in VVC draft 6, which consists in sending pivot points of a piecewise linear function. The proposed modifications are: making the first point relative to QP 26 instead of -QpBdOffsetC, transmitting luma to chroma QP differences instead of chroma QPs, and inferring the first difference to be zero. This is said to reduce the number of bits needed in the SPS and to make the syntax independent from bit depth.

# Introduction

During the 15th JVET meeting in Gothenburg, the fixed luma to chroma QP mapping table has been removed from the “Derivation process for quantization parameters”, and a method to transmit custom tables has been adopted, as proposed by JVET-O0650. The syntax and reconstruction process are shown below:

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_out_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

qpInVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_in_val_minus1[ i ][ 0 ]
qpOutVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_out_val[ i ][ 0 ]
for( j = 1; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j ] = qpOutVal[ i ][ j − 1 ] + delta_qp_out_val[ i ][ j ]
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(7-31)
for( j = 0; j < num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 1 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInVal[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( delta_qp_out_val[ i ][j + 1] * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

This syntax can cause a significant increase in SPS bit count, which can be reduced by the modifications proposed by this contribution:

- Making the first pivot point relative to QP 26

- Transmitting differences between luma and chroma QPs instead of chroma QPs

- Inferring the first difference to be zero

# Proposal description

## Make the first point relative to QP 26

As the first pivot point is expected to fall in the 20 – 30 range most of the time, it is proposed to make it relative to QP 26 instead of transmitting an absolute value, which is in fact relative to -QpBdOffsetC, thus probably a large value if we consider 10-bits coding to be the nominal case. This is consistent with init_qp_minus26 in the PPS, and makes the syntax independent from bit depth.

## Transmit luma to chroma QP differences

As chroma QP is expected to mostly follow luma QP, but with chroma generally increasing less than luma, the difference is expected to increase slowly, thus dpcm-coded difference values should be low, and biased towards negative values (if difference is understood as chroma QP – luma QP).

This contribution proposes to code the chroma QPs of pivot points as dpcm-coded luma to chroma QP difference, with a +1 bias, as illustrated by the following equation:

coded_delta_qp_diff(n) = ( QPchroma(n) – QPluma(n) ) – ( QPchroma(n-1) – QPluma(n-1) ) + 1

This makes the value 0 more probable, which reduces bit count.

## Infer the first difference to be zero

The luma/chroma QP difference of the first pivot point is equivalent to a global offset embedded in the custom mapping table. However, there are two chroma offsets available already: in PPS and slice header. A third offset at SPS level is considered redundant, and is new in VVC. It is proposed to remove it by forcing the first difference to be zero.

Consequently, at least two pivot points are needed to differentiate from an identity table; the number of points can then be signalled with a -2 offset, potentially reducing bit count. In other words, the first point (start point) is considered apart from others, and only the number of subsequent points is signalled (with -1 offset).

This may have other benefits:

- If offsets are required at SPS level, specifying them explicitly as in PPS and slice header probably makes the specification text more readable

- If starting QP point is limited to e.g. positive values, the previous chroma QPs do not need to be stored as they are equal to luma QPs, potentially saving decoder memory.

## Draft text

Modifications to VVC draft 6 are detailed in the following subsections.

Note: in the semantics section, 5th line of equation (7-31), the +1 from from delta_qp_in_val_minus1 and -1 from delta_qp_diff_val bias have been combined: they cancel each other.

### SPS syntax

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_outdiff_val[ i ][ j ] | uese(v) |
| } |  |
| } |  |
| … |  |
| } |  |

### SPS semantics

[…]

same_qp_table_for_chroma equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals as well as joint Cb-Cr residuals. same_qp_table_for_chroma equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS. When same_qp_table_for_chroma is not present in the bistream, the value of same_qp_table_for_chroma is inferred to be equal to 1.

qp_table_start_minus26[ i ] plus 26 specifies the starting luma and chroma QP used to describe the i-th chroma QP mapping table. The value of start_qp_minus26[ i ] shall be in the range of -26 - QpBdOffsetC to 36 inclusive. When start_qp_minus26[ i ] is not present in the bitstream, the value of start_qp_minus26[ i ] is inferred to be equal to 0.

num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 62 + QpBdOffsetC, inclusive. When num_points_in_qp_table_minus1[ 0 ] is not present in the bitstream, the value of num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_in_val_minus1[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_diff_val[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_diff_val[ 0 ][ j ] is inferred to be equal to 0.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..same_qp_table_for_chroma ? 0 : 2 is derived as follows:

qpInVal[ i ][ 0 ] = −QpBdOffsetC + qp_table_start_minus26[ i ] + 26
qpOutVal[ i ][ 0 ] = −QpBdOffsetCqpInVal[ i ][ 0 ] + delta_qp_diff_val[ i ][ 0 ]
for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j + 1 ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j + 1 ] = qpOutVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + delta_qp_diff_val[ i ][ j ]
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(7-31)
for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 1 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInVal[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][ j + 1 ] − qpOutVal[ i ][ j ] ) * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] + 1 ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When same_qp_table_for_chroma is equal to 1, ChromaQpTable[ 1 ][ k ] and ChromaQpTable[ 2 ][ k ] are set equal to ChromaQpTable[ 0 ][ k ] for k = −QpBdOffsetC..63.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffsetC to 63, inclusive for i = 0..same_qp_table_for_chroma ? 0 : 2 and j = 0..num_points_in_qp_table_minus1[ i ] + 1.

[…]

# Bit counts for example QP mapping tables

| Table | Pivot points | VVC draft 6 (10-bit) | Proposed |
| --- | --- | --- | --- |
| CTC: All-INTRA | (1, 1), (31, 32), (43, 41) | 51 | 40 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 39 | 20 |
| CTC: Class H | (9, 9), (23, 24), (33, 33), (42, 37) | 65 | 48 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 47 | 24 |
| JVET-O0433 (HLG) | (24, 24), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 77 | 52 |

# Annex: alternate syntax with explicit first difference

If keeping an embedded offset in the first pivot point is preferred, the following alternative syntax may be adopted.

### Alternate SPS syntax

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| delta_qp_in_val_minus1[ i ][ 0 ] | se(v) |
| delta_qp_diff_val[ i ][ 0 ] | se(v) |
| for( j = 1; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

### Alternate SPS semantics

[…]

same_qp_table_for_chroma equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals as well as joint Cb-Cr residuals. same_qp_table_for_chroma equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS. When same_qp_table_for_chroma is not present in the bistream, the value of same_qp_table_for_chroma is inferred to be equal to 1.

num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 63 + QpBdOffsetC, inclusive. When num_points_in_qp_table_minus1[ 0 ] is not present in the bitstream, the value of num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_in_val_minus1[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_diff_val[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_diff_val[ 0 ][ j ] is inferred to be equal to 0.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..same_qp_table_for_chroma ? 0 : 2 is derived as follows:

qpInVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_in_val_minus1[ i ][ 0 ] + 26
qpOutVal[ i ][ 0 ] = −QpBdOffsetCqpInVal[ i ][ 0 ] + delta_qp_diff_val[ i ][ 0 ]
for( j = 1; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j ] = qpOutVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + delta_qp_diff_val[ i ][ j ]
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(7-31)
for( j = 0; j < num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 1 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInVal[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][ j + 1 ] − qpOutVal[ i ][ j ] ) * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When same_qp_table_for_chroma is equal to 1, ChromaQpTable[ 1 ][ k ] and ChromaQpTable[ 2 ][ k ] are set equal to ChromaQpTable[ 0 ][ k ] for k = −QpBdOffsetC..63.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffsetC to 63, inclusive for i = 0..same_qp_table_for_chroma ? 0 : 2 and j = 0..num_points_in_qp_table_minus1[ i ].

[…]

### Alternate bit counts for example mapping tables

| Table | Pivot points | VVC draft 6 (10-bit) | Proposed |
| --- | --- | --- | --- |
| CTC: All-INTRA | (1, 1), (31, 32), (43, 41) | 51 | 41 |
| CTC: LDP, LDB, RA | (32, 32), (44, 41) | 39 | 23 |
| CTC: Class H | (9, 9), (23, 24), (33, 33), (42, 37) | 65 | 51 |
| VTM5 | (26, 26), (33, 32), (43, 37) | 47 | 25 |
| JVET-O0433 (HLG) | (24, 24), (34, 33), (45, 37), (60, 49), (61, 50), (63, 51) | 77 | 53 |

# References

[JVET-O2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Gothenburg, SE, July 2019

[JVET-O0650]	A. K. Ramasubramonian, G. V. der Auwera, D. Rusanovskyy, and M. Karczewicz, “AHG15: On signalling of chroma QP tables”, JVET-O0650, Gothenburg, SE, July 2019.

[JVET-O1166]	F. Bossen, A. Segall, A. K. Ramasubramonian, G. V. der Auwera, “AHG15: Efficient encoding of chroma QP tables using piecewise linear representations”, JVET-O1166, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).