| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0412-v1 |
| --- | --- |

| Title: | CE5-related: On further modification of Hadamard filter from CE5-3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Alexander Karabutov Sofya Nikolaeva | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com karabutov.alexander@huawei.com nikolaeva.sofya@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports results of modifications of test CE5-3.2 on combination of Hadamard and SAO filter related to encoder in-loop RDO decision fix and bugfix of SAO and Hadamard filter averaging stage. In addition, the effect of applying Hadamard filter to chroma components is reported.

# Introduction

In CE5-3.2.1 Hadamard filter was tested in combination with SAO (JVET-P0078). In this implementation the encoder makes a decision whether to apply Hadamard filter on CTU and slice level by evaluating RDO cost at in-loop filtering stage. The output of SAO and Hadamard filtering is averaged before proceeding to next in-loop filtering stage.

# Filter combination fixes

## Aspect 1

In CE5-3.2.1 at in-loop filtering stage SAO and Hadamard filter has its own RDO to make a decision whether to apply filter at CTU and slice level. However, the result of joint work was not evaluated. In order to compensate that the following modification was made. When cost of SAO filtered frame is above than Hadamard filtered frame then SAO is disabled at the slice level.

## Aspect 2

In implementation tested in CE5-3.2.1 the averaging of SAO and Hadamard output is performed at the frame basis without consideration of whether filter is applied to particular CTU or not. This lead to degradation of joint filtering performance. This bug was eliminated by performing averaging only for those CTUs where both filters are enabled.

In a table below the simulation results of both aspects applied to CE5-3.2.1 are provided.

Test 1. The test results of Aspect 1 and Aspect 2 applied to CE5-3.2.1.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.54% | 0.34% | 0.55% | 108% | 105% |
| Class E | -0.47% | 0.25% | 0.25% | 108% | 105% |
| Overall |  |  |  |  |  |
| Class D | -0.47% | 0.50% | 0.38% | 107% | 104% |
| Class F |  |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.47% | 0.28% | 0.50% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.59% | 0.07% | 0.12% | 103% | 102% |
| Class F |  |  |  |  |  |

Test 2. Test results of CE5-3.2.1 (for reference).

| . | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | 0.52% | 0.49% | 108% | 110% |
| Class A2 | -0.38% | 0.40% | 0.31% | 107% | 109% |
| Class B | -0.26% | 0.50% | 0.42% | 107% | 108% |
| Class C | -0.42% | 0.21% | 0.35% | 107% | 107% |
| Class E | -0.33% | 0.34% | 0.29% | 107% | 108% |
| Overall | -0.33% | 0.39% | 0.37% | 107% | 108% |
| Class D | -0.36% | 0.48% | 0.32% | 107% | 106% |
| Class F | -0.17% | 0.17% | 0.06% | 105% | 107% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.25% | 0.17% | 103% | 105% |
| Class A2 | -0.60% | 0.16% | 0.06% | 104% | 104% |
| Class B | -0.34% | 0.38% | 0.49% | 105% | 103% |
| Class C | -0.38% | 0.33% | 0.36% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | 0.30% | 0.31% | 104% | 103% |
| Class D | -0.50% | 0.06% | 0.18% | 103% | 102% |
| Class F | -0.09% | 0.03% | 0.15% | 103% | 103% |

# Effect of chroma filtering

In CE5-3.2 Hadamard filter is applied to luma component only. The simulation results demonstrated some drop for chroma components. In order to compensate that the filter is applied to chroma components as well. The following results were obtained.

Test 3. SAO is off (corresponding to CE5-3.2.2)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.55% | -0.34% | -0.15% | 109% | 109% |
| Class E | -0.51% | -0.24% | 0.07% | 109% | 109% |
| Overall |  |  |  |  |  |
| Class D | -0.50% | -0.22% | -0.15% | 109% | 108% |
| Class F |  |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0.noSAO |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.46% | -0.06% | 0.42% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.48% | -0.05% | 0.28% | 104% | 100% |
| Class F | -0.44% | -0.38% | 0.12% | 104% | 101% |

Test 4. SAO and ALF are off (corresponding to CE5-3.2.2 NO ALF)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -1.03% | -0.94% | -1.01% | 109% | 110% |
| Class E | -1.21% | -0.85% | -1.08% | 109% | 112% |
| Overall |  |  |  |  |  |
| Class D | -0.83% | -0.80% | -1.03% | 109% | 107% |
| Class F |  |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0.noSAO.noALF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.99% | 0.07% | 0.77% | 105% | 102% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.66% | 0.24% | 0.58% | 105% | 102% |
| Class F | -1.05% | -0.49% | -0.35% | 104% | 102% |

# Conclusion

Applying fixes to in-loop RDO decision and to SAO and Hadamard filter averaging stage provides additionally ~0.1% of coding gain in comparison to test CE5-3.2.1. Additionally, enabling filtering for chroma components allows further increase filter performance.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).