| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0413 |
| --- | --- |

| Title: | CE4-related: Clipping for PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Proposed specification text change for simplification test 2

#### Derivation process for motion vector arrays from affine control point motion vectors

When cbProfFlagLX is 1, the motion vector difference array diffMv is derived as follows:

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1  <<  Max( 65, BitDepthY − 67 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

  - For i = 0..1, the following applieis:

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 7(13 – Max( 5, BitDepthY – 7 )), and leftShift set equal to 0 as inputs and the rounded diffMv[ x ][ y ][ i ] as output.

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

#### Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

Variable dILimit is set equal to 1 << (max(14, BitDepthY +2) - 1).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3 ( -dILimit, dILimit – 1, (( dI + 1 ) >> 1) dI )	(8-799)