| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0416 |
| --- | --- |

| Title: | Non-CE4: Using integer MV in CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In the current VVC SPEC [4], CIIP is adopted to improve the performance of the merge mode by combining inter and intra planar prediction with an adaptive weight. The inter prediction signal in the CIIP mode is derived using the same inter prediction process applied to regular merge mode. This contribution proposes to simplify CIIP by using integer MV in the inter prediction process, such that the expensive interpolation process can be skipped. In the first test, integer MV is used in bi-prediction process of CIIP.

The Y BD-rate changes for the first test are 0.03% with the RA configuration, and -0.02% with LDB configuration.

# Introduction

In VTM6 [1], when a CU is coded in non-subblock merge mode, if the CU contains at least 64 luma samples, and if both CU width and CU height are less than 128 luma samples, the combined inter/intra prediction (CIIP) mode may be applied to the current CU. The CIIP prediction combines an inter prediction signal with an intra prediction signal. The inter prediction signal in the CIIP mode is derived using the same inter prediction process applied to regular merge mode; and the intra prediction signal is derived following the intra prediction process with the planar mode. As a result, the complexity in CIIP is also a combination of regular inter prediction and intra prediction process.

# Proposal

In order to reduce overall complexity, this contribution proposes to simplify the inter prediction process in CIIP by using integer MV. Specifically, MVs are rounded to the nearest integers, such that the subsequent motion compensation process uses integer reference samples directly to generate the inter prediction signals without the expensive interpolation process.

In the first test, integer MV is used only in bi-prediction mode for CIIP to reduce the worst-case complexity.

# Simulation results

The proposed methods were implemented based on the VTM-6.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE4 [3].

Table 1.  Test 1 use integer MV for bi prediction in CIIP

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.00% | 101% | 100% | - | - | - | - | - |
| Class A2 | 0.03% | 0.03% | 0.06% | 101% | 99% | - | - | - | - | - |
| Class B | 0.02% | 0.01% | 0.01% | 100% | 99% | 0.00% | 0.12% | 0.10% | #VALUE! | #VALUE! |
| Class C | 0.04% | -0.05% | 0.00% | 100% | 98% | 0.00% | 0.09% | -0.04% | 101% | 99% |
| Class E | - | - | - | - | - | -0.07% | 0.56% | 0.65% | 102% | 100% |
| Overall | 0.03% | 0.01% | 0.02% | 100% | 99% | -0.02% | 0.22% | 0.19% | #VALUE! | #VALUE! |
| Class D | -0.03% | 0.13% | -0.06% | 100% | 99% | -0.06% | -0.05% | 0.39% | 100% | 98% |
| Class F | 0.00% | 0.00% | -0.05% | 101% | 99% | 0.06% | 0.00% | 0.85% | 101% | 100% |

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6) v2”, JVET-O2002, Gothenburg, SE, July 2019.

- VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

- C.-C. Chen, H. Yang, X. Xiu, “Description of Core Experiment 4 (CE4): Inter prediction”, JVET-O2024, July 2019, Gothenburg, Sweden.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).