| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0418 |
| --- | --- |

| Title: | Non-CE3: Common Reference Pixel Storage for MRL Intra Prediction and Cross-Component Linear Model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Minhua Zhou Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The Multi-Line Reference (MLR) Intra prediction mode in VVC requires the storage of 3 neighboring luma reference lines with a CTU to generate predictions.  The Cross-Component Linear Model (CCLM) tool also requires 3 neighboring luma reference lines for its downsampling filters.  This contribution proposes to modify the definition of MLR to use the same 3 lines as CCLM.  It claims that this change aligns MLR with CCLM in terms of neighboring pixel referencing and reduces the storage requirements for decoders.  Reported VTM-6.0 test results show a 0.01% RA coding loss from this change.

# Description

Multi-Line Reference (MLR) Intra prediction allows an encoder to choose an alternate set of luma reference pixels for prediction generation.  As shown in Figure 1, the pixels used are 1, 2, and 4 lines distant from the edge of the predicted CU.

Figure 1: Neighboring Luma Pixels (colored) used by MRL

The Cross-Component Linear Model (CCLM) tool also references CU-neighboring luma pixels.  With this tool the neighboring luma pixels are fed to a filter to downsample the luma reference to match the chroma resolution.  Figure 2 gives an example of LM chroma prediction for an 8x8 CU.

Figure 2: Neighboring Luma Pixels (grey) used by LM

As can be seen from Figure 2, CCLM uses pixels that are 1, 2 and 3 lines distant from the edge of the predicted CU.  In both cases an exception is made at the top edge of a CTU to avoid adding extra line buffers to the intra prediction module.  However, the mismatch in neighboring pixel requirements does mean that an extra CTU line of storage is needed vertically and horizontally (256 pixels) to accommodate the 1 line used by MLR that’s not used by CCLM.

The optimization proposed here is simply to change the definition of the furthest reference line used by MRL to align it with those required by CCLM, as shown in Figure 3.  Doing so makes the specification more consistent, and makes the decoder design impact of MRL close to insignificant, as no additional pixel storage is needed to implement it.

Figure 3: Proposed Reference Lines for MRL

# Experimental Results

The tests were carried out by using VTM6.0 reference software. The summary results are provided in Table 1.

When compared to the VTM6.0 anchor, the proposed MRL change results in an average BD-rate increase in AI/RA/LD_B of 0.04%/ 0.01%/ x.xx%.

Although the run times are not be accurate due to the heterogeneous nature of the LSF farm, there is no reason to believe there is any effect on decoder or encoder run time.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.10% | 0.06% | 104% | 79% |
| Class A2 | 0.04% | 0.02% | -0.03% | 104% | 98% |
| Class B | 0.04% | 0.04% | 0.02% | 96% | 73% |
| Class C | 0.06% | -0.08% | 0.04% | 96% | 82% |
| Class E | 0.04% | 0.01% | 0.00% | 94% | 83% |
| Overall | 0.04% | 0.02% | 0.02% | 98% | 81% |
| Class D | 0.02% | 0.03% | -0.06% | 102% | 91% |
| Class F (optional) | 0.06% | -0.06% | -0.08% | 94% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.05% | 106% | 102% |
| Class A2 | 0.04% | -0.02% | 0.08% | 108% | 102% |
| Class B | 0.01% | 0.09% | 0.05% | 102% | 92% |
| Class C | -0.01% | 0.09% | -0.07% | 101% | 80% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.02% | 104% | 92% |
| Class D | -0.08% | -0.37% | 0.10% | 97% | 78% |
| Class F (optional) | 0.02% | 0.06% | 0.07% | 100% | 90% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class C | -0.03% | -0.13% | -0.06% | 106% | 103% |
| Class E | -0.26% | -0.28% | 0.93% | 94% | 82% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class D | 0.03% | 0.20% | 0.66% | 89% | 80% |
| Class F (optional) | -0.01% | -0.06% | 0.31% | 95% | 84% |

# Conclusion

This proposal contends that the decoder implementation cost of MRL mode can be significantly reduced by changing the definition of one of the 3 reference lines used by the tool, and that the resulting coding loss is nearly negligible.  It therefore recommends that this proposal be adopted into the draft text.

The authors would like to thank Sharp for cross-checking.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-v14, the 15th JVET meeting, July 2019, Gothenburg, SE

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.

# Text Changes

The complete text change is highlighted in yellow below.  Table 7-15 on page 138:

Table 7-15 – Specification of IntraLumaRefLineIdx[ x ][ y ] based on intra_luma_ref_idx[ x0 ][ y0 ].

| intra_luma_ref_idx[ x0 ][ y0 ] | IntraLumaRefLineIdx[ x ][ y ] x = x0..x0 + cbWidth − 1 y = y0..y0 + cbHeight − 1 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |

Note – it’s possible to completely remove this table now along with the variable IntraLumaRefLineIdx, since it is now a 1:1 mapping.