| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0419 |
| --- | --- |

| Title: | AHG16/AHG17: Simplification of CU Splitting Controls |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Wade Wan Minhua Zhou Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

VVC Draft 6 contains 14 syntax elements that control CU splitting compared to five in the HEVC standard.  This contribution contents that such a large number of controls adds unnecessary design complexity to VVC and introduces a significant burden to decoder testing and verification.  It proposes to reduce the number of syntax elements to eight, and to remove them all from the slice header.  The proposal claims that these reductions provide the same level of useful encoder flexibility while appreciably reducing the number of combinations to be implemented and tested.

# Problem Statement

Coding units (CUs) in HEVC are sub-divided into quadtrees – four equal sized squares split from one larger square.  HEVC transform units (TUs) can be further sub-divided by quadtrees.  VVC utilizes a more flexible scheme in which CUs can be split in three ways (TUs are not generally further sub-divided):

- Quad split (Qt):  four square sub-CUs from a square CU, as in HEVC

- Binary split (Bt):  two rectangular CUs from a CU

- Ternary split (Tt):  three rectangular CUs from  a CU

The latter two are collectively referred to as multi-type tree (Mtt) splits.  VVC specifies that further Qt splits are not allowed in a CU that has been Mtt split.  VVC also introduces separate luma/chroma CU splitting for intra slices, in which the chroma structure may differ considerably from the luma.

These splitting methods provide significant flexibility in picture partitioning, but not all encoders are equipped to take full advantage of this flexibility, and not all video sequences require it.  In such cases a small amount of coding gain can be achieved by placing limits on the splitting flexibility.  HEVC provides five such controls in the SPS (names shortened for legibility):

- min_luma_coding_block_size:  Smallest allowed CU size

- min/max_transform_block_size (2): Smallest/Largest allowed TU size

- max_transform_hiearchy_depth_inter/intra (2):  Max TU split level for intra/inter slices

VVC expands this list considerably to 14 parameters, and also allows all parameters to be changed at the slice level.  The VVC parameters (simplified names) are:

- min_luma_coding_block_size (1): Smallest allowed CU size (width or height)

- max_transform_block_size (1);  Largest allowed TU size

- MaxMttDepth_inter/intra_y/intra_c (3): Max Mtt split level

- MinQtSize_inter/intra_y/intra_c (3): Minimum size allowed for a Qt split

- MaxBtSize_inter/intra_y/intra_c (3): Maximum size allowed for a Bt split

- MaxTtSize_inter/intra_y/intra_c (3): Maximum size allowed for a Tt split

The first two parameters roughly correspond to their HEVC equivalents.  The latter four groups of parameters are illustrated in figure 1.  Inter = P/B slice types, Intra_y = I slice luma and Intra_c = I slice dual-tree chroma.

Figure 1:  Example of CU Splitting Controls

The cost of this flexibility results in a huge increase in the number of test cases needed to verify a decoder design.  The range of allowed values for each syntax element means that thousands of combinations are possible.  Additionally, the values interact with each other, increasing the chances for design error.

It is important to note that none of this split control flexibility is needed by encoders.  An encoder can emulate all these controls by its choice of split methods.  These controls simply provide some level of coding efficiency for cases where an encoder either cannot use all the choices available, or determines that not all choices are useful.  Moreover, it is difficult to imagine that any encoder would take full advantage of all the selections provided.

# Proposed Simplifications

Three changes are proposed.  Simplified names are used here for ease of discussion.  The detailed specification changes attached to this contribution has the full name changes.

- Merge MaxBtSize & MaxTtSize into MaxMttSize (removes 3 syntax elements)

- Remove MinQtSize (removes 3 syntax elements)

- Remove all controls from the slice header.  They appear only in the SPS, as in HEVC.

The motivation for these changes is described below.

## Merge MaxBtSize & MaxTtSize into MaxMttSize

It is unclear what encoder benefit is likely to be gained by separately specifying the maximum size for binary and ternary splitting.  Ternary spitting is not allowed for 128x128 CUs, but that requirement is separately enforced and does not require separate syntax elements.

The syntax elements for MaxBtSize and MaxTtSize are presently specified relative to MinQtSize.  This proposal changes them to be relative to MaxCUSize instead to increase the probability that their most commonly used values will be 0.

## Remove MinQtSize

It’s very unlikely that an encoder would want to limit the level of Qt splitting while still allowing Bt/Tt splits, which is basically what this parameter controls.  The minimum CU size control can be used by an encoder which wants to limit the overall level of CU splitting.

The one case where a Qt split does not make sense is for an inter CU 8x8, because inter 4x4 CUs are not allowed.  However, this case is already handled explicitly in the Qt splitting rules.

## Remove all controls from the slice header

As noted above, these controls match the splitting rules to encoder capabilities for some coding gain.  As such they’re really sequence-related.  It’s difficult to imagine an encoder that would optimize these at the picture level, much less at the slice level.  Having them present in the slice header just adds to a decoder’s slice processing burden.

# Impact on CTC

No test results are provided, as the changes proposed do not affect the CTC.  This section shows the values presently in use, and why the proposed changes do not affect them.

## Merge MaxBtSize & MaxTtSize into MaxMttSize

| Syntax Element | Value | Notes |
| --- | --- | --- |
| sps_log2_diff_max_bt_min_qt_inter_slice | 4 | MaxBtSize = 128 |
| sps_log2_diff_max_tt_min_qt_inter_slice | 3 | MaxTtSize = 64 |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | 2 | MaxBtSize = 32 |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | 2 | MaxTtSize = 32 |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | 4 | MaxBtSize = 64 |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | 3 | MaxTtSize = 32 |

The CTC uses the same value for Bt and Tt except for the largest CU size (128x128 luma, 64x64 chroma), but this just matches the rule that Tt splits are not allowed for 128x128 CUs, which is separately enforced.

## Remove MinQtSize

| Syntax Element | Value | Notes |
| --- | --- | --- |
| sps_log2_diff_min_qt_min_cb_inter_slice | 1 | MinQtSize = 8 |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | 1 | MinQtSize = 8 |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | 0 | MinQtSize = 4 |

The CTC doesn’t allow luma Qt splits below 8x8, and allows all chroma Qt splits.  The proposal achieves the same effect in the semantics for Qt splitting without needing these syntax elements.

## Remove all controls from the slice header

The CTC does not use slice-level overrides, so this change has no effect.

# Conclusion

This contribution highlights the considerable increase in CU splitting controls in the current VVC draft text compared to HEVC.  It is asserted that the increased number of controls and moving these controls from the sequence level to the slice level result in a significant burden on decoder design, testing and verification without providing any additional encoder functionality.  It is recommended to reduce the number of CU splitting control parameters and to remove them from the slice level, leaving controls only at the SPS level.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-v14, the 15th JVET meeting, July 2019, Gothenburg, SE

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).