| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0421-v1 |
| --- | --- |

| Title: | Non-CE8: Palette mode with 8-bit entries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang, Hsiao Chiang Chuang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current palette design, the bit depth of palette entries is equal to the internal bit depth. This document proposes to set the bit depth of palette entries to 8. With the proposed method, it is claimed that the on-chip memory cost could be saved by up to 50%. The proposed method is implemented and compared with VTM-6.0. Simulation results are summarized as below.

Dual tree on: AI: 0.05%, 100%, 99%; RA: 0.14%, 99%, 100%; LDB: 0.05%, 101%, 99%

Dual tree off: AI: 0.42%, 100%, 99%; RA: 0.20%, 99%, 99%; LDB: 0.23%, 98%, 100%.

# Introduction

In the palette mode, the bit depth of palette entries is same with the internal bit depth, which may be unnecessarily large. A predictor palette with size equal to 63 is used to predict the palette of every CU, which increases the on-chip memory cost. For the worst case that bit depth is equal to 16, the required memory for predictor palette is 16*63*3 = 3024 bits.

# Proposed method

This document proposes to set the bit depth of palette entries to a constant, e.g. 8, to reduce the on-chip memory cost and align to the bit depth of input video.

At encoder side: The palette entries are derived by a clustering algorithm wherein the samples are classified into several groups and the average of each group is used as palette entries. Let S be the sum of samples in a group, N is the number of samples in that group, BD be the internal bit depth. The palette entry in the proposed method is derived as (((S << 6) + (N/2))/N) >> shift, where shift is 6 + BD – 8. The palette entries in the proposed method is represented by 8 bits.

At decoder side:The decoder side only needs a shifting operation. The palette entries are left shifted by deltaBD where the detalBD is the internal bit depth minus 8.

By using the proposed method, the on-chip memory cost of the palette prediction table can be reduced from 16*63*3 = 3024 bits to 8*63*3 = 1512 bits, that is, by 50%.

# Simulation results

The proposed method is evaluated with VTM6. The simulation follows the CE 8 test condition [1]. The detailed results are given in Table 1-6. More detailed results can be found in the attached spreadsheets.

Table 1. Coding performance of the proposed scheme under AI configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.08% | 0.35% | 0.35% | 100% | 99% |
| TGM 720p | 0.16% | 0.19% | 0.26% | 99% | 98% |
| Animation | 0.11% | 0.22% | 0.21% | 100% | 100% |
| Mixed content | 0.02% | 0.18% | 0.35% | 100% | 101% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.05% | 0.21% | 0.26% | 100% | 99% |

Table 2. Coding performance of the proposed scheme under RA configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.06% | 0.16% | 0.26% | 100% | 100% |
| TGM 720p | 0.24% | 0.07% | 0.01% | 99% | 102% |
| Animation | 0.29% | 0.28% | 0.33% | 99% | 99% |
| Mixed content | 0.07% | 0.48% | 0.37% | 99% | 100% |
| Camera-Captured content | -0.01% | 0.03% | -0.02% | 98% | 98% |
| Overall | 0.14% | 0.20% | 0.20% | 99% | 100% |

Table 3. Coding performance of the proposed scheme under LDB configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.25% | 0.00% | 0.29% | 102% | 101% |
| TGM 720p | -0.08% | -0.13% | 0.25% | 99% | 95% |
| Animation | 0.28% | 0.31% | 0.20% | 100% | 100% |
| Mixed content | -0.23% | 0.84% | 0.33% | 103% | 100% |
| Camera-Captured content | -0.02% | 0.00% | -0.02% | 99% | 99% |
| Overall | 0.05% | 0.19% | 0.23% | 101% | 99% |

Table 4. Coding performance of the proposed scheme under AI configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.46% | 0.38% | 0.43% | 101% | 100% |
| TGM 720p | 0.70% | 0.30% | 0.44% | 99% | 96% |
| Animation | 0.41% | 0.97% | 0.85% | 100% | 100% |
| Mixed content | 0.29% | 0.79% | 0.91% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.42% | 0.50% | 0.55% | 100% | 99% |

Table 5. Coding performance of the proposed scheme under RA configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.15% | 0.09% | 0.23% | 98% | 99% |
| TGM 720p | 0.27% | 0.26% | 0.46% | 99% | 98% |
| Animation | 0.34% | 0.68% | 0.65% | 98% | 98% |
| Mixed content | 0.17% | 0.37% | 0.38% | 99% | 100% |
| Camera-Captured content | -0.01% | 0.00% | 0.01% | 98% | 100% |
| Overall | 0.20% | 0.28% | 0.37% | 99% | 99% |

Table 6. Coding performance of the proposed scheme under LDB configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.33% | 0.24% | 0.21% | 99% | 102% |
| TGM 720p | 0.21% | 0.03% | 0.79% | 99% | 97% |
| Animation | 0.27% | 0.85% | 0.67% | 99% | 100% |
| Mixed content | 0.24% | 1.50% | 1.35% | 96% | 99% |
| Camera-Captured content | 0.02% | -0.02% | -0.08% | 100% | 100% |
| Overall | 0.23% | 0.50% | 0.62% | 98% | 100% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

8.4.5.3 Decoding process for palette mode

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

- a variable startComp specifies the first colour component in the palette table,

- a variable cIdx specifying the colour component of the current block,

- two variables nCbW and nCbH specifying the width and height of the current block, respectively.

Output of this process is an array recSamples[ x ][ y ], with x = 0.. nCbW − 1, y = 0.. nCbH − 1 specifying reconstructed sample values for the block.

Depending on the value of cIdx, the variables nSubWidth and nSubHeight are derived as follows:

- If cIdx is equal to 0, nSubWidth is set to 1 and nSubHeight is set to 1.

- Otherwise, nSubWidth is set to SubWidthC and nSubHeight is set to SubHeightC.

The ( nCbW x nCbH ) block of the reconstructed sample array recSamples at location ( xCb, yCb ) is represented by recSamples[ x ][ y ] with x = 0..nCTbW − 1 and y = 0..nCbH − 1, and the value of recSamples[ x ][ y ] for each x in the range of 0 to nCbW − 1, inclusive, and each y in the range of 0 to nCbH − 1, inclusive, is derived as follows:

…

- If bIsEscapeSample is equal to 0, the following applies:

bitDepth = ( cIdx = = 0 ) ? BitDepthY : BitDepthC

recSamples[ x ][ y ] = CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ](8-270)

recSamples[ x ][ y ] = 
CurrentPaletteEntries[ cIdx ][ PaletteIndexMap[ xCb + xL ][ yCb + yL ] ] << (bitDepth - 8 )(8-270)

- Otherwise, if cu_transquant_bypass_flag is equal to 1, the following applies:

recSamples[ x ][ y ] = PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ]	(8-271)

Table 9-77 – Syntax elements and associated binarizations

| Syntax element |  |  |
| --- | --- | --- |
|  | Process | Input parameters |
| new_palette_entries | FL | cMax = cIdx = = 0 ? ( ( 1 << BitDepthY ) − 1 ) : ( (1 << BitDepthC ) − 1 ) 8 |

# Conclusions

It is proposed to set the bit depth of palette entries to 8 to reduce the on-chip memory cost for the palette mode. We recommend adopting the proposed method to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).