| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0422-v1 |
| --- | --- |

| Title: | Non-CE7: Unified neighbouring block selection in context and MV/mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The MV predictor and most probable modes list construction processes access different above and left neighbouring blocks from those used in the context derivation process. This document proposes to unify the positions of neighbouring blocks for different cases, to reduce the number of access of neighbouring information and number of cases for the conformance test. Simulation results reported show that the BD-rate changes are 0.00%, -0.02% and -0.06% for AI, RA and LDB, respectively.

# Introduction

In the process of the merge list construction and the list construction of most probable modes (MPM), the left neighbouring and above neighbouring blocks of the current prediction unit (PU) are fetched. Their positions are ( xCb − 1,  yCb + cbHeight ) and ( xCb + cbWidth, yCb − 1), respectively, wherein (xCb, yCb) is the top left position of the current PU, cbWidth is the width of the PU and cbHeight is the height of the PU, which is illustrated in Figure 1.

However, in the processs of the context derivaiton for syntaxt elments like split_qt_flag, mode_constraint_flag, cu_skip_flag etc., different positions of left neighbouring and above neighbouring blocks are used where the positions are ( xCb − 1,  yCb) and ( xCb, yCb − 1 ) respectively, which is illustrated in Figure 2.

Different from HEVC, VVC supports more flexible partitioning trees (including quad-tree/binanry tree/ternary tree) and more flexible partitioning types (including slice/tile/brick/subpicture). Therefore, checking avaiablity of neighbourbing blocks at different locations would bring too much efforts for conformance/verification tests.

Figure 1 An illustration of left and above blocks in the construction of MV/MPM list.

Figure 2 An illustration of left and above blocks in the process of context derivation.

# Proposed method

To reduce the number of information access and the number of conformance testing cases, this document proposes to unify the positions of left neighbouring and above neighbouring blocks employed in the above processes. For the syntax elements which require to access neighbouring blocks, such as alf_ctb_flag, split_qt_flag, split_cu_flag, mode_constraint_flag, cu_skip_flag, pred_mode_flag, pred_mode_ibc_flag, intra_mip_flag, merge_subblock_flag and inter_affine_flag, the left neighbouring block and above neighbouring blocks used in the MV/mode derivation process are used.

# Simulation Results

The proposed method is evaluated with VTM-6.0. The simulation follows the common test condition [1]. The detailed results are given in Table 1-3. More detailed results can be found in the attached spreadsheets.

Table 1. Coding performance of the proposed scheme under AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.03% | 0.05% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | -0.01% | 98% | 98% |
| Class B | 0.00% | 0.02% | 0.01% | 99% | 99% |
| Class C | 0.00% | -0.12% | 0.02% | 99% | 98% |
| Class E | -0.01% | 0.01% | 0.17% | 100% | 101% |
| Overall | 0.00% | -0.02% | 0.04% | 99% | 99% |
| Class D | -0.01% | 0.07% | -0.05% | 101% | 101% |
| Class F | -0.02% | 0.01% | -0.12% | 100% | 100% |

Table 2. Coding performance of the proposed scheme under RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.02% | -0.15% | 0.06% | 101% | 100% |
| Class A2 | 0.01% | 0.08% | -0.07% | 100% | 100% |
| Class B | -0.05% | 0.06% | 0.05% | 100% | 99% |
| Class C | -0.02% | -0.03% | -0.11% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.07% | -0.28% | -0.11% | 100% | 104% |
| Class F | -0.06% | -0.01% | 0.01% | 100% | 102% |

Table 3. Coding performance of the proposed scheme under LDB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.04% | -0.01% | 100% | 99% |
| Class C | -0.07% | -0.13% | -0.16% | 101% | 99% |
| Class E | -0.11% | 0.08% | 0.73% | 100% | 96% |
| Overall | -0.06% | -0.01% | 0.13% | 100% | 98% |
| Class D | 0.04% | 0.03% | 0.23% | 101% | 100% |
| Class F | -0.03% | 0.35% | 0.92% | 102% | 104% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

9.3.4.4.2 Derivation process of ctxInc using left and above syntax elements

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 ) ( xCb − 1, yCb + cbHeight − 1 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0, y0 − 1 ) ( xCb + cbWidth − 1, yCb − 1 ) and the derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availableA.

# Conclusion

It is proposed to unify the neighbouring block positions to reduce the number of information access and the number of conformance testing cases. We recommend adopting the proposed method to the next version of VVC draft.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and

software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).