| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0423-v1 |
| --- | --- |

| Title: | Non-CE5: Modified boundary strength derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The current design to determine boundary strength needs a number of checks, especially for luma block with blocks at both sides are coded with inter or IBC mode. This document proposes to reduce the number of checks. More specifically, the mv difference is directly compared without considering reference pictures, which is asserted to be beneficial for both hardware and software design. Simulation results are reported to be as follows:

ALF on: AI: 0.00%, 98%, 99%; RA: -0.03%, 99%, 99%; LDB: -0.19%, 99%, 97%

ALF off: AI: 0.00%, 98%, 97%; RA: -0.01%, 97%, 96%; LDB: -0.14%, 98%, 98%.

# Introduction

In the deblocking process, the boundary strength needs to be derived for each boundary to be deblocked. In the current design, the process of boundary strength derivation depends on a significant number of sequential conditions. Therefore, the derivation process is much complicated, especially when the boundary strength needs to be determined by the motion information. Table 1 tabulates these conditions which are checked in the order of conditions 1 to 8.

Table 1. Boundary strength derivation

|  | Conditions | Y | U | V |
| --- | --- | --- | --- | --- |
| 1 | P and Q are BDPCM | 0 | N/A | N/A |
| 2 | P or Q is intra | 2 | 2 | 2 |
| 3 | It is a transform block edge, and P or Q is CIIP | 2 | 2 | 2 |
| 4 | It is a transform block edge, and P or Q has non-zero transform coefficients | 1 | 1 | 1 |
| 5 | It is a transform block edge, and P or Q is JCCR | N/A | 1 | 1 |
| 6 | P and Q are in different coding modes | 1 | 1 | 1 |
| 7 | One or more of the following conditions are true: - P and Q are both IBC, and the BV distance >= half-pel in x- or y-di - P and Q have different ref pictures*, or have different number of MVs - Both P and Q have only one mv, and the MV distance >= half -pel in x- or y-dir - P has two MVs pointing to two different ref pictures, P has two MVs pointing to two same ref pictures, P and Q have same ref pictures in the list 0, the MV pair used in the prediction of the two coding subblocks for the same reference picture in the list 0 or list 1 has a distance >= half-pel in x- or y-dir - Both P and Q have two MVs pointing to the same ref pictures, and both of the following two conditions are satisfied: - The MV of P in the list 0 and the MV of Q in the list 0 has a distance >= half-pel in x- or y-dir or the MV of P in the list 1 and the MV of Q in the list 1 has a distance >= half-pel in x- or y-dir - The MV of P in the list 0 and the MV of Q in the list 1 has a distance >= half-pel in x- or y-dir or the MV of P in the list 1 and the MV of Q in the list 0 has a distance >= half-pel in x- or y-dir *Note: The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different. | 1 | N/A | N/A |
| 8 | Otherwise | 0 | 0 | 0 |

# Proposed method

In the proposed method, if the boundary strength needs to be determined by motion information, only one condition is checked. Let MVM[i].x and MVM[i].y denote the horizontal and vertical component of the motion vector in reference picture list i (i being 0 or 1) of the block at M (M being P or Q) side. If the following equation is satisfied, the boundary strength is set to 1.

(Abs(MVP[0].x - MVQ[0].x) > Th || Abs(MVP[0].y - MVQ[0].y) > Th

|| Abs(MVP[1].x - MVQ[1].x) > Th) || Abs(MVP[1].y - MVQ[1].y) > Th)

Otherwise, the boundary strength is set to 0. The proposed method determines the boundary strength without comparing the reference pictures so it could reduce the number of different cases then reduce the number of branches which benefits both the software and hardware design. Table 2 tabulates the conditions in the proposed boundary strength derivation process.

Table 2. The proposed boundary strength derivation

|  | Conditions | Y | U | V |
| --- | --- | --- | --- | --- |
| 1-6 | Same as VVC design |  |  |  |
| 7 | The absolute difference between the horizontal or vertical component of list 0 or list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples. | 1 | N/A | N/A |
| 8 | Otherwise | 0 | 0 | 0 |

# Simulation results

The proposed method is evaluated with VTM6. The simulation follows the CE 5 test condition [1]. The detailed results are given in Table 3-8.

Table 3. Coding performance of the proposed scheme under AI configuration with ALF on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 99% |

Table 4. Coding performance of the proposed scheme under RA configuration with ALF on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.01% | 0.05% | -0.01% | 99% | 98% |
| Class B | -0.04% | 0.00% | 0.01% | 99% | 99% |
| Class C | -0.06% | 0.05% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.02% | 0.01% | 99% | 99% |
| Class D | -0.03% | -0.09% | 0.02% | 100% | 121% |
| Class F | -0.08% | -0.02% | -0.02% | 100% | 102% |

Table 5. Coding performance of the proposed scheme under LDB configuration with ALF on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.08% | -0.07% | 98% | 99% |
| Class C | -0.08% | -0.01% | 0.09% | 99% | 96% |
| Class E | -0.56% | 0.75% | 0.42% | 98% | 94% |
| Overall | -0.19% | 0.22% | 0.11% | 99% | 97% |
| Class D | -0.10% | 0.28% | 0.18% | 100% | 101% |
| Class F | -0.03% | 0.11% | 0.30% | 101% | 102% |

Table 6. Coding performance of the proposed scheme under AI configuration with ALF off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 95% |

Table 7. Coding performance of the proposed scheme under RA configuration with ALF off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.02% | 0.07% | 0.03% | 97% | 97% |
| Class A2 | 0.01% | -0.08% | 0.03% | 98% | 98% |
| Class B | -0.01% | -0.02% | 0.00% | 97% | 97% |
| Class C | -0.02% | 0.00% | -0.01% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 97% | 96% |
| Class D | -0.03% | 0.00% | 0.04% | 98% | 104% |
| Class F | -0.07% | -0.03% | 0.03% | 98% | 96% |

Table 8. Coding performance of the proposed scheme under LDB configuration with ALF off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.06% | -0.01% | 99% | 99% |
| Class C | -0.05% | 0.12% | -0.05% | 98% | 97% |
| Class E | -0.40% | -0.07% | -0.34% | 98% | 98% |
| Overall | -0.14% | 0.00% | -0.11% | 98% | 98% |
| Class D | 0.00% | 0.37% | 0.08% | 97% | 91% |
| Class F | -0.02% | 0.33% | -0.13% | 98% | 96% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

#### Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

…

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the block edge is also a transform block edge, cIdx is greater than 0, and the sample p0 or q0 is in a transform unit with tu_joint_cbcr_residual_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The absolute difference between the horizontal or vertical component of list 0 or list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples.

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

# Conclusion

The proposed method simplifies the derivation process of boundary strength. We recommend adopting the proposed method to the next version of VVC draft.

# References

- C.-Y. Chen, A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering,” JVET-O2025, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).