| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0425-v1 |
| --- | --- |

| Title: | Non-CE8: On index adjustment in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the palette mode, a palette index adjustment process is utilized to derive the palette index to be used. However, the parsing process of escape pixels depends on the derived palette index, which means that parsing shall wait for the index derivation process. This contribution proposes to remove such a palette index adjustment process when escape pixels are involved. Thus, parsing can be performed without waiting for the index derivation. The proposed method is implemented and evaluated with VTM6 following the CE8 common test condition. Results are summarized as follows.

Dual tree on: AI: 0.04%, 100%, 100%; RA: 0.06%, 99%, 101%; LDB: 0.10%, 99%, 100%

Dual tree off: AI: 0.12%, 99%, 99%; RA: 0.04%, 99%, 100%; LDB: 0.05%, 99%, 99%.

# Introduction

In the palette mode, when signalling the current index, if the left neighbouring selects INDEX mode, the current index should not be the same with its left neighbor since the current encoder will group them together as a run if they are same. Similarly, if the left neighboring selects COPY_ABOVE mode, the current index should not be the same with its above neighbor. Therefore, the encoder in the VTM can remove one candidate based on the neighboring index to shorten the codeword of a palette index. This process is denoted as index adjustment process. Related draft text is

The variable adjustedRefPaletteIndex is derived as follows:

adjustedRefPaletteIndex = MaxPaletteIndex + 1
if( PaletteScanPos > 0 ) {
	xcPrev = x0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 0 ]
	ycPrev = y0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 1 ]
	if( CopyAboveIndicesFlag[ xcPrev ][ ycPrev ]  = =  0 ) {
		adjustedRefPaletteIndex = PaletteIndexMap[ xcPrev ][ ycPrev ] {	(7-157)
	}
	else {
	 	if( !palette_transpose_flag )
			adjustedRefPaletteIndex = PaletteIndexMap[ xC ][ yC − 1 ]
		else
			adjustedRefPaletteIndex = PaletteIndexMap[ xC − 1 ][ yC ]
	}
}

When CopyAboveIndicesFlag[ xC ][ yC ] is equal to 0, the variable CurrPaletteIndex is derived as follows:

if( CurrPaletteIndex  >=  adjustedRefPaletteIndex )
	CurrPaletteIndex++

When a palette index is adjusted to be the maximum index when palette_escape_val_present_flag is equal to 1, syntax palette_escape_val needs to be parsed. Thus, the palette index adjustment process shall be finished before parsing later bits because the derived index may become the index for escape value. Unfortunately, the index adjustment process needs to determine if the neighboring mode is of COPY_ABOVE mode. Based on the information, it is to access palette index in different positions and a comparison is needed to finally get the adjusted index. Basically the whole index map needs to be reconstructed before parsing can start. This process is complicated and the dependency of parsing on the process may influence throughput.

# Proposed method

In this document, it is proposed to perform the index adjustment process based on whether the current CU has escape samples. The adjustment is only performed if the current CU does not have an escape sample, i.e. when palette_escape_val_present_flag is equal to 0.

By doing so, parsing process does not need to wait for palette index adjustment.

# Simulation results

The proposed method is evaluated with VTM6. The simulation follows the CE 8 test conditions [1]. The results are summarized in Table 1-6. More detailed results can be found in the attached spreadsheets.

Table 1. Coding performance of the proposed scheme versus VTM 6 under AI with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.11% | 0.12% | 0.09% | 101% | 101% |
| TGM 720p | 0.00% | 0.12% | -0.03% | 99% | 97% |
| Animation | 0.01% | 0.03% | 0.02% | 100% | 100% |
| Mixed content | 0.07% | 0.08% | 0.11% | 101% | 101% |
| Camera-Captured content | 0.00% | 0.01% | 0.00% | 100% | 104% |
| Overall | 0.04% | 0.08% | 0.04% | 100% | 100% |

Table 2. The performance of the proposed scheme versus VTM 6 under RA with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.03% | 0.09% | 0.08% | 100% | 101% |
| TGM 720p | 0.09% | -0.03% | -0.06% | 99% | 102% |
| Animation | 0.04% | 0.01% | 0.06% | 99% | 100% |
| Mixed content | 0.10% | 0.14% | 0.09% | 99% | 100% |
| Camera-Captured content | 0.01% | -0.02% | 0.02% | 99% | 100% |
| Overall | 0.06% | 0.04% | 0.03% | 99% | 101% |

Table 3. The performance of the proposed scheme versus VTM 6 under LDB with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.22% | 0.13% | 0.16% | 101% | 101% |
| TGM 720p | 0.34% | -0.13% | 0.36% | 97% | 98% |
| Animation | -0.01% | 0.01% | 0.07% | 99% | 98% |
| Mixed content | -0.20% | 0.12% | -0.01% | 100% | 102% |
| Camera-Captured content | 0.02% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.10% | 0.02% | 0.14% | 99% | 100% |

Table 4. The performance of the proposed scheme versus VTM 6 under AI with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.20% | 0.17% | 0.16% | 100% | 100% |
| TGM 720p | 0.20% | -0.15% | 0.07% | 98% | 96% |
| Animation | 0.03% | 0.08% | 0.12% | 99% | 100% |
| Mixed content | 0.05% | 0.05% | 0.04% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.12% | 0.03% | 0.09% | 99% | 99% |

Table 5. The performance of the proposed scheme versus VTM 6 under RA with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.09% | 0.09% | 0.15% | 99% | 101% |
| TGM 720p | 0.07% | 0.13% | -0.08% | 99% | 99% |
| Animation | 0.03% | 0.02% | 0.03% | 98% | 98% |
| Mixed content | 0.00% | 0.42% | 0.35% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.04% | 0.14% | 0.09% | 99% | 100% |

Table 6. The performance of the proposed scheme versus VTM 6 under LDB with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.18% | 0.15% | 0.09% | 100% | 100% |
| TGM 720p | -0.09% | -0.01% | 0.48% | 99% | 97% |
| Animation | -0.04% | -0.01% | -0.01% | 99% | 99% |
| Mixed content | 0.19% | 0.01% | -0.07% | 98% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.05% | 0.04% | 0.13% | 99% | 99% |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

7.3.8.6	Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps ) { | Descriptor |
| --- | --- |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < PredictorPaletteSize[ startComp ] && !palettePredictionFinished && NumPredictedPaletteEntries[ startComp ] < palette_max_size; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlags[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |
| if( NumPredictedPaletteEntries < palette_max_size ) |  |
| num_signalled_palette_entries | ae(v) |
| for( cIdx = startComp; cIdx < ( startComp + numComps); cIdx++ ) |  |
| for( i = 0; i < num_signalled_palette_entries; i++ ) |  |
| new_palette_entries[ cIdx ][ i ] | ae(v) |
| if( CurrentPaletteSize[ startComp ] > 0 ) |  |
| palette_escape_val_present_flag | ae(v) |
| if( MaxPaletteIndex > 0 ) { |  |
| num_palette_indices_minus1 | ae(v) |
| adjust = 0 |  |
| for( i = 0; i <= num_palette_indices_minus1; i++ ) { |  |
| if( MaxPaletteIndex − adjust > 0 ) { |  |
| palette_idx_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_idx_idc |  |
| } |  |
| adjust = 1 palette_escape_val_present_flag |  |
| } |  |
| copy_above_indices_for_final_run_flag | ae(v) |
| palette_transpose_flag | ae(v) |
| } |  |

7.4.9.6	Palette coding semantics

…

The variable adjustedRefPaletteIndex is derived as follows:

adjustedRefPaletteIndex = MaxPaletteIndex + 1
if( PaletteScanPos > 0 ) {
	xcPrev = x0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 0 ]
	ycPrev = y0 + TraverseScanOrder[ log2CbWidth ][ log2bHeight ][ PaletteScanPos − 1 ][ 1 ]
	if( CopyAboveIndicesFlag[ xcPrev ][ ycPrev ]  = =  0 ) {
		adjustedRefPaletteIndex = PaletteIndexMap[ xcPrev ][ ycPrev ] {	(7-157)
	}
	else {
	 	if( !palette_transpose_flag )
			adjustedRefPaletteIndex = PaletteIndexMap[ xC ][ yC − 1 ]
		else
			adjustedRefPaletteIndex = PaletteIndexMap[ xC − 1 ][ yC ]
	}
}

When CopyAboveIndicesFlag[ xC ][ yC ] is equal to 0 and palette_escape_val_present_flag is equal to 0, the variable CurrPaletteIndex is derived as follows:

if( CurrPaletteIndex  >=  adjustedRefPaletteIndex )
	CurrPaletteIndex++		(7-158)

…

palette_idx_idc is an indication of an index to the palette table, CurrentPaletteEntries. The value of palette_idx_idc shall be in the range of 0 to MaxPaletteIndex, inclusive, for the first index in the block and in the range of 0 to ( MaxPaletteIndex – 1 palette_escape_val_present_flag ), inclusive, for the remaining indices in the block.

# Conclusion

It is proposed to apply index adjustment only when the current CU does not have any escape samples to simply the parsing process. We recommend adopting the proposed method to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).