| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0427 |
| --- | --- |

| Title: | AHG17: Bugfixes on constant slice header parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Adarsh Ramasubramonian Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com aramasub@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution is summarized as follows:

- Infer pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 to be equal to zero if pps_six_minus_max_num_merge_cand_plus1 is equal to zero.

- The numerical range of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 on semantics is modified as “0 to 6 – pps_six_minus_max_num_merge_cand_plus1”.

- MaxNumTriangleMergeCand is inferred to be equal to 2 in slice header if MaxNumMergeCand is equal to 2 without signaling in either PPS or slice headers.

# Introduction

In JVET-O0238 it was reported that some of the slice parameters, i.e., dep_quant_enabled_flag, ref_pic_list_sps_flag, slice_temporal_mvp_enabled_flag, mvd_l1_zero_flag, collocated_from_l0_flag, six_minus_max_num_merge_cand, five_minus_max_num_subblock_merge_cand and max_num_merge_cand_minus_max_num_triangle_cand, might stay constant for all slices referring to the same PPS. Therefore, new PPS syntax elements were adopted into VVC WD 6 to indicate whether a corresponding slice header syntax element is constant or not and what the constant value is if it is constant. The syntax design in the VVC Draft 6 [1] are extracted in the table below:

Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. constant_slice_header_params_enabled_flag; u(1). if( constant_slice_header_params_enabled_flag ) {. pps_dep_quant_enabled_idc; u(2). for( i = 0; i < 2; i++ ). pps_ref_pic_list_sps_idc[ i ]; u(2). pps_temporal_mvp_enabled_idc; u(2). pps_mvd_l1_zero_idc; u(2). pps_collocated_from_l0_idc; u(2). pps_six_minus_max_num_merge_cand_plus1; ue(v). pps_five_minus_max_num_subblock_merge_cand_plus1; ue(v). pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1; ue(v). }. …. Section 7.3.6.1 in VVC Draft 6 version 14: slice_header( ) {; Descriptor. …. if ( slice_type != I ) {. …. if( !pps_six_minus_max_num_merge_cand_plus1 ). six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). slice_fpel_mmvd_enabled_flag; u(1). if( sps_bdof_dmvr_slice_present_flag ). slice_disable_bdof_dmvr_flag; u(1). if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ). max_num_merge_cand_minus_max_num_triangle_cand; ue(v). }. ….

Issues on three aspects and the corresponding syntax changes are discussed in the following sections.

# Bug issue on MaxNumTriangleMergeCand

MaxNumTriangleMergeCand shall be smaller than or equal to MaxNumMergeCand based on the adoption of N0447/N400/N500/N0851.

- MaxNumTriangleMergeCand cannnot be larger than MaxNumMergeCand because there may be no enough candidates when triangle merge list construction reuses the regular merge list.

- MaxNumTriangleMergeCand shall be larger than or equal to 2 because triangle merge mode requires two merging candidates, which are selected out of the triangle merge list.

- MaxNumTriangleMergeCand shall be zero if MaxNumMergeCand is equal to 1.

However, the adoption of pps_six_minus_max_num_merge_cand_plus1 and pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 in O0238 may break this requirement if the following two conditions are concurrently met:

- pps_six_minus_max_num_merge_cand_plus1 = 0 : It implies the value of MaxNumMergeCand is not a constant in slices, and is in the range of 1 to 6, inclusive.

- pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 > 0: The value of MaxNumTriangleMergeCand is a constant in the slice headers in slices referring to the PPS.

It can produce a case that MaxNumTriangleMergeCand > MaxNumMergeCand in some slices, e.g. MaxNumTriangleMergeCand = 6 and MaxNumMergeCand could be smaller than 5 in some slices.

- This case produces a bug of no enough merging candidates for triangular merge list.

Proposed 1:  Infer pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 to be equal to zero if pps_six_minus_max_num_merge_cand_plus1 is equal to zero.

- MaxNumTriangleMergeCand shall be derived in slice headers as well if MaxNumMergeCand, which is not a constant, is derived in slice header of slices.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. constant_slice_header_params_enabled_flag; u(1). if( constant_slice_header_params_enabled_flag ) {. pps_dep_quant_enabled_idc; u(2). for( i = 0; i < 2; i++ ). pps_ref_pic_list_sps_idc[ i ]; u(2). pps_temporal_mvp_enabled_idc; u(2). pps_mvd_l1_zero_idc; u(2). pps_collocated_from_l0_idc; u(2). pps_six_minus_max_num_merge_cand_plus1; ue(v). pps_five_minus_max_num_subblock_merge_cand_plus1; ue(v). if ( pps_six_minus_max_num_merge_cand_plus1 ). pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1; ue(v). }. …. Section 7.4.3.4 in VVC Draft 6 version 14: pps_six_minus_max_num_merge_cand_plus1 equal to 0 specifies that six_minus_max_num_merge_cand is present in slice header of slices referring to the PPS. pps_six_minus_max_num_merge_cand_plus1 greater than 0 specifies that six_minus_max_num_merge_cand is not present in slice header of slices referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 6, inclusive. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice header of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is not present in slice header of slices referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_ plus1 shall be in the range of 0 to MaxNumMergeCand − 1. If not present, the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 is inferred to be equal to 0.

# Bug in the semantics of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1

In the current specification, the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1. However, MaxNumMergeCand is actually a variable, i.e., not a constant, which is derived in slice headers of different slices. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 in PPS cannot understand what the value of MaxNumMergeCand is.

- The values of MaxNumMergeCand are different for different slices if six_minus_max_num_merge_cand is present.

- There is a bug for PPS to refer to a parameter like MaxNumMergeCand which is actually a variable in slices and unknown for PPS.

- It is also a text improvement to avoid misunderstanding.

Propopsed 2: The numerical range of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 on semantics is modified as “0 to 6 – pps_six_minus_max_num_merge_cand_plus1”.

- Another solution is to use a new parameter PpsMaxNumMergeCand = 6 – (pps_six_minus_max_num_merge_cand_plus1 – 1) = 7 – pps_six_minus_max_num_merge_cand_plus1. Then the semantics can be modified as “0 to PpsMaxNumMergeCand – 1”.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.4.3.4 in VVC Draft 6 version 14: pps_six_minus_max_num_merge_cand_plus1 equal to 0 specifies that six_minus_max_num_merge_cand is present in slice header of slices referring to the PPS. pps_six_minus_max_num_merge_cand_plus1 greater than 0 specifies that six_minus_max_num_merge_cand is not present in slice header of slices referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 6, inclusive. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice header of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is not present in slice header of slices referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to 6 – pps_six_minus_max_num_merge_cand_plus1.

# Redundant signaling in the case of MaxNumMergeCand = 2

Because MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive, MaxNumTriangleMergeCand must be 2 if MaxNumMergeCand is equal to 2. However, in the case of MaxNumMergeCand = 2, MaxNumTriangleMergeCand is always signaled in either PPS or slice headers.

- If MaxNumTriangleMergeCand is a constant in all slices, pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 = 1 is still signaled in PPS if MaxNumMergeCand = 2.

- If MaxNumTriangleMergeCand is not a constant in all slices, max_num_merge_cand_minus_max_num_triangle_cand = 0 is still signaled if MaxNumMergeCand = 2.

Propopsed 3: MaxNumTriangleMergeCand is inferred to be equal to 2 in slice header if MaxNumMergeCand is equal to 2 without signaling in either PPS or slice headers.

- Infer pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 to be 0 if pps_six_minus_max_num_merge_cand_plus1 is equal to 5.

- Infer max_num_merge_cand_minus_max_num_triangle_cand to be 0 if MaxNumMergeCand is equal to 2.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 6 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. constant_slice_header_params_enabled_flag; u(1). if( constant_slice_header_params_enabled_flag ) {. pps_dep_quant_enabled_idc; u(2). for( i = 0; i < 2; i++ ). pps_ref_pic_list_sps_idc[ i ]; u(2). pps_temporal_mvp_enabled_idc; u(2). pps_mvd_l1_zero_idc; u(2). pps_collocated_from_l0_idc; u(2). pps_six_minus_max_num_merge_cand_plus1; ue(v). pps_five_minus_max_num_subblock_merge_cand_plus1; ue(v). if ( pps_six_minus_max_num_merge_cand_plus1 < 5 ). pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1; ue(v). }. …. Section 7.4.3.4 in VVC Draft 6 version 14: pps_six_minus_max_num_merge_cand_plus1 equal to 0 specifies that six_minus_max_num_merge_cand is present in slice header of slices referring to the PPS. pps_six_minus_max_num_merge_cand_plus1 greater than 0 specifies that six_minus_max_num_merge_cand is not present in slice header of slices referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 6, inclusive. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice header of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is not present in slice header of slices referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_ plus1 shall be in the range of 0 to MaxNumMergeCand − 1. If not present, the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 is inferred to be equal to 0. Section 7.3.6.1 in VVC Draft 6 version 14: slice_header( ) {; Descriptor. …. if ( slice_type != I ) {. …. if( !pps_six_minus_max_num_merge_cand_plus1 ). six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). slice_fpel_mmvd_enabled_flag; u(1). if( sps_bdof_dmvr_slice_present_flag ). slice_disable_bdof_dmvr_flag; u(1). if( sps_triangle_enabled_flag && MaxNumMergeCand >= > 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ). max_num_merge_cand_minus_max_num_triangle_cand; ue(v). }. …. Section 7.4.3.4 in VVC Draft 6 version 14: six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows: MaxNumMergeCand = 6 − six_minus_max_num_merge_cand (7-111) The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive. When not present, the value of six_minus_max_num_merge_cand is inferred to be equal to pps_six_minus_max_num_merge_cand_plus1 − 1. max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slice subtracted from MaxNumMergeCand. When max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 – 1. The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows: MaxNumTriangleMergeCand = MaxNumMergeCand − max_num_merge_cand_minus_max_num_triangle_cand (7-113) When max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive. When max_num_merge_cand_minus_max_num_triangle_cand is not present, and the following applies: - If sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand is equal to 2, MaxNumTriangleMergeCand is set equal to 2. - Otherwise if (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0. When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the current slice.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, the 15th JVET meeting, Jul. 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).