| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0432 |
| --- | --- |

| Title: | AHG12: On subpicture signaling and grid size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The proposal is summarized as follows:

- Replace max_subpics_minus1 with max_subpics_minus2 to start the number of subpictures from 2, and add a requirement of bitsteam conformance that the following constraints apply:

  - NumSubPicGridCols and NumSubPicGridRows cannot be concurenttly equal to 1 if subpics_present_flag is equal to 1.

  - NumSubPics shall be equal to 0 if subpics_present_flag is equal to 0.

- The grid size of subpictures is conditioned on CTU size.

# Introduction

The subpictures signaling was adopted in the previous meeting, and the syntax design [1] is extracted in the table below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

Two issues and the related proposed changes are discussed in the following sections.

# Signaling of subpictures

subpics_present_flag is enabled meaningfully only when there are at least 2 or more subpictures, but the current design allows the case of single subpicture by one of the following setting:

- max_subpics_minus1 is equal to 0.

- NumSubPicGridCols and NumSubPicGridRows are concurrently equal to 1

Note that the number of subpictures equal to 1 is already signaled by setting subpics_present_flag to be 0.

Proposed 1: Replace max_subpics_minus1 with max_subpics_minus2 to start the number of subpictures from 2, and add a requirement of bitsteam conformance that the following constraints apply:

- NumSubPicGridCols and NumSubPicGridRows cannot be concurenttly equal to 1 if subpics_present_flag is equal to 1.

- NumSubPics shall be equal to 0 if subpics_present_flag is equal to 0.

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.3 in VVC Draft 6 version 14: seq_parameter_set_rbsp( ) {; Descriptor. …. subpics_present_flag; u(1). if( subpics_present_flag ) {. max_subpics_minus2; u(8). subpic_grid_col_width_minus1; u(v). subpic_grid_row_height_minus1; u(v). for( i = 0; i < NumSubPicGridRows; i++ ). for( j = 0; j < NumSubPicGridCols; j++ ). subpic_grid_idx[ i ][ j ]; u(v). for( i = 0; i <= NumSubPics; i++ ) {. subpic_treated_as_pic_flag[ i ]; u(1). loop_filter_across_subpic_enabled_flag[ i ]; u(1). }. }. …. Section 7.4.3.3 in VVC Draft 6 version 14: subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax. NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs. max_subpics_minus1 plus 1 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus1 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T ¦ ISO/IEC. max_subpics_minus2 plus 2 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus2 shall be in the range of 0 to 253. The values of 254 and 255 are reserved for future use by ITU-T ¦ ISO/IEC. subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / 4) ) bits. The variable NumSubPicGridCols is derived as follows: NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * 4 + 3 ) / ( subpic_grid_col_width_minus1 * 4 + 4 ) (7-5) subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of 4 samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / 4) ) bits. The variable NumSubPicGridRows is derived as follows: NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * 4 + 3 ) / ( subpic_grid_row_height_minus1 * 4 + 4 ) (7-6) It is a requirement of bitstream conformance that NumSubPicGridCols and NumSubPicGridRows cannot be concurenttly equal to 1 if subpics_present_flag is equal to 1. subpic_grid_idx[ i ][ j ] specifies the subpicture index of the grid position (i, j). The length of the syntax element is Ceil( Log2( max_subpics_minus1 + 1 ) ) bits. The variables SubPicTop[ subpic_grid_idx[ i ][ j ] ], SubPicLeft[ subpic_grid_idx[ i ][ j ] ], SubPicWidth[ subpic_grid_idx [ i ][ j ] ], SubPicHeight[ subpic_grid_idx[ i ][ j ] ], and NumSubPics are derived as follows: NumSubPics = 0 for( i = 0; i. < NumSubPicGridRows; i++ ) { for( j = 0; j < NumSubPicGridCols; j++ ) { if ( i = = 0) SubPicTop[ subpic_grid_idx[ i ][ j ] ] = 0 else if( subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i – 1 ][ j ] ) { SubPicTop[ subpic_grid_idx[ i ][ j ] ] = i SubPicHeight[ subpic_grid_idx[ i − 1][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] } if ( j = = 0) SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = 0 (7-7) else if (subpic_grid_idx[ i ][ j ] != subpic_grid_idx[ i ][ j − 1 ] ) { SubPicLeft[ subpic_grid_idx[ i ][ j ] ] = j SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] } if ( i = = NumSubPicGridRows − 1) SubPicHeight[ subpic_grid_idx[ i ][ j ] ] = i − SubPicTop[ subpic_grid_idx[ i − 1 ][ j ] ] + 1 if (j = = NumSubPicGridRows − 1) SubPicWidth[ subpic_grid_idx[ i ][ j ] ] = j − SubPicLeft[ subpic_grid_idx[ i ][ j − 1 ] ] + 1 if( subpic_grid_idx[ i ][ j ] > NumSubPics) NumSubPics = subpic_grid_idx[ i ][ j ] } } It is a requirement of bitstream conformance that NumSubPics shall be equal to 0 if subpics_present_flag is equal to 0.

# Grid size of subpictures

The current design might need large memory to store the data of subpicture indexes, i.e., subpic_grid_idx[ i ][ j ], because the grid size, 4x4, is too small.

- Take 8192x4320 content as an example, it takes 2.21 Mbyte to store subpic_grid_idx[ i ][ j ] in the worst case

Proposed 2: The grid size of subpictures is conditioned on CTU size.

Solution 2.1: Dependent on CTU size only

- Take 8192x4320 content as an example, it takes 34 kbyte to store subpic_grid_idx[ i ][ j ] in the worst case according to the smallest grid size, i.e., 32x32. 98.4% memory reduction.

Solution 2.2: Dependent on CTU size only, but not smaller than 64x64.

- Take 8192x4320 content as an example, it takes 8.6 kbyte to store subpic_grid_idx[ i ][ j ] in the worst case according to the smallest grid size, i.e., 64x64. 99.6% memory reduction.

To achieve Proposed 2.1 and Proposed 2.2, the signaling of CTU size, log2_ctu_size_minus5, must be moved forward and prior to the syntax element subpic_grid_idx[ i ][ j ].

The syntax changes to VVC Draft 6 are as described in the following table with yellow change mark:

Section 7.3.2.3 in VVC Draft 6 version 14: seq_parameter_set_rbsp( ) {; Descriptor. …. log2_ctu_size_minus5; u(2). …. subpic_grid_col_width_minus1; u(v). subpic_grid_row_height_minus1; u(v). …. log2_ctu_size_minus5; u(2). …. Section 7.4.3.3 in VVC Draft 6 version 14: log2_ctu_size_minus5 plus 5 specifies the luma coding tree block size of each CTU. It is a requirement of bitstream conformance that the value of log2_ctu_size_minus5 be less than or equal to 2. CtbLog2SizeY = log2_ctu_size_minus5 + 5 (7-15) CtbSizeY = 1 << CtbLog2SizeY (7-16) subpic_grid_col_width_minus1 plus 1 specifies the width of each element of the subpicture identifier grid in units of CtbSizeY samples. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY) ) bits. The variable NumSubPicGridCols is derived as follows: NumSubPicGridCols = ( pic_width_max_in_luma_samples + subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY – 1 ) / ( subpic_grid_col_width_minus1 * CtbSizeY + CtbSizeY ) (7-5) subpic_grid_row_height_minus1 plus 1 specifies the height of each element of the subpicture identifier grid in units of CtbSizeY samples. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY) ) bits. The variable NumSubPicGridRows is derived as follows: NumSubPicGridRows = ( pic_height_max_in_luma_samples + subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY – 1 ) / ( subpic_grid_row_height_minus1 * CtbSizeY + CtbSizeY ) (7-6)

The syntax change in Solution 2.2 simply replaces CtbSizeY with max(64, CtbSizeY) in the above table.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, the 15th JVET meeting, Jul. 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).