| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0434 |
| --- | --- |

| Title: | Non-CE4: Enabling conditions of PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Na Zhang, Kui Fan Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the last JVET meeting, PROF was adopted with the merit of similar optical flow operations required by BDOF. Enabling and disabling PROF is signalled in SPS. Therefore, it lacks flexibility to disable it for different pictures/slices. In addition, BDOF can be disabled in slice level with a flag signalled in slice header. In this contribution, it is proposed to use the same slice level flag to control the usage of BDOF and PROF. It is reported that there are no performance changes under the common test conditions.

# Introduction

PROF and BDOF are two coding tools that replies on the usage of optical flow operations. In current VVC WD6 [1], BDOF can be disabled through a slice level BDOF disabling flag. However, PROF cannot be disabled at slice level, which lacks flexibility for slice/picture level PROF control.

# Proposal

It is proposed to use the same flag slice_disable_bdof_dmvr_flag to control the usage of PROF at slice level. Since it now controls three coding tools, i.e., BDOF, DMVR and PROF, the name is revised accordingly.

The proposed changes on top of VVC draft 6 [1] are described as below. Newly added changes are highlighted in yellow and deleted texts are marked with strikethrough.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| … |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag ¦ ¦ sps_affine_prof_enabled_flag) |  |
| sps_bdof_dmvr_prof_slice_present_flag | u(1) |
| … |  |

### 7.3.6Slice header syntax

#### 7.3.6.1General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| if( sps_bdof_dmvr_prof_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_prof_flag | u(1) |
| … |  |
| } |  |

#### 7.4.3.3. Sequence parameter set RBSP semantics

sps_bdof_dmvr_prof _slice_present_flag equal to 1 specifies that slice_disable_bdof_dmvr_prof_flag is present in slice headers referring to the SPS. sps_bdof_dmvr_prof_slice_present_flag equal to 0 specifies that slice_disable_bdof_dmvr_prof _flag is not present in slice headers referring to the SPS. When sps_bdof_dmvr_prof _slice_present_flag is not present, the value of sps_bdof_dmvr_prof _slice_present_flag is inferred to be equal to 0.

7.4.7.1 General slice header semantics

slice_disable_bdof_dmvr_prof_flag equal to 1 specifies that neither of bi-directional optical flow inter prediction, prediction refinement with optical flow and decoder motion vector refinement based inter bi-prediction is enabled in the current slice. slice_disable_bdof_dmvr_prof_flag equal to 0 specifies that bi-directional optical flow inter prediction or decoder motion vector refinement based inter bi-prediction or prediction refinement with optical flow may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_prof_flag is not present, the value of slice_disable_bdof_dmvr_prof_flag is inferred to be 0.

8.5.5.9	Derivation process for motion vector arrays from affine control point motion vectors

…

The variable cbProfFlagLX is derived as follows:

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

  - sps_affine_prof_enabled_flag slice_disable_bdof_dmvr_prof_flag is equal to 10.

  - fallbackModeTriggered is equal to 1.

  - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

  - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

- Otherwise, cbProfFlagLX set equal to TRUE.

8.5.6.3.1	General

The prediction block border extension size brdExtSize is derived as follows:

brdExtSize = ( bdofFlag | | ( inter_affine_flag[ xSb ][ ySb ] && sps_affine_prof_enabled_flag slice_disable_bdof_dmvr_prof_flag ) ) ? 2 : 0	(8-752)

The variable fRefWidth is set equal to the PicOutputWidthL of the reference picture in luma samples.

…

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag slice_disable_bdof_dmvr_prof_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

…

# Simulation results

No performance changes are observed under the common test conditions.

# Conclusions

It is proposed to use the same slice-level on/off controlling method for PROF and BDOF. With the proposed method, the optical flow operations could be controlled in slice level. It is recommended adopting the proposed method.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).