| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0435 |
| --- | --- |

| Title: | CE7-related: Modifications to transform skip significant flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Xin Zhao, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In VVC Draft 6, three contexts are used to encode significant coefficient flag in transform skip mode. This contribution proposes to use four contexts to improve coding efficiency.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37], experimental results show -0.17% AI, -0.19% RA, -0.25% LB luma BD-rate for Class F, and -0.34% AI, -0.28% RA, -0.29% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17], experimental results show -0.08% AI, -0.20% RA, -0.17% LB luma BD-rate for Class F, and -0.21% AI, -0.24% RA, -0.24% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37] with the proposed at 1.75 bins/coefficient, experimental results show -0.07% AI, -0.03% RA, -0.22% LB luma BD-rate for Class F, and -0.06% AI, -0.10% RA, -0.25% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17] with the proposed at 1.75 bins/coefficient, experimental results show 0.24% AI, 0.27% RA, 0.19% LB luma BD-rate for Class F, and 0.24% AI, 0.31% RA, 0.09% LB luma BD-rate for Class TGM.

# Introduction

This contribution proposes a method to improve the coding efficiency of significant flag signaling in transform skip mode. This is a part of three contributions intended to improve the coding efficiency in transform skip mode at 1.75 bins per coefficients at standard QPs and low QPs.

In VVC Draft 6 [1] transform skip mode, the ctxInc of the significant coefficient flag is

ctxInc = 60 + offsetCtxInc

where the offset context increment, offsetCtxInc, can be determined by Table 1, where

leftSigFlag = xC > 0 ? sig_coeff_flag[ xC - 1 ][ yC ] : 0

aboveSigFlag = yC > 0 ? sig_coeff_flag[ xC ][ yC − 1 ] : 0

As shown in Table 1, VTM-6.0 [2] divides all possible outcomes of the pair (leftSigFlag, aboveSigFlag) into three classes:

The leftSigFlag and the aboveSigFlag are both zero.

The leftSigFlag and the aboveSigFlag are both one.

The leftSigFlag and the aboveSigFlag are different.

Each class is assigned with a unique offsetCtxInc number between 0 to 2.

Table 1: A table to select ctxIncOffset based on leftSigFlag and aboveSigFlag for VTM-6.0 [3].

| leftSigFlag | aboveSigFlag | offsetCtxInc |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |

In particular to VVC Draft 6 [1], the number of context models for coding sig_coeff_flag[ xC ][ yC ] with transform skip mode is 3.

# Proposed transform skip mode significant flag coding

In this contribution, the value of the offset context increment, OffsetCtxInc, defined as ctxInc – 60, can be determined by Table 2, where

leftSigFlag = xC > 0 ? sig_coeff_flag[ xC – 1 ][ yC ] : 0

aboveSigFlag = yC > 0 ? sig_coeff_flag[ xC ][ yC – 1 ] : 0

diagSigFlag = (xC > 0 && yC > 0) ? sig_coeff_flag[ xC – 1 ][ yC – 1 ] : 0

As shown in Table 2, this contribution divides all possible outcomes of the tuple

(leftSigFlag, aboveSigFlag, diagSigFlag)

into four classes:

The leftSigFlag and the aboveSigFlag are both zero.

The leftSigFlag and the aboveSigFlag are both one.

The leftSigFlag and the aboveSigFlag are different and the diagSigFlag is zero.

The leftSigFlag and the aboveSigFlag are different and the diagSigFlag is one.

Each class is assigned with a unique offsetCtxInc number between 0 to 3.

Table 2: A table to select offsetCtxInc based on (leftSigFlag, aboveSigFlag, diagSigFlag).

|  |  | diagSigFlag |  |
| --- | --- | --- | --- |
| leftSigFlag | aboveSigFlag | 0 | 1 |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 3 |
| 1 | 0 | 1 | 3 |
| 1 | 1 | 2 | 2 |

In this contribution, the number of context models for coding sig_coeff_flag[ xC ][ yC ] with transform skip mode is increased from 3 to 4.

# Results

The testing condition of the proposed follows the CE7 description [3] testing conditions with standard and low QP. In particular, the anchor results are encoded at 2 bins per coefficient in transform skip mode. The proposed is tested at both 2 and 1.75 bins per coefficient in transform skip mode.

As reference, the results of VTM-6.0 at 2 bins per coefficient is compared with VTM-6.0 at 1.75 bins per coefficient in Table 3 for standard QPs and Table 4 for low QPs to show the coding losses at 1.75 bins per coefficient.

A summary of all results for Class F and Class TGM is in the following Table:

Table 3: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with standard QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 98% |
| Class E | 0.00% | 0.10% | -0.01% | 99% | 95% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.01% | 99% | 96% |
| Class F | 0.11% | 0.10% | 0.15% | 99% | 100% |
| Class TGM | 0.26% | 0.22% | 0.23% | 97% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 99% | 99% |
| Class A2 | 0.02% | 0.14% | 0.09% | 99% | 99% |
| Class B | 0.01% | 0.10% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 99% | 99% |
| Class D | -0.03% | -0.48% | 0.02% | 100% | 99% |
| Class F | 0.07% | 0.10% | 0.17% | 99% | 99% |
| Class TGM | 0.20% | 0.14% | 0.13% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.04% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 99% |
| Class E | -0.09% | 0.46% | 0.57% | 100% | 99% |
| Overall | -0.03% | 0.15% | 0.10% | 99% | 99% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 98% |
| Class F | 0.05% | 0.29% | 0.62% | 99% | 99% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100% | 102% |

Table 4: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with low QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 98% |
| Class E | 0.05% | 0.00% | 0.02% | 99% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 99% | 98% |
| Class F | 0.30% | 0.14% | 0.14% | 98% | 99% |
| Class TGM | 0.46% | 0.42% | 0.41% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.04% | 99% | 98% |
| Class F | 0.38% | 0.00% | 0.12% | 99% | 99% |
| Class TGM | 0.49% | 0.40% | 0.40% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.02% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | -0.02% | 0.03% | -0.02% | 99% | 98% |
| Class F | 0.03% | 0.09% | 0.05% | 99% | 99% |
| Class TGM | 0.41% | 0.32% | 0.42% | 99% | 96% |

## Standard QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 99% | 101% |
| Class A2 | 0.00% | 0.01% | -0.01% | 100% | 101% |
| Class B | 0.00% | 0.02% | -0.03% | 100% | 98% |
| Class C | -0.01% | -0.12% | -0.02% | 100% | 98% |
| Class E | -0.05% | -0.02% | 0.00% | 100% | 97% |
| Overall | -0.01% | -0.02% | -0.01% | 100% | 99% |
| Class D | -0.01% | 0.00% | -0.23% | 99% | 98% |
| Class F | -0.17% | -0.15% | -0.27% | 100% | 99% |
| Class TGM | -0.34% | -0.33% | -0.34% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | 0.06% | 99% | 100% |
| Class A2 | 0.01% | 0.13% | -0.07% | 99% | 100% |
| Class B | -0.02% | -0.02% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.08% | -0.06% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.06% | -0.19% | 0.11% | 100% | 98% |
| Class F | -0.19% | -0.19% | -0.18% | 100% | 99% |
| Class TGM | -0.28% | -0.30% | -0.25% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.12% | -0.06% | 99% | 100% |
| Class C | -0.05% | -0.05% | -0.18% | 99% | 99% |
| Class E | -0.05% | 0.37% | 1.29% | 100% | 99% |
| Overall | -0.03% | 0.13% | 0.24% | 99% | 99% |
| Class D | 0.00% | 0.13% | -0.14% | 100% | 99% |
| Class F | -0.25% | 0.02% | 0.26% | 99% | 98% |
| Class TGM | -0.29% | -0.16% | -0.19% | 100% | 100% |

## Low QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.02% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.02% | 0.01% | 99% | 99% |
| Class F | -0.08% | -0.13% | -0.14% | 99% | 98% |
| Class TGM | -0.21% | -0.21% | -0.22% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.01% | -0.02% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.01% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.05% | -0.02% | 100% | 99% |
| Class F | -0.20% | -0.32% | -0.21% | 100% | 99% |
| Class TGM | -0.24% | -0.19% | -0.19% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class C | 0.00% | -0.02% | 0.00% | 100% | 99% |
| Class E | 0.01% | -0.04% | 0.05% | 99% | 99% |
| Overall | 0.00% | -0.02% | 0.02% | 100% | 99% |
| Class D | 0.00% | 0.02% | -0.04% | 99% | 96% |
| Class F | -0.17% | -0.23% | -0.22% | 100% | 99% |
| Class TGM | -0.24% | -0.32% | -0.27% | 100% | 98% |

## Standard QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.01% | -0.06% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.05% | 100% | 99% |
| Class E | -0.04% | -0.03% | -0.01% | 99% | 97% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 99% |
| Class D | 0.02% | -0.15% | -0.12% | 99% | 98% |
| Class F | -0.07% | -0.12% | -0.07% | 100% | 100% |
| Class TGM | -0.06% | -0.11% | -0.10% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.06% | 99% | 100% |
| Class A2 | 0.02% | 0.01% | -0.01% | 99% | 100% |
| Class B | -0.02% | 0.02% | 0.09% | 100% | 100% |
| Class C | 0.00% | 0.10% | 0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.06% | 100% | 100% |
| Class D | -0.05% | -0.43% | 0.31% | 100% | 99% |
| Class F | -0.03% | -0.08% | 0.01% | 100% | 99% |
| Class TGM | -0.10% | -0.15% | -0.18% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.03% | -0.34% | 99% | 100% |
| Class C | 0.00% | 0.09% | -0.01% | 100% | 97% |
| Class E | 0.00% | 0.46% | 0.67% | 100% | 98% |
| Overall | -0.01% | 0.16% | 0.02% | 99% | 99% |
| Class D | 0.00% | 0.15% | 0.22% | 99% | 96% |
| Class F | -0.22% | -0.43% | 0.02% | 99% | 97% |
| Class TGM | -0.25% | -0.24% | -0.20% | 101% | 101% |

## Low QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.04% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class C | 0.01% | 0.01% | 0.00% | 100% | 98% |
| Class E | 0.05% | 0.02% | 0.04% | 99% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.03% | 0.01% | 99% | 99% |
| Class F | 0.24% | 0.04% | 0.04% | 99% | 99% |
| Class TGM | 0.24% | 0.20% | 0.20% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.01% | 99% | 100% |
| Class A2 | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.02% | 100% | 101% |
| Class C | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.02% | -0.03% | 0.03% | 99% | 98% |
| Class F | 0.27% | -0.08% | 0.01% | 100% | 101% |
| Class TGM | 0.31% | 0.12% | 0.10% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.01% | 0.01% | 99% | 98% |
| Class E | 0.03% | -0.02% | 0.01% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.01% | 0.06% | 0.00% | 99% | 98% |
| Class F | 0.19% | 0.14% | 0.16% | 100% | 98% |
| Class TGM | 0.09% | 0.21% | 0.23% | 99% | 97% |

# Proposed Text

The proposed method is described in the form of text modifications relative to the specification text of VVC Draft 6 [1] shown below. The text changes are highlighted in yellow.

9.3.4.2.8	Derivation process of ctxInc for the syntax element sig_coeff_flag

Inputs to this process are the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the variable ctxInc.

The variable locSumAbsPass1 is derived by invoking the derivation process for the variables locNumSig and locSumAbsPass1 specifies in clause 9.3.4.2.7 with colour component index cIdx, the luma location ( x0, y0), the current coefficient scan location (xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as input.

The variable d is set equal to xC + yC.

The variable ctxInc is derived as follows:

- If transform_skip_flag[ xS ][ yS ] is equal to 1, the following applies:

diagSigFlag = (xC > 0 && yC > 0) ? sig_coeff_flag[ xC – 1 ][ yC – 1 ] : 0

ctxInc = 60 + ((locNumSig == 1) ? 2 *diagSigFlag + 1 : locNumSig)	(9-51)

- Otherwise ( transform_skip_flag[ xS ][ yS ] is equal to 1 ), the following applies:

- If cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = 12 * Max( 0, QState − 1) + Min( ( locSumAbsPass1 + 1 ) >> 1, 3 ) + 	(9-52)
	( d < 2  ?  8  :  ( d < 5  ?  4  :  0 ) )

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 36 + 8 * Max( 0, QState − 1) + Min( ( locSumAbsPass1 + 1 ) >> 1, 3 ) + ( d < 2  ?  4  :  0 )	(9-53)

Table 4: Association of ctxIdx and residual coding syntax elements for each initializationType in the initialization process extracted from Table 9-4 of VCC Draft 6[1] with modifications in yellow.

| residual_coding( ) | last_sig_coeff_x_prefix | Table 9-70 | 0..22 | 23..45 | 46..68 |
| --- | --- | --- | --- | --- | --- |
|  | last_sig_coeff_y_prefix | Table 9-71 | 0..22 | 23..45 | 46..68 |
|  | coded_sub_block_flag[ ][ ] | Table 9-72 | 0..7 | 8..15 | 16..23 |
|  | sig_coeff_flag[ ][ ] | Table 9-73 | 0..63 | 64..127 | 128..191 |
|  | par_level_flag[ ] | Table 9-74 | 0..32 | 33..65 | 66..98 |
|  | abs_level_gtx_flag[ ][ ] | Table 9-75 | 0..73 | 74..147 | 148..220 |
|  | coeff_sign_flag[ ] | Table 9-76 | 0..5 | 6..11 | 12..17 |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001-vE, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-6.0.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027-v3, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).