| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0444 |
| --- | --- |

| Title: | Non-CE4: DMVR without padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xiaozhong Xu, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to use actual reference samples instead of padded samples for DMVR, in order to reduce DMVR processing time on decoder. To keep worst case memory bandwidth unchanged, DMVR search range is reduced to 1 luma sample when block size is 8x16 or 16x8 and unchanged for other block sizes. The proposed method was implemented on top of VTM-6.0, and simulations were conducted with common test conditions on random access configuration. It is reported that compared with VTM-6.0 the proposed methods have overall performance of RA 0.05%, Enc 100%, Dec 96%, DMVR processing time 73%, worst case memory bandwidth unchanged.

# Introduction

In current VTM-6.0, DMVR process needs to add padded samples for each processing sub-PU. This sub-PU based padding process has significant runtime impact on software decoders. For efficient SIMD optimization, samples in padded sub-PUs need to be in continuous buffer. However, DMVR is based on samples in reference pictures and those samples cannot be overwritten. Therefore, samples in sub-PUs are first copied to temporary buffers and then padded. After that, DMVR matching is performed on the temporary buffers. Such operations introduce a lot of memory copy, which will slow down practical software decoder significantly.

Figure 1 - DMVR processing sub-PUs

# Proposed Method

It is proposed to load the actual reference pixels of a coding block with extended range on each side for DMVR processing purpose but still at sub-PU level. By this way, padding process is skipped so that additional temporal buffer and memory copy are both avoided. Note that as no padding is used, sub-PU level memory load and CU level memory load as shown below actually lead to the same coding results, which gives codec implementation more room for optimization.

Figure 2 - Extra loaded reference samples for DMVR at CU level

## DMVR on actual reference samples with modified search range

In this method, DMVR search range remains as 2 samples on each side, except for cases that the block size is 8x16 or 16x8 luma samples. When block size is 8x16 or 16x8, the DMVR search range is changed to 1 sample on each side.

With this method, the worst-case memory bandwidth requirement was when CU size is 8x16 or 16x8 luma samples, which can be calculated as in equation (1). The memory bandwidth is lower than regular 8x8 bi-prediction.

2 * (16 + 7 + 2*1) * (8 + 7 + 2*1) / (16 * 8) = 6.64			(1)

For comparison purposes, regular 8x8 bi-prediction has memory bandwidth requirement as calculated in equation (2).

2 * (8 + 7) * (8 + 7) / (8 * 8) = 7.03						(2)

# Experimental Results

The proposed methods were implemented based on VTM-6.0. Since DMVR is only available in Random Access configuration, simulations were conducted on RA following JVET common test conditions. VTM-6.0 was used as anchor.

## Results of method with modified search range

As shown in Table 3.1, it was observed that the BD-rate change of proposed method is 0.05% on RA compared with VTM-6.0 anchor. Decoding time is reduced by 4%.

DMVR processing time is also measured for both VTM-6.0 anchor and the proposed method. It was observed that the proposed method reduced DMVR processing time by 27% compared to VTM-6.0.

Table 3.1 Simulation Results of Method with modified search range

|  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |  |
|  | Y | U | V | EncT | DecT | DMVR Time |
| Class A1 | 0.02% | 0.04% | 0.07% | 100% | 95% | 70% |
| Class A2 | 0.08% | 0.02% | 0.06% | 100% | 95% | 72% |
| Class B | 0.05% | 0.10% | 0.04% | 100% | 96% | 73% |
| Class C | 0.04% | 0.09% | 0.04% | 100% | 97% | 76% |
| Class E |  |  |  |  |  |  |
| Overall | 0.05% | 0.07% | 0.05% | 100% | 96% | 73% |
| Class D | 0.05% | 0.09% | 0.10% | 100% | 98% | 71% |
| Class F | 0.03% | 0.05% | 0.05% | 100% | 98% | 76% |

# Conclusion

It was observed that the proposed method shows minor BD-rate change on RA configuration, with 4% decoding time reduction. DMVR processing time was reduced significantly by 27%. It is proposed to adopt this method into VVC and VTM.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).