| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0451_r1 |
| --- | --- |

| Title: | CE7-related: Combination tests of JVET-P0435, JVET-P0437, and JVET-P0447 for residual coding at 1.75 bins/coefficient. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Xin Zhao, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports results of combination tests of JVET-P0435, JVET-P0437, and JVET-P0447 for residual coding at 1.75 bins/coefficient.

When VTM-6.0 with 1.75 bins/coefficient and standard QPs with QP in [22, 27, 32, 37] is compared with VTM-6.0 with 2 bins/coefficient, experimental results show 0.11% AI, 0.07% RA, 0.05% LB luma BD-rate for Class F, and 0.26% AI, 0.20% RA, -0.02% LB luma BD-rate for Class TGM.

Using VTM-6.0 (i.e. with 2 bins/coefficient in transform skip mode) and standard QPs as anchor, the experimental results of combined JVET-P0435, JVET-P0437 and JVET-P0447 at 1.75 bins/coefficient and  standard QPs show -0.11% AI, -0.06% RA, -0.24% LB luma BD-rate for Class F, and -0.09% AI, -0.14% RA, -0.35% LB luma BD-rate for Class TGM.

When VTM-6.0 with 1.75 bins/coefficient and low QPs with QP in [2, 7, 12, 17]  is compared with VTM-6.0 with 2 bins/coefficient, experimental results show 0.30% AI, 0.38% RA, 0.03% LB luma BD-rate for Class F, and 0.46% AI, 0.49% RA, 0.41% LB luma BD-rate for Class TGM.

Using VTM-6.0 (i.e. with 2 bins/coefficient in transform skip mode) and low QPs as anchor, the experimental results of combined JVET-P0435, JVET-P0437 and JVET-P0447 at 1.75 bins/coefficient and low QPs, show 0.02% AI, 0.00% RA, -0.23% LB luma BD-rate for Class F, and -0.18% AI, -0.45% RA, -0.37% LB luma BD-rate for Class TGM.

This revision is updated to report all low QP test results.

# Introduction

This contribution reports results of combination tests of JVET-P0435 [1], JVET-P0437 [2], and JVET-P0447 [3] for residual coding. The contribution JVET-P0435 proposed to modify the coding of transform skip significant flag. The contribution JVET-P0437 proposed to modify the coding of the transform skip coefficient sign flag. The contribution JET-P0447 reported the results of extending the range of rice parameter for residual coding in transform skip mode.

In this report, the anchor is VTM-6.0 [4] (i.e. with 2 bins/coefficient in transform skip mode) at standard QP and low QP in accordance to CE7 [5]. The tested are encoded at 1.75 bins/coefficient with and without transform skip.

As reference, the results of VTM-6.0 at 2 bins per coefficient is compared with VTM-6.0 at 1.75 bins per coefficient in Table 3 for standard QPs and Table 4 for low QPs to show the coding losses at 1.75 bins per coefficient.

A summary of all results for Class F and Class TGM at 1.75 bins/coefficient is in the following Table:

Table 3: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with standard QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 98% |
| Class E | 0.00% | 0.10% | -0.01% | 99% | 95% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.01% | 99% | 96% |
| Class F | 0.11% | 0.10% | 0.15% | 99% | 100% |
| Class TGM | 0.26% | 0.22% | 0.23% | 97% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 99% | 99% |
| Class A2 | 0.02% | 0.14% | 0.09% | 99% | 99% |
| Class B | 0.01% | 0.10% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 99% | 99% |
| Class D | -0.03% | -0.48% | 0.02% | 100% | 99% |
| Class F | 0.07% | 0.10% | 0.17% | 99% | 99% |
| Class TGM | 0.20% | 0.14% | 0.13% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.04% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 99% |
| Class E | -0.09% | 0.46% | 0.57% | 100% | 99% |
| Overall | -0.03% | 0.15% | 0.10% | 99% | 99% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 98% |
| Class F | 0.05% | 0.29% | 0.62% | 99% | 99% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100% | 102% |

Table 4: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with low QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 98% |
| Class E | 0.05% | 0.00% | 0.02% | 99% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 99% | 98% |
| Class F | 0.30% | 0.14% | 0.14% | 98% | 99% |
| Class TGM | 0.46% | 0.42% | 0.41% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.04% | 99% | 98% |
| Class F | 0.38% | 0.00% | 0.12% | 99% | 99% |
| Class TGM | 0.49% | 0.40% | 0.40% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.02% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | -0.02% | 0.03% | -0.02% | 99% | 98% |
| Class F | 0.03% | 0.09% | 0.05% | 99% | 99% |
| Class TGM | 0.41% | 0.32% | 0.42% | 99% | 96% |

# Results of combining JVET-P0435, JET-P0437 and JVET-P0447

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.  The test is the combined significant flag coding in JVE-P0435 [1], sign flag coding in JVE-P0437 [2] and Rice parameter range extension in JET-P0447 [3].

## Standard QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.02% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.05% | 100% | 98% |
| Class C | 0.00% | -0.11% | -0.02% | 100% | 98% |
| Class E | -0.04% | 0.01% | -0.01% | 99% | 97% |
| Overall | -0.01% | -0.01% | -0.02% | 100% | 99% |
| Class D | 0.01% | -0.06% | -0.12% | 99% | 96% |
| Class F | -0.11% | -0.20% | -0.22% | 99% | 99% |
| Class TGM | -0.09% | -0.13% | -0.14% | 97% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 99% | 100% |
| Class A2 | 0.04% | 0.00% | 0.03% | 99% | 100% |
| Class B | -0.02% | 0.03% | 0.07% | 100% | 100% |
| Class C | 0.01% | 0.06% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 99% | 99% |
| Class D | -0.01% | -0.41% | 0.06% | 100% | 98% |
| Class F | -0.06% | -0.18% | -0.05% | 99% | 98% |
| Class TGM | -0.14% | -0.18% | -0.16% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.11% | -0.06% | 99% | 100% |
| Class C | -0.03% | 0.03% | -0.13% | 100% | 95% |
| Class E | -0.09% | 0.03% | 0.73% | 99% | 97% |
| Overall | -0.04% | 0.06% | 0.11% | 99% | 98% |
| Class D | -0.02% | -0.23% | 0.27% | 100% | 97% |
| Class F | -0.24% | -0.08% | -0.17% | 99% | 98% |
| Class TGM | -0.35% | -0.26% | -0.27% | 100% | 101% |

## Low QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.02% | 0.03% |  | 0.04% |  | 100% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
| Class B | 0.01% | -0.01% |  | 0.00% |  | 100% |  | 100% |  |
| Class C | 0.00% | 0.02% |  | 0.03% |  | 100% |  | 99% |  |
| Class E | 0.07% | -0.03% |  | 0.02% |  | 101% |  | 102% |  |
| Overall | 0.02% | 0.00% |  | 0.01% |  | 100% |  | 100% |  |
| Class D | 0.01% | -0.02% |  | 0.01% |  | 100% |  | 98% |  |
| Class F | 0.02% | -0.10% |  | -0.07% |  | 99% |  | 98% |  |
| Class TGM | -0.18% | -0.16% |  | -0.15% |  | 100% |  | 98% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.02% | -0.01% |  | -0.02% |  | 100% |  | 100% |  |
| Class A2 | 0.00% | 0.00% |  | 0.01% |  | 100% |  | 100% |  |
| Class B | 0.01% | 0.01% |  | 0.00% |  | 100% |  | 101% |  |
| Class C | 0.00% | -0.03% |  | -0.01% |  | 100% |  | 99% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class D | -0.01% | -0.09% |  | -0.06% |  | 99% |  | 97% |  |
| Class F | 0.00% | -0.26% |  | -0.25% |  | 99% |  | 99% |  |
| Class TGM | -0.45% | -0.10% |  | -0.09% |  | 100% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.01% |  | 0.01% |  | 100% |  | 99% |  |
| Class C | 0.00% | 0.03% |  | 0.01% |  | 100% |  | 98% |  |
| Class E | 0.01% | -0.01% |  | -0.03% |  | 100% |  | 100% |  |
| Overall | 0.01% | 0.01% |  | 0.00% |  | 100% |  | 99% |  |
| Class D | 0.01% | 0.02% |  | -0.02% |  | 99% |  | 96% |  |
| Class F | -0.23% | -0.16% |  | -0.17% |  | 101% |  | 99% |  |
| Class TGM | -0.37% | -0.08% |  | -0.08% |  | 99% |  | 98% |  |

# Results of combining JVET-P0435 and JVET-P0437

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

The test is the combined significant flag coding in JVE-P0435 [1] and sign flag coding in JVE-P0437 [2].

## Standard QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.01% | 100% | 99% |
| Class A2 | 0.01% | 0.02% | -0.02% | 99% | 100% |
| Class B | -0.01% | 0.03% | -0.05% | 100% | 98% |
| Class C | 0.00% | -0.11% | -0.02% | 100% | 95% |
| Class E | -0.04% | 0.01% | -0.01% | 99% | 96% |
| Overall | -0.01% | -0.01% | -0.02% | 99% | 98% |
| Class D | 0.01% | -0.05% | -0.12% | 99% | 96% |
| Class F | -0.10% | -0.21% | -0.23% | 100% | 99% |
| Class TGM | -0.10% | -0.13% | -0.13% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 99% | 99% |
| Class A2 | 0.04% | 0.00% | 0.03% | 99% | 99% |
| Class B | -0.02% | 0.02% | 0.07% | 99% | 99% |
| Class C | 0.01% | 0.06% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 99% | 99% |
| Class D | -0.01% | -0.39% | 0.05% | 99% | 97% |
| Class F | -0.05% | -0.20% | -0.08% | 99% | 97% |
| Class TGM | -0.13% | -0.21% | -0.18% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.12% | -0.05% | 99% | 99% |
| Class C | -0.03% | 0.03% | -0.13% | 99% | 96% |
| Class E | -0.09% | 0.03% | 0.73% | 99% | 98% |
| Overall | -0.04% | 0.06% | 0.12% | 99% | 98% |
| Class D | -0.02% | -0.23% | 0.27% | 100% | 94% |
| Class F | -0.15% | -0.14% | -0.11% | 99% | 97% |
| Class TGM | -0.28% | -0.21% | -0.15% | 100% | 103% |

## Low QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.04% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.02% | 0.02% | 100% | 100% |
| Class E | 0.08% | -0.01% | -0.02% | 100% | 99% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class F | 0.19% | 0.02% | 0.05% | 99% | 98% |
| Class TGM | 0.23% | 0.15% | 0.16% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | -0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | -0.04% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% | 100% | 99% |
| Class D | 0.00% | -0.03% | -0.04% | 99% | 98% |
| Class F | 0.12% | -0.10% | -0.05% | 100% | 99% |
| Class TGM | 0.20% | 0.13% | 0.12% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.03% | 0.02% | 99% | 98% |
| Class E | 0.02% | -0.02% | -0.04% | 100% | 101% |
| Overall | 0.01% | 0.01% | -0.01% | 99% | 99% |
| Class D | -0.01% | 0.01% | -0.02% | 100% | 98% |
| Class F | -0.10% | -0.08% | -0.08% | 100% | 99% |
| Class TGM | 0.13% | 0.13% | 0.21% | 99% | 96% |

# Results of combining JVET-P0435 and JVET-P0447

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

The test is the combined significant flag coding in JVE-P0435 [1] and Rice parameter range extension in JET-P0447 [3].

## Standard QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class B | 0.00% | -0.01% | -0.06% | 100% | 100% |
| Class C | 0.02% | -0.03% | 0.05% | 100% | 96% |
| Class E | -0.04% | -0.03% | -0.01% | 100% | 96% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 99% |
| Class D | 0.02% | -0.14% | -0.12% | 99% | 97% |
| Class F | -0.06% | -0.12% | -0.03% | 99% | 98% |
| Class TGM | -0.06% | -0.12% | -0.10% | 98% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.06% | 99% | 99% |
| Class A2 | 0.02% | 0.01% | -0.01% | 99% | 99% |
| Class B | -0.02% | 0.02% | 0.10% | 100% | 100% |
| Class C | 0.00% | 0.10% | 0.07% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.06% | 99% | 99% |
| Class D | -0.05% | -0.43% | 0.31% | 100% | 99% |
| Class F | -0.04% | -0.12% | 0.00% | 99% | 98% |
| Class TGM | -0.12% | -0.17% | -0.15% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.03% | -0.33% | 99% | 99% |
| Class C | 0.00% | 0.09% | -0.01% | 99% | 97% |
| Class E | 0.00% | 0.46% | 0.67% | 99% | 98% |
| Overall | -0.01% | 0.16% | 0.03% | 99% | 98% |
| Class D | 0.00% | 0.15% | 0.22% | 100% | 97% |
| Class F | -0.22% | -0.42% | 0.18% | 99% | 98% |
| Class TGM | -0.15% | -0.11% | -0.08% | 100% | 101% |

## Low QP results at 1.75 bins per coefficient

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.02% | 0.01% |  | 0.04% |  | 101% |  | 101% |  |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class B | 0.01% | 0.00% |  | -0.01% |  | 100% |  | 99% |  |
| Class C | 0.01% | 0.01% |  | 0.01% |  | 100% |  | 99% |  |
| Class E | 0.05% | 0.02% |  | 0.05% |  | 100% |  | 97% |  |
| Overall | 0.02% | 0.01% |  | 0.01% |  | 100% |  | 99% |  |
| Class D | 0.01% | -0.01% |  | -0.01% |  | 100% |  | 98% |  |
| Class F | 0.07% | -0.06% |  | -0.07% |  | 98% |  | 98% |  |
| Class TGM | -0.14% | -0.12% |  | -0.11% |  | 100% |  | 97% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.02% | 0.02% |  | -0.01% |  | 99% |  | 100% |  |
| Class A2 | 0.01% | 0.00% |  | 0.01% |  | 100% |  | 100% |  |
| Class B | 0.01% | 0.02% |  | 0.00% |  | 100% |  | 100% |  |
| Class C | 0.00% | -0.01% |  | -0.02% |  | 100% |  | 98% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% |  | 0.00% |  | 100% |  | 100% |  |
| Class D | 0.02% | -0.08% |  | -0.01% |  | 100% |  | 97% |  |
| Class F | 0.07% | -0.33% |  | -0.36% |  | 99% |  | 99% |  |
| Class TGM | -0.35% | -0.08% |  | -0.05% |  | 100% |  | 99% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% |  | -0.02% |  | 99% |  | 99% |  |
| Class C | 0.00% | 0.01% |  | 0.03% |  | 100% |  | 100% |  |
| Class E | 0.03% | -0.01% |  | -0.03% |  | 100% |  | 100% |  |
| Overall | 0.01% | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class D | 0.00% | 0.00% |  | -0.03% |  | 99% |  | 96% |  |
| Class F | -0.16% | -0.19% |  | -0.10% |  | 99% |  | 98% |  |
| Class TGM | -0.06% | -0.01% |  | 0.00% |  | 99% |  | 99% |  |

# References

- C. Auyeung, X. Li, X. Zhao, S. Liu, “CE7-related: Modifications to transform skip significant flag coding,” JVET-P0435, 16th Meeting: Geneva, CH, 1–11 October 2019.

- C. Auyeung, X. Li, X. Zhao, S. Liu, “CE7-related: Modifications to transform skip coefficient sign flag coding,” JVET-P0437, 16th Meeting: Geneva, CH, 1–11 October 2019.

- C. Auyeung, X. Li, X. Zhao, S. Liu, “CE7-related: Rice parameter range extension in residual coding with transform skip,” JVET-P0447, 16th Meeting: Geneva, CH, 1–11 October 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-6.0.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027-v3, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).