| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0453 |
| --- | --- |

| Title: | AHG8: On diagonal referencing for layered VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add support of diagonal referencing for layered VVC bitstream, i.e., inter-layer dependency for picture of different access units.

# Introduction

Last meeting, JVET-O1159 was adopted for layered scalability in VVC. During the discussion, it was pointed out that “diagonal” referencing could be a desirable functionality. In fact, “diagonal” referencing would allow Open GOP switching to enhancement layers, where RASL pictures use pictures from lower layers as reference when the corresponding higher layer pictures are not available.

With further changes (allowing referencing of higher layers), MGS-like approaches similar to SVC could be implemented in VVC.

# Proposal

It is proposed to add syntax to the SPS to indicate that diagonal referencing is allowed.

It is proposed to add the following syntax elements to express how “diagonal” referencing is carried out.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_diagonal_ref_pics_enabled_flag | u(1) |
| … |  |
| } |  |

At the same time, it is proposed to add the layer indication to signal explicitly the layer from which it is predicted within the RPL struct:

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| if( inter_layer_diagonal_ref_pics_enabled_flag ) |  |
| ildrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } else |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ildrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILDRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ildrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive.

# Discussion regarding MGS

Although not included in this proposal, MGS-like referencing could be achieved if prediction from higher layers would be allowed. However, this is currently not feasible since layer dependencies are described within the VPS for layer_ids lower than the current layer id. It is proposed for the group to discuss whether MGS-functionality with its associated changes would be desirable in VVC.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).