| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0455-v2 |
| --- | --- |

| Title: | Non-CE8: IBC slice level on/off |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to control the usage of IBC at slice level, in addition to the current IBC SPS flag. That means, when IBC SPS flag is true, for each slice, an encoder can choose to turn on or off the usage of IBC. The indication of whether IBC is used for a slice is through the value of a slice level variable MaxNumIbcMergeCand. When this variable is set equal to 0, IBC is turned off for the slice. On top of this change, an encoder algorithm is provided to switch IBC on/off using the same logic as disabling MMVD fractional-pel offset, which already exists in the VTM. The intend of the suggested algorithm is that IBC mode is turned off for slices where IBC may not contribute much to the BD rate reduction while IBC mode is turned on for screen content materials. Simulation results report that:

- The BD rate/runtime changes for CTC (class A to E) average are -0.02%/0.02%/0.03% and 103%/100%/100% for AI/RA/LB, separately, when compared to VTM-6.0+IBC=0 anchor;

- The BD rate/runtime changes for Class F average are 0.00%/0.00%/0.00% and 98%/99%/99% for AI/RA/LB, separately, when compared to VTM-6.0+IBC=1 anchor

- The BD rate/runtime changes for Class TGM 1080p average are 0.00%/0.00%/0.00% and 98%/99%/99% for AI/RA/LB, separately, when compared to VTM-6.0+IBC=1 anchor

Problem Statement

In current VVC specification [1], IBC mode is enabled at sequence level. However, the use of IBC mode for different types of contents produced unbalanced encoder runtime/BD rate reduction trade-offs. More specifically, the encoder time spent on IBC search for natural contents seems to be too high while no enough coding benefits can be achieved to justify the complexity. It was recommended that IBC mode can be turned off for those content in the common test condition (CTC). At the meantime, it is desirable to have the flexibility to control the use of IBC at picture/slice level to adapt to content switch across the sequence.

In this proposal, a slice level IBC mode switching is proposed, together with an encoder algorithm to turn on/off IBC mode at slice level is proposed.

Proposed methods

Slice level IBC Control

In this proposal, a slice level control for IBC mode is proposed, using the value of MaxNumIbcMergeCand. When MaxNumIbcMergeCand is equal to 0, IBC mode is disabled in this slice; when MaxNumIbcMergeCand is greater than 0, IBC mode is enabled for this slice.

Encoder algorithm

In the current VTM implementation, a hash hit-ratio based decision for fractional-pel offset disabling in MMVD is used. In this proposal, the same decision is reused (with threshold set to 3%) for deciding whether to set MaxNumIbcMergeCand equal to 0. When the hash hit-ratio is not high enough, fractional-pel offset in MMVD will be used. In the same condition, IBC will be disabled for the slice.

Simulation results

The proposed method is implemented on top of VTM-6.0.

Results are compared to VTM-6.0 CTC conditions.

Table I: Performance of proposed methods vs. VTM-6.0 CTC (IBC on for class F/TGM; IBC off for class A~E)

Conclusion

In this contribution, a slice level IBC on/off control is implemented by setting MaxNumIbcMergeCand equal to 0. In this way, the use of IBC can be controlled at slice level, with no new syntax element needed.

Text changes (based on JVET-O2001-vE)

- 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flagMaxNumIbcMergeCand ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && ! sps_ibc_enabled_flagMaxNumIbcMergeCand ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flagMaxNumIbcMergeCand && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flagMaxNumIbcMergeCand ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && ! sps_ibc_enabled_flagMaxNumIbcMergeCand && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_TYPE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |

- 7.4.7.1 General slice header semantics

slice_six_minus_max_num_ibc_merge_cand specifies the maximum number of IBC merging block vector prediction (BVP) candidates supported in the slice subtracted from 6. The maximum number of IBC merging BVP candidates, MaxNumIbcMergeCand is derived as follows:

MaxNumIbcMergeCand = 6 − slice_six_minus_max_num_ibc_merge_cand	(7-114)

The value of MaxNumIbcMergeCand shall be in the range of 10 to 6, inclusive.

When not signaled, MaxNumIbcMergeCand is inferred to be equal to 0. When MaxNumIbcMergeCand is equal to 0, IBC prediction mode is not used in the slice.

- 7.4.9.5	Coding unit semantics

pred_mode_ibc_flag equal to 1 specifies that the current coding unit is coded in IBC prediction mode. pred_mode_ibc_flag equal to 0 specifies that the current coding unit is not coded in IBC prediction mode.

When pred_mode_ibc_flag is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, and cbWidth is equal to 4, and cbHeight is equal to 4, pred_mode_ibc_flag is inferred to be equal 1.

- Otherwise, if either cbWidth or cbHeight are equal to 128, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if modeType is equal to MODE_TYPE_INTER, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, if treeType is equal to DUAL_TREE_CHROMA, pred_mode_ibc_flag is inferred to be equal to 0.

- Otherwise, pred_mode_ibc_flag is infered to be equal to the value of sps_ibc_enabled_flag ( MaxNumIbcMergeCand != 0 ) when decoding an I slice, and 0 when decoding a P or B slice, respectively.

References

[1]	B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, July. 2019

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).