| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0457-v2 |
| --- | --- |

| Title: | Non-CE8: On IBC merge list size signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, a spec/software mismatch has been identified with the current spec. Currently, IBC and inter mode have independent merge size signaling at slice level. The merge_idx signaling at block level is shared by both inter merge and IBC merge modes. However, the range of merge index is decided by the value of MaxNumMergeCand while the actual maximum values for the two modes can be different. Undefined behaviour can happen if MaxNumIbcMergeCand is greater than MaxNumMergeCand. Accordingly, three solutions have been proposed as suggested potential fixes.

- In solution 1: the merge size of IBC is limited to be no greater than the merge size of inter.

- In solution 2: the merge_idx range is set to be the larger of the two sizes.

- In solution 3: the merge idx range is switched at block level, depending on the current block’s prediction mode.

Problem Statement

In current VVC specification [1], separate merge size signaling is used for IBC mode and inter mode. The variables MaxNumIbcMergeCand and MaxNumMergeCand are both range from 1 to 6, inclusively.

On the other hand, the signaling flag of merge index, merge_idx is shared for both modes, and it is coded using TR (truncated unary). The maximum range of the syntax is decided by MaxNumMergeCand. When MaxNumIbcMergeCand is greater than MaxNumMergeCand, the behaviour is undefined.

Proposed solutions

Solution 1:

Constraint is imposed such that MaxNumIbcMergeCand shall not be greater than MaxNumMergeCand.

Text changes:

six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. When not presented, the value of six_minus_max_num_merge_cand is inferred to be 0 (MaxNumMergeCand is inferred to be 6). The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive.

six_minus_max_num_ibc_merge_cand specifies the maximum number of ibc merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. The maximum number of ibc merging MVP candidates, MaxNumIbcMergeCand is derived as follows:

MaxNumIbcMergeCand = 6 − six_minus_max_num_ibc_merge_cand

If the slice type is I, the value of MaxNumIbcMergeCand shall be in the range of 1 to 6, inclusive; Otherwise (the slice type is P or B), the value of MaxNumIbcMergeCand shall be in the range of 1 to 6MaxNumMergeCand, inclusive.

Solution 2:

Set the max value of merge_idx to be the max of MaxNumIbcMergeCand and MaxNumMergeCand

Text changes:

six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. When not presented, the value of six_minus_max_num_merge_cand is inferred to be 5 (MaxNumMergeCand is inferred to be 1). The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand

Table 9 11 – Syntax elements and associated binarizations

| merge_data( ) | …… |  |  |
| --- | --- | --- | --- |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 2, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = Max(MaxNumMergeCand, MaxNumIbcMergeCand) − 1, cRiceParam = 0 |

Solution 3:

Switch the max value of merge_idx at block level according to the prediction mode (MODE_IBC or MODE_INTER). Note this is the current VTM implementation.

Text changes:

Table 9 11 – Syntax elements and associated binarizations

| merge_data( ) | …… |  |  |
| --- | --- | --- | --- |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = MaxNumTriangleMergeCand − 2, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = ( CuPredMode[ 0 ][ x0 ][ y0 ] != MODE_IBC ? MaxNumMergeCand : MaxNumIbcMergeCand ) − 1, cRiceParam = 0 |

Simulation results

The proposed method is implemented on top of VTM-6.0. Solution 3 is used as anchor, and the anchor data is generated by setting --MaxNumMergeCand=6 and --MaxNumIBCMergeCand=5 in Table I and setting --MaxNumMergeCand=5 and --MaxNumIBCMergeCand=6.

For solution 1, simulation results are shown in Table I below.

Table I:

Additional test results by setting inter merge size to 6 and ibc merge size to 2 are provided below in Table II:

For solution 2, simulation results are shown in Table III below.

Table III:

Conclusion

In this contribution, an undefined behaviour is identified in the spec when MaxNumIbcMergeCand is greater than MaxNumMergeCand. Three solutions are suggested to fix this issue. It is recommended to take one of them into the next release of VVC specification.

References

[1]	B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, July. 2019

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).