| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0461 |
| --- | --- |

| Title: | Non-CE4: Usage of half-pel switchable interpolation filter for pairwise candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, the alternative half-pel interpolation filter flag (denoted as UseAltHpelIf) of the pairwise average candidate is derived dependent on the UseAltHpelIf flags of both associated merge candidates with a comparison operation to switch between two cases. This contribution proposes two alternative methods to derive the UseAltHpelIf flag for a pairwise average candidate without any comparison or switching operations.

Method #1: Always set the UseAltHpelIf flag of the pairwise average candidate to false.

Method #2: UseAltHpelIf flag of the associated merge candidate with smaller merge index is used for the pairwise average candidate.

Simulation results reportedly show that both methods have negligible impact on the coding performance.

-	Method #1: RA: 0.00%/ 100% / 100%, 	LDB: -0.01%/ 100%/ 99%.

-	Method #2: RA: 0.00%/ 101%/ 101%, 	LDB: 0.00%/ 100%/ 98%.

# Introduction

In VTM-6.0, if the alternative luma half-pel interpolation filter flags of the two merge candidates used for generating a pairwise merge candidate are equal, the alternative luma half-pel interpolation filter flag of the pairwise merge candidate is set equal to that of the merge candidate with a smaller merge index; otherwise, it is set equal to false. The derivation switches between two cases depending on one comparison operation.

Hereinafter, UseAltHpelIf is used to represent whether an alternative luma half-pel interpolation filter is employed or not (e.g., when UseAltHpelIf is equal to 1, the alternative luma half-pel interpolation filter is used; otherwise, the alternative luma half-pel interpolation filter is not used).

- Proposed methods

This contribution proposes to simplify the derivation process of the UseAltHpelIf flag for the pairwise candidate without any comparison or switching operations.

  - Method #1

UseAltHpelIf flag of the pairwise average candidate is always set equal to false.

  - Method #2

UseAltHpelIf flag of the associated merge candidate with smaller merge index is used for the pairwise average candidate.

# Experimental Results

The proposed methods have been implemented on top of VTM-6.0. Results under RA/LDB configurations are summarized in Table 1 ~ Table 2. The simulations were performed following the the common test conditions [1]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method 1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | -0.04% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.03% | 0.08% | 0.06% | 100% | 101% |  |  |  |  |  |
| Class B | -0.01% | 0.07% | -0.01% | 100% | 100% | -0.01% | 0.08% | -0.07% | 99% | 98% |
| Class C | -0.01% | 0.00% | -0.01% | 100% | 100% | -0.04% | 0.05% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  | 0.04% | 0.25% | 0.29% | 100% | 99% |
| Overall | 0.00% | 0.07% | 0.00% | 100% | 100% | -0.01% | 0.11% | 0.03% | 100% | 99% |
| Class D | 0.00% | 0.10% | 0.10% | 100% | 99% | -0.03% | 0.01% | -0.01% | 100% | 101% |
| Class F | 0.00% | -0.01% | 0.00% | 100% | 100% | 0.05% | 0.13% | 0.44% | 100% | 99% |
| Class SCC | 0.00% | -0.01% | -0.02% | 100% | 100% | 0.02% | -0.01% | 0.04% | 99% | 99% |

Table 2: Coding performance of the proposed method 2.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | -0.07% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.02% | 0.00% | 0.02% | 101% | 101% |  |  |  |  |  |
| Class B | -0.01% | 0.11% | -0.01% | 101% | 100% | 0.02% | 0.13% | 0.00% | 100% | 98% |
| Class C | -0.02% | 0.02% | 0.02% | 101% | 103% | -0.04% | 0.04% | 0.02% | 101% | 99% |
| Class E |  |  |  |  |  | 0.01% | 0.38% | 0.14% | 100% | 98% |
| Overall | 0.00% | 0.03% | -0.01% | 101% | 101% | 0.00% | 0.16% | 0.04% | 100% | 98% |
| Class D | -0.01% | 0.07% | 0.06% | 101% | 103% | 0.01% | -0.05% | 0.06% | 101% | 102% |
| Class F | -0.01% | 0.00% | 0.01% | 102% | 102% | -0.02% | 0.08% | 0.21% | 102% | 102% |
| Class SCC | -0.02% | -0.01% | 0.00% | 101% | 100% | -0.03% | 0.02% | 0.03% | 99% | 98% |

- Conclusions

In this contribution, the derivation process for UseAltHpelIf of the pairwise candidate is simplified. There is a minor overall coding performance change with almost no impact on encoding/decoding time compared to VTM-6.0. It is recommended to adopt this simplification method into VVC.

- Suggested changes to working draft

The proposed changes on top of JVET-O2001-vE are specified as below, with deleted text highlighted in turquoise strikethrough.

  - Method #1

8.5.2.4	Derivation process for pairwise average merging candidate

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCand and refIdxL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCand and mvL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the half sample interpolation filter index hpelIfIdxavgCand of every candidate avgCand added into mergeCandList during the invocation of this process.

The variable numRefLists is derived as follows:

numRefLists  =  ( slice_type  = =  B )  ?  2 : 1	(8-344)

The following assignments are made, with p0Cand being the candidate at position 0 and p1Cand being the candidate at position 1 in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ 0 ]	(8-345)

p1Cand = mergeCandList[ 1 ]	(8-346)

The candidate avgCand is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCand, and the reference indices, the prediction list utilization flags and the motion vectors of avgCand are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], and mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp0Cand	(8-347)

predFlagLXavgCand = 1	(8-348)

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 0 ] as output.

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 1 ] as output.

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp0Cand	(8-349)

predFlagLXavgCand = 1	(8-350)

mvLXavgCand[ 0 ] = mvLXp0Cand[ 0 ] 	(8-351)

mvLXavgCand[ 1 ] = mvLXp0Cand[ 1 ] 	(8-352)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp1Cand	(8-353)

predFlagLXavgCand = 1	(8-354)

mvLXavgCand[ 0 ] = mvLXp1Cand[ 0 ] 	(8-355)

mvLXavgCand[ 1 ] = mvLXp1Cand[ 1 ] 	(8-356)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = −1	(8-357)

predFlagLXavgCand = 0	(8-358)

mvLXavgCand[ 0 ] = 0	(8-359)

mvLXavgCand[ 1 ] = 0	(8-360)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCand = −1	(8-361)

predFlagL1avgCand = 0	(8-362)

- The half sample interpolation filter index hpelIfIdxavgCand is derived as follows:

- If hpelIfIdxp0Cand is equal to hpelIfIdxp1Cand, hpelIfIdxavgCand is set equal to hpelIfIdxp0Cand.

- Otherwise, hpelIfIdxavgCand is set equal to 0.

  - Method #2

8.5.2.4	Derivation process for pairwise average merging candidate

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCand and refIdxL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCand and mvL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the half sample interpolation filter index hpelIfIdxavgCand of every candidate avgCand added into mergeCandList during the invocation of this process.

The variable numRefLists is derived as follows:

numRefLists  =  ( slice_type  = =  B )  ?  2 : 1	(8-344)

The following assignments are made, with p0Cand being the candidate at position 0 and p1Cand being the candidate at position 1 in the merging candidate list mergeCandList:

p0Cand = mergeCandList[ 0 ]	(8-345)

p1Cand = mergeCandList[ 1 ]	(8-346)

The candidate avgCand is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCand, and the reference indices, the prediction list utilization flags and the motion vectors of avgCand are derived as follows and numCurrMergeCand is incremented by 1:

- For each reference picture list LX with X ranging from 0 to ( numRefLists − 1 ), the following applies:

  - If predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], and mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp0Cand	(8-347)

predFlagLXavgCand = 1	(8-348)

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 0 ] as output.

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ], rightShift set equal to1, and leftShift set equal to 0 as inputs and the rounded mvLXavgCand[ 1 ] as output.

  - Otherwise, if predFlagLXp0Cand is equal to 1 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp0Cand	(8-349)

predFlagLXavgCand = 1	(8-350)

mvLXavgCand[ 0 ] = mvLXp0Cand[ 0 ] 	(8-351)

mvLXavgCand[ 1 ] = mvLXp0Cand[ 1 ] 	(8-352)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 1, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = refIdxLXp1Cand	(8-353)

predFlagLXavgCand = 1	(8-354)

mvLXavgCand[ 0 ] = mvLXp1Cand[ 0 ] 	(8-355)

mvLXavgCand[ 1 ] = mvLXp1Cand[ 1 ] 	(8-356)

  - Otherwise, if predFlagLXp0Cand is equal to 0 and predFlagLXp1Cand is equal to 0, the variables refIdxLXavgCand, predFlagLXavgCand, mvLXavgCand[ 0 ], mvLXavgCand[ 1 ] are derived as follows:

refIdxLXavgCand = −1	(8-357)

predFlagLXavgCand = 0	(8-358)

mvLXavgCand[ 0 ] = 0	(8-359)

mvLXavgCand[ 1 ] = 0	(8-360)

- When numRefLists is equal to 1, the following applies:

refIdxL1avgCand = −1	(8-361)

predFlagL1avgCand = 0	(8-362)

- The half sample interpolation filter index hpelIfIdxavgCand is derived as follows:

- If hpelIfIdxp0Cand is equal to hpelIfIdxp1Cand, hpelIfIdxavgCand is set equal to hpelIfIdxp0Cand.

- Otherwise, hpelIfIdxavgCand is set equal to 0.

- References

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).